3GPP SA WG2 Meeting #166	S2-2412217

# Orlando, US, 18-22 November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0509 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Identifiers for 5G ProSe Multi-hop UE-to-UE Relay for IP PDU type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The information about identifiers include in clause 5.8.6.2 is inconsistent with the one described in the Procedure in the clause 6.3.2.6.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the additional parameter of the target 5G prose UE-to-UE relay in clause 5.8.6.2 - In clause 6.3.2.6.1 clarifed that the procedure can be used by Multi Hop relays among themselves in order to join the MANET network - Other minor corrections at few other clause |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.6.2; 6.3.2.6.1; 6.3.2.6.2; 6.4.3.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

#### 5.8.6.2	Identifiers for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type

The Multi-hop UE-to-UE Relay Discovery Announcement message (Model A) is sent by the relay and contains the following information:

-	Source Layer-2 ID: the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay self-selects a Source Layer-2 ID. It is used for the transmitting the message at AS layer, and included in the AS layer headers only.

-	Destination Layer-2 ID: the Destination Layer-2 ID for Multi-hop UE-to-UE Relay Discovery Announcement message (Model A) is selected based on the configuration (associated with the RSC) as defined in clause 5.1.5.1.

-	User Info ID of 5G ProSe Multi-hop UE-to-UE Relay: provides information about the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

-	Relay Service Code: information to indicate the connectivity service the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay provides to the 5G ProSe End UEs.

The 5G ProSe Multi-hop UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the source 5G ProSe End UE or a 5G ProSe Layer-3 Multi-hop UE-to-UE Relay, and includes the following information:

-	Source Layer-2 ID: the discoverer 5G ProSe End UE or 5G ProSe Layer-3 Multi-hop UE-to-UE Relay self-selects a Source Layer-2 ID.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe Multi-hop UE-to-UE Relay Discovery Solicitation message is selected based on the configuration (associated with the RSC) as defined in clause 5.1.5.1.

User Info ID of discoverer: provides information about the 5G ProSe Multi-hop UE-to-UE Relay sending the Solicitation message. It is not present when the message is sent by the source 5G Prose End UE

-	(optional) User Info ID of 5G Prose UE-to-UE Relay: this may be used by the receiver of the message to determine whether to respond to the solicitation. It is only included when the sender wants to find a specific target relay.

-	Relay Service Code: information about connectivity service offered by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s).

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC includes the following:

-	Source Layer-2 ID: the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay self-selects a Source Layer-2 ID.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe Multi-hop UE-to-UE Relay Discovery Solicitation message.

-	User Info ID of discoverer: the User Info ID from the received Solicitation message, to ensure the response can be matched at the receiver.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe Multi-hop UE-to-UE Relay sending the Response message.

-	Relay Service Code: information about connectivity service offered by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

Editor's note:	Additional information or identifiers for MANET operation assistance may be added.

CHANGE 2

##### 6.3.2.6.1	General

The 5G ProSe Multi-hop UE-to-UE Relay discovery may use one of the two different types of discovery operations, i.e. discovery for IP PDU type, and discovery for non-IP PDU type, depends on the RSC for the discovery.

5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type only needs to discover the presence of the 5G ProSe Multi-hop UE-to-UE Relay. The discovery of the target 5G ProSe End UEs are performed via DNS queries after establshing a link with the 5G ProSe Multi-hop UE-to-UE Relay(s). This procedure is also used by Multi-hop UE-to-UE Relay(s) among themselves in order to join/create MANET network. The details of the procedure are defined in clause 6.3.2.6.2.

5G ProSe Multi-hop UE-to-UE Relay discovery of non-IP (Ethernet or Unstructured) PDU type is for the discovery of a target 5G ProSe End UE and the corresponding routes. The procedures for 5G ProSe Multi-hop UE-to-UE Relay discovery Model A/B for non-IP PDUs (e.g. Ethernet or Unstructured) are defined in clause 6.3.2.6.3.

CHANGE 3

##### 6.3.2.6.2	Procedure for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type

Depicted in figure 6.3.2.6.2-1 is the procedure for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type, with Model A.

Figure 6.3.2.6.2-1: 5G ProSe Multi-hop UE-to-UE Relay Discovery of IP PDU type with Model A

1.	The 5G ProSe Multi-hop UE-to-UE Relay is configured to operate IP PDU type of discovery based on the indication associated with the RSC, as defined in defined in clause 5.1.5.1.

2.	The 5G ProSe Multi-hop UE-to-UE Relay sends a Multi-hp UE-to-UE Relay Discovery Announcement message. This message contains the Type of Discovery Message, User Info ID of the 5G ProSe Multi-hop UE-to-UE Relay, and the RSC, as defined in clause 5.8.6.2.

The 5G ProSe Multi-hop UE-to-UE Relay self generates a Source Layer-2 ID, and decides a Destiantion Layer-2 ID as defined in clause 5.8.6.2.

The 5G ProSe End UE(s) or other 5G ProSe Multi-hop UE-to-UE Relay(s) in proximity monitoring the same RSC may receive the announcement message and process it accordingly, e.g. initiates a Layer-2 Link establishment with the announcing 5G ProSe Multi-hop UE-to-UE Relay.

Depicted in figure 6.3.2.6.2-2 is the procedure for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type, with Model B. A 5G ProSe Multi-hop UE-to-UE Relay may also use the procedure, i.e. send the Multi-hop UE-to-UE Relay Discovery Solicitation (taking the role of the 5G ProSe End UE (Discoverer)), if it needs to discover other 5G ProSe Multi-hop UE-to-UE Relays.

Figure 6.3.2.6.2-2: 5G ProSe Multi-hop UE-to-UE Relay Discovery of IP PDU type with Model B

1.	The 5G ProSe End UE is configured to operate IP PDU type of discovery based on the indication associated with the RSC, as defined in defined in clause 5.1.5.1.

2.	The 5G ProSe End UE sends a Multi-hop UE-to-UE Relay Discovery Solicitation message. This message contains the Type of Discovery Message, RSC, and (optional) User Info ID of the 5G ProSe Multi-hop UE-to-UE Relay, as defined in clause 5.8.6.2.

The User Info ID of the 5G ProSe Multi-hop UE-to-UE Relay is only included if the 5G ProSe End UE wants to find a specific target relay.

The 5G ProSe End UE self generates a Source Layer-2 ID, and decides a Destination Layer-2 ID as defined in clause 5.8.6.2.

3.	The 5G ProSe Multi-hop UE-to-UE Relay-1 may send a Multi-hop UE-to-UE Relay Discovery Response message, if the RSC contained in the solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.5.1. If User Info ID is included in the solicitation message, 5G ProSe Multi-hop UE-to-UE Relay-1 only sends the response message when it matches its own User Info ID.

The 5G ProSe Multi-hop UE-to-UE Relay-1 in proximity may decide not to respond if the criteria are not met, or due to UE local configurations.

CHANGE 4

##### 6.4.3.10.1	Layer-2 Link establishment and management for IP PDU type

For the 5G ProSe Communication via 5G ProSe UE-to-UE Relay as described in clause 6.7.5.2:

-	The Direct Communication Request message over the PC5 reference point (between a 5G ProSe End UE and 5G ProSe Multi-hop UE-to-UE Relay, or between 5G ProSe Multi-hop UE-to-UE Relays) includes:

-	User info (i.e. Application Layer ID) of source 5G ProSe End UE or User Info ID of 5G ProSe Multi-hop UE-to-UE Relay: the identity of the source 5G ProSe End UE or 5G ProSe Multi-hop UE-to-UE Relay requesting the connection establishment.

-	User Info ID of 5G ProSe Multi-hop UE-to-UE Relay: the identity of the target 5G ProSe Multi-hop UE-to-UE Relay provided during the 5G ProSe Multi-hop UE-to-UE Relay Discovery procedure.

-	(optional)ProSe Service Info: the information about the ProSe identifier(s) requesting Layer-2 link establishment.

-	RSC: the connectivity service provided by the 5G ProSe Multi-hop UE-to-UE Relay as requested.

-	Security Information: the information for the establishment of security for the PC5 link establishment (see TS 33.503 [29]).

The Direct Communication Accept message over the PC5 reference point (between a 5G ProSe End UE and 5G ProSe Multi-hop UE-to-UE Relay, or between 5G ProSe Multi-hop UE-to-UE Relays) includes:

-	User Info ID of 5G ProSe UE-to-UE Relay: the identity of the 5G Prose Multi-hop UE-to-UE Relay sending the Accept message.

Other link management procedures as defined in clause 6.4.3 (i.e. Link identifier update in 6.4.3.2, Layer-2 link release in clause 6.4.3.3, Layer-2 link modification in clause 6.4.3.4, Layer-2 link maintenance in 6.4.3.5) can be used for the layer-2 link without enhancements.

When a new layer-2 link is established, or an existing layer-2 link is released, the 5G ProSe Multi-hop UE-to-UE Relay need to inform the status change internally to the MANET router stack.

Editor's note:	It is FFS if the Layer-2 link management messages need to be enhanced to support efficient MANET operations.

END of Changes