3GPP TSG-SA2 Meeting #166	S2-2412220

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1299 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean up on QoS sustainability Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As indicated in the LS in from SA5 (S3-241777, S2-2405872), SA5 had a conclusion on the UL and DL capacity GTP for QoS sustainability analytics base on SA2 request. In SA2 163, CR (S2-2407015) introduced modifications in TS 23.288, deleting UL/DL available capacity GTP from the table and relevant descriptions, because LS in stated that the definition was not necessary, and OAM didn’t provide the data in the present release. However, it its stated that the NWDAF could simply calculate the UL/DL available capacity GTP by implementation, subtracting the “UL/DL traffic volume” from the maximum GTP capacity. The available capacity is normally not easily measurable, and usually requires support from monitoring systems with access to utilization measurement for all links. Such access is also normally limited to the corresponding network operators. Furthermore, from IETF-RFC 5136 (Defining Network Capacity), the “IP Link Usage” definition states that the information transmitted across the link can be generated by any source, including those sources that may not be directly attached to either side of the link. Consequently, the UL/DL traffic volume is not necessary representative to the real usage of network, and we can’t assume to provide available capacity GTP by this way. Being an implementation aspect specific, we propose to remove the last sentence and consider that the value determination is not covered in this release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify explanations to describe how NWDAF can calculate UL/DL available capacity GTP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading of UL/DL available capacity GTP determination, which is implementation specific |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of 1st Change * * *

6.9.2	Input data

To derive the QoS Sustainability analytics for a path of interest or for an area of interest with coarse granularity (i.e. TAIs or Cell IDs), the input data is listed in Table 6.9.2-1.

Table 6.9.2-1: Data collection for "QoS Sustainability" analytics

| Information | Source | Description |
| --- | --- | --- |
| RAN UE Throughput | OAM TS 28.554 [10] | Average UE bitrate in the cell (Payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface), per timeslot, per cell, per 5QI and per S-NSSAI. |
| QoS flow Retainability | OAM TS 28.554 [10] | Number of abnormally released QoS flows during the time the QoS Flows were used per timeslot, per cell, per 5QI and per S-NSSAI. |
| Delay in RAN | OAM TS 28.554 [10] | Average Uplink and downlink packet transmission delay through RAN part to the UE, per timeslot, per cell, per 5QI and per S-NSSAI. |

NOTE 1:	The timeslot is the time interval split according to the time unit of the OAM statistics defined by operator.

To derive the QoS Sustainability analytics for an area of interest with fine granularity (i.e. geographical area that can be smaller than a cell), additional input data is listed in Table 6.9.2-2.

Table 6.9.2-2: UE level data collection for "QoS Sustainability" analytics with fine granularity

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | LCS (NOTE 1) | A time stamp associated with the collected information. |
| UE ID | LCS, AMF (NOTE 1) | (list of) SUPI(s). |
| Finer granularity location data | LCS (NOTE 1) | UE position in the area of interest. |
| Speed | LCS (NOTE 1) | Current UE speed. |
| SMF info | AMF | SMF address for the SMF serving the UE |
| UPF info | SMF | Address for the UPF serving the UE. |
| 5QI | SMF | 5G QoS Identifier. |
| PEI | UDM | Permanent Equipment Identifier, as described in clause 5.9.3 of 1 23.501 [2]. |
| Equipment type | GSMA IMEI database or OAM | Equipment Type such as smartphone, tablet, dongle, etc. as described in GSMA TS.06 [38]. |
| NOTE 1: The procedure to collect location data using LCS is described in clause 6.2.12 |  |  |

NOTE 2:	How to use the input data from GSMA for QoS Sustainability analytics is up to NWDAF implementation.

Additionally, the NWDAF collects the following input (Table 6.9.2-3) according to measurements defined in clause 5.33.3 QoS Monitoring to Assist URLLC Service of TS 23.501 [2] and IP-layer clause capacity definition from ITU-T Y.1540 [40] between UE, NG-RAN and UPF at GTP level.  The UL/DL available GTP capacity between UPF and UE will also be used as inputs for the QoS Sustainability analytics. The NWDAF calculates this value by implementation, e.g., by subtracting the UL/DL traffic volume from the maximum value of the GTP capacity.

NOTE 3: How the NWDAF determines this value is not specified in this release

Table 6.9.2-3: Data collection for QoS Sustainability analytics at GTP level

| Information | Source | Description |
| --- | --- | --- |
| UL/DL packet delay GTP | OAM TS 28.552 [8] (NOTE) | UL/DL packet delay measurement round trip on GTP path on N3 for non-GBR traffic. |
| UL/DL capacity GTP between UPF and NG-RAN | OAM TS 28.552 [8] TS 28.554 [10] | Maximum achievable UL/DL capacity measurement from UPF to NG-RAN based on GTP path. The capacity measurement corresponds to the IP-layer section capacity definition from ITUT Y.1540 [40]. It also corresponds to the descriptions in clause 5.1(DL GTP Capacity) and clause 5.4(UL GTP Capacity) of TS 28.552[8] and clause 6.3 of TS 28.554[X]. |
| UL/DL capacity GTP between UPF and UE | OAM TS 28.552 [8] TS 28.554 [10] | Maximum achievable UL/DL capacity measurement from UE to UPF based on GTP path. The capacity measurement corresponds to the IP-layer section capacity definition from ITUT Y.1540 [40]. It also corresponds to the descriptions in clause 5.4 of TS 28.552[8] and clause 6.3 of TS 28.554[X]. |
| NOTE : Refer to clause 5.1 of TS 28.552 [8] for the performance measurement in NG-RAN and clause 5.4 of TS 28.552 [8] for the performance measurement in UPF. In addition, Annex A of TS 28.552 [8] describes various performance measurements, especially, clause A.61 "Monitoring of one way delay between PSA UPF and NG-RAN" indicates that the measurements on the one way DL and UL delay between PSA UPF and NG-RAN can be used to evaluate and optimize the DL and UL user plane delay performance between 5GC and NG-RAN. NOTE 2 : UL/DL available capacity GTP between UPF and NG-RAN and UL/DL available capacity GTP between UPF and UE are not specified in this release |  |  |

* * * End of 1st Change * *