3GPP TSG-WG SA2 Meeting #166	S2-2412246

# 18 – 22 November 2024, Orlando, USA		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0511 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on U2N common identifiers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The identifier of Accumulated QoS for PC5 link can apply not only for model A discovery but also for model B discovery. Since the identifier of Path information in Discovery Response message is optional, it should also be optional in Discovery Solicitation message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Accumulated QoS for PC5 link for 5G ProSe multi-hop UE-to-Network Relay discovery(model B) - Revise Path information in Discovery Solicitation message to optional |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing or inaccurate identifiers for 5G ProSe UE-to-Network Relay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.8.3.1	Common identifiers for 5G ProSe UE-to-Network Relay

The following parameters are used for the 5G ProSe UE-to-Network Relay Discovery Announcement message (Model A), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Announcer Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1.

-	Announcer Info: provides information (i.e. User Info ID) about the announcing user.

-	Relay Service Code: parameter identifying a connectivity service the 5G ProSe UE-to-Network Relay provides to a 5G ProSe Remote UE. The Relay Service Codes are configured in a 5G ProSe UE-to-Network Relay for advertisement. Additionally, the Relay Service Code may also identifies authorized users the 5G ProSe UE-to-Network Relay would offer service to and may be used to select the related security policies or information e.g. necessary for authentication and authorization between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay (e.g. a Relay Service Code for relays for police members only would be different than a Relay Service Code for relays for Fire Fighters only, even though potentially they provided connectivity to same DN e.g. to support Internet Access).

The 5G ProSe UE-to-Network Relay Discovery Announcement message (Model A) is extended and modified as follows when used for 5G ProSe multi-hop UE-to-Network Relay discovery:

-	Hop-Count: This value reflects the number of PC5 hops for the 5G ProSe Remote UE to reach the network. It is set to 1 by the 5G ProSe UE-to-Network Relay and shall be incremented by 1 every time the 5G ProSe UE-to-Network Relay Discovery Announcement message is forwarded by a 5G ProSe Intermediate UE-to-Network Relay.

A 5G ProSe Intermediate UE-to-Network Relay shall only process the 5G ProSe UE-to-Network Relay Discovery Announcement if the Hop-Count is present and the value is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay self-selects a Source Layer-2 ID when sending the discovery message.

-	Destination Layer-2 ID: the 5G ProSe Intermediate UE-to-Network Relay sets the Desination Layer-2 ID of the 5G ProSe UE-to-Network Relay Discovery Announcement message based on configuration described in clause 5.1.4.1.

-	Announcer Info: identify information (i.e. User Info ID) of the announcing 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay.

-	(optinal) Root Relay Info: this is the User Info ID of the 5G ProSe UE-to-Network Relay. A 5G ProSe Intermediate UE-to-Network Relay may stored it in the discovery entry and included it in the 5G ProSe UE-to-Network Relay Discovery Announcement message. This information can be used in the relay selection at the 5G ProSe Remote UE or other 5G ProSe Intermediate UE-to-Network Relays.

-	(optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links to the Root Relay, i.e. the 5G ProSe UE-to-Network Relay, for this RSC.

-	(optional) Hop-Limit: this is the hop limit set by the announcing 5G ProSe UE-to-Network Relay to a value smaller than the configured maximum number of hops.

Editor's note:	Whether the same 5G ProSe UE-to-Network Relay Discovery Announcement message is reused for multi-hop operation or a new message type is to be defined will be decided by Stage 3.

The following parameters are used for the 5G ProSe UE-to-Network Relay Discovery Solicitation message (Model B), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Discoverer Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe Remote-UE self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1.

-	Discoverer Info: provides information (i.e. User Info ID) about the discoverer user.

-	Target Info: provides information (i.e. User Info ID) about the targeted discoveree user.

-	Relay Service Code: information about connectivity that the discoverer UE is interested in. The Relay Service Codes are configured in the 5G ProSe Remote UEs interested in related connectivity services.

To support Multi-hop 5G ProSe UE-to-Network Relay discovery the following parameters are added:

-	(Optional) Hop count: indicates the number of hops that the message is already relayed. It is increased by 1 per hop.

-	(Optional) Hop-Limit: a unmodified value that indicates the hop limit of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than the (pre)configured maximum number of hops.

-	(Optional)Path information: an (ordered) list of User Info ID(s) of Intermediate UE-to-Network Relay(s) that indicates the transmitted path of the message.

-    (Optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links along the path to the Intermediate UE-to-Network Relay.

The following parameters are used in the 5G ProSe UE-to-Network Relay Discovery Response message (Model B), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Discoveree Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe UE-to-Network Relay Discovery Solicitation message.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-Network Relay provides to 5G ProSe Remote UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

-	Discoveree Info: provides information (i.e. User Info ID) about the discoveree.

To support Multi-hop 5G ProSe UE-to-Network Relay discovery the following parameters are added:

-	(Optional) Hop count: indicates the number of hops between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay on the path selected by the 5G ProSe UE-to-Network Relay.

-	(Optional) Path information: an (ordered) list of User Info ID(s) of Intermediate UE-to-Network Relay(s) on the path selected by the 5G ProSe UE-to-Network Relay.

-    (Optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links along the path to the 5G ProSe UE-to-Network Relay.

The following parameters may be used in the Relay Discovery Additional Information message (using Model A) based on the procedure defined in clause 6.5.1.3 for 5G ProSe UE-to-Network Relay and clause 6.3.2.5.4 for 5G ProSe multi-hop UE-to-Network Relay where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and the other parameters are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay self-selects a Source Layer-2 ID to send the Relay Discovery Additional Information message.

-	Destination Layer-2 ID: the Destination Layer-2 ID to send the Relay Discovery Additional Information message is selected based on the configuration as described in clause 5.1.4.1.

-	Relay Service Code: the Relay Service Code associated with the message. The Relay Service Code is used to identify the security parameters needed by the receiving UE to process the discovery message.

-	Announcer Info: provides information about the announcing user (i.e. User Info ID of the 5G ProSe UE-to-Network Relay).

-	Hop-Count: This value reflects the number of PC5 hops for the 5G ProSe Remote UE to reach the network. It is set to 1 by the 5G ProSe UE-to-Network Relay and shall be incremented by 1 every time the Relay Discovery Additional Information message is forwarded by a 5G ProSe Intermediate UE-to-Network Relay.

A 5G ProSe Intermediate UE-to-Network Relay shall only process the Relay Discovery Additional Information message if the Hop-Count is present and the value is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

-	Announcer Info of 5G ProSe Intermediate UE-to-Network Relay: identify information (i.e. User Info ID) of the 5G ProSe Intermediate UE-to-Network Relay connected to 5G ProSe UE-to-Network Relay. This parameter is only applicable to the additional parameter announcement procedure over 5G ProSe multi-hop UE-to-Network Relay specified in clause 6.3.2.5.4.

-	(optional) Hop-Limit: this is the hop limit set by the announcing 5G ProSe UE-to-Network Relay to a value smaller than the configured maximum number of hops.

-	Additional parameters: the additional parameters for 5G ProSe Layer-3 UE-to-Network Relay (when applicable) are defined in clause 5.8.3.2.

NOTE 1:	The UE implementation needs to ensure that when the UE self-selects Source Layer-2 IDs, the self-selected Source Layer-2 IDs are different between 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery) in clause 6.3.2 and 5G ProSe Direct Communication (including 5G ProSe UE-to-Network Relay Communication) in clause 6.4 and are different from any other provisioned Destination Layer-2 IDs as described in clause 5.1 and any other self-selected Source Layer-2 IDs used in a simultaneous 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery) with a different discovery model.

NOTE 2:	If a 5G ProSe UE-to-Network Relay and 5G ProSe Remote UE from different PLMNs discover each other, it means that the Relay Service Code is associated with the same connectivity service, and the same Relay Service Code is provisioned based on Service Level Agreement among PLMNs.

NOTE 3:	The Hop count, Hop-Limit and Path information are used for multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B. The multi-hop UE-to-Network Relay Discovery message has no impact on 5G ProSe UE-to-Network Relay or 5G ProSe Remote UE that only supports the single-hop UE-to-Network Relay. Single-hop and Multi-hop UE-to-Network Relays discoveries can be distinguished based on RSC.

# * * * * End of changes * * * *