3GPP TSG-WG SA2 Meeting #166	S2-2412249

# 18 – 22 November 2024, Orlando, USA

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0514 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on ProSe Multihop U2N Relay reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SA2#164 it is approved that the relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay, taking the role of the 5G ProSe Remote UE in the procedure described in 6.4.3.8.3. But when the 5G ProSe Intermediate UE-to-Network Relay can not reselect the relay path, e.g. it is not able to discover the 5G ProSe UE-to-Network Relay, it is better for the 5G ProSe Intermediate UE-to-Network Relay to notify 5G ProSe Remote UE to trigger the reselection of the relay path, considering the new path reselected by 5G ProSe Remote UE always has less hops than the new path reselected by any of the 5G ProSe Intermediate UE-to-Network Relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add procedures of 5G ProSe Intermediate UE-to-Network Relay notifies 5G ProSe Remote UE to trigger reselection of the relays. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Relay reselection will not succeed if the 5G ProSe Intermediate UE-to-Network Relay can not complete the reselecttion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.4.3.8.3	Relay reselection and mobility support

There are two types of situations for parent relay reselection, i.e. for a single 5G ProSe Remote UE, and for a 5G ProSe Intermediate UE-to-Network Relay with multiple child relays or 5G ProSe Remote UEs.

For a single 5G ProSe Remote UE, when the connection with an old parent relay (either a 5G ProSe Intermediate UE-to-Network Relay or a 5G ProSe UE-to-Network Relay) is lost or the link quality drops below a configured threshold, the 5G ProSe Remote UE triggers the reselection of the relays that offers the same service, identified by the same RSC.

Figure 6.4.3.8.3-1: Relay reselection for 5G ProSe Remote UE with 5G ProSe Multi-hop UE-to-Network Relays

Figure 6.4.3.8.3-1 depicts the Relay reselection operation for this situation. When the 5G ProSe Remote UE has lost the connection or has an imminent connection loss/deteriorated connection with the 5G ProSe Intermediate UE-to-Network Relay-1, the 5G ProSe Remote UE performs relay reselection by using the relay discovery procedure in clause 6.3.2.5. The 5G ProSe Remote UE discovered the 5G ProSe Intermediate UE-to-Network Relay-2 that offers the same RSC. In case there are multiple 5G ProSe Intermediate UE-to-Network Relays discovered, the 5G ProSe Intermediate UE-to-Network Relay that offers connection to the same 5G ProSe UE-to-Network Relay is prioritized.

5G ProSe Remote UE establishes the Layer-2 link with the 5G ProSe Intermediate UE-to-Network Relay-2 as specified in clause 6.4.3.8.1. After the connection establishment, the Intermediate UE-to-Network Relay-2 uses the Layer-2 Link Modification procedure to update the 5G ProSe UE-to-Network Relay regarding the 5G ProSe Remote UE. If the same 5G ProSe UE-to-Network Relay is used, it switches the connection/routing context for the 5G ProSe Remote UE from the Layer-2 link with the 5G Intermediate UE-to-Network Relay-1 to that of the Layer-2 link with the 5G ProSe Intermediate UE-to-Network Relay-2.

The 5G ProSe Remote UE tears down the link with the 5G ProSe Intermediate UE-to-Network Relay-1 (or the link failure is detected), which will trigger the 5G ProSe Intermediate UE-to-Network Relay-1 to update the 5G ProSe UE-to-Network Relay to remove its association with 5G ProSe Remote UE.

If the same 5G ProSe UE-to-Network Relay is selected after the relay reselection, the 5G ProSe Remote UE can keep its IP address/prefix (in case of Layer-3 UE-to-Network relay).

If a new 5G ProSe UE-to-Network Relay is used after the relay reselection, the procedures as that described in clause 6.4.3.8.1 will be executed. In this case, IP address/prefix changes may happen (in case of Layer-3 UE-to-Network relay).

The relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay, taking the role of the 5G ProSe Remote UE in the procedure, when its connection with an old parent relay (either a 5G ProSe Intermediate UE-to-Network Relay or a 5G ProSe UE-to-Network Relay) is lost or the link quality drops below a configured threshold.

If the 5G ProSe Intermediate UE-to-Network Relay can not complete the reselection procedure, e.g. it is not able to discover the 5G ProSe UE-to-Network Relay, or the 5G ProSe Intermediate UE-to-Network Relay determines to not perform the reselection procedure, e.g. by configuration, it sends reselection indication and optionally with a cause value to 5G ProSe Remote UE via the established multipath relay. After receving the reselection indication the 5G ProSe Remote UE triggers the reselection of the relays as described above.

During the transmition of reselection indication to 5G ProSe Remote UE, after receving the reselection indication a 5G ProSe Intermediate UE-to-Network Relay in the path based on configuration can determine to relay the reselection indication to 5G ProSe Remote UE, or perform relay reselection by iteself, e.g. if it already has another path to 5G ProSe UE-to-Network Relay of the same RSC.

# * * * * End of changes * * * *