3GPP TSG SA WG2#166	S2-2412258

# Orlando, US, 18 – 22 November 2024        (revision of S2-2409182, 9147, 8661, 7903, 11190)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0241 | rev | 12 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Support of Local Offloading Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Huawei, HiSilicon], Nokia, [Samsung, China Mobile, CATT, LG Electronics, Ericsson] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusion as described in the cluase 8.1 of TR 23.700-49, I-SMF based approach is concluded for the key issue of the Session Breakout model. This contribution proposes to name this scenario as “Local Offloading Management” and add corresponding descriptions into the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add a new clause 6.X for support of the local traffic routing with I-SMF for reducing impact on central SMF . 2. Add supporting Local Offloading Management related descriptions in the clause of abbreviations, architecture and EASDF service. 3. Provide details on I-SMF insertion during and after PDU session establishment in 6.X.2. |  |  |  |  |  |  |  |  |
| ` |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | I-SMF based solution for Local Offloading Management to reduce the impact on central SMF is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 6.2.1, 6.X(new), 6.X.1(new), 6.X.2(new), 7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev12: changes on the text are highlighted in yellow - Removes ENs on details of I-SMF insertion procedure. - Provide details on I-SMF insertion during (6.X.2.2) and after (6.X.2.3) PDU session establishment in 6.X.2. |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 4.2	Reference Architecture for Supporting Edge Computing

The reference architectures for supporting Edge Computing are based on the reference architectures specified in clause 4.2 of TS 23.501 [2]. The following reference architectures for non-roaming, LBO roaming and HR with Session Breakout (HR-SBO) roaming scenarios further depict the relationship between the 5GS and a DN where Edge Application Servers (EASs) are deployed in an EHE.

Figure 4.2-1 depicts 5GS architecture for non-roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-1: 5GS providing access to EAS with UL CL/BP for non-roaming scenario

Figure 4.2-2 depicts 5GS architecture for non-roaming scenario supporting Edge Computing without UL CL/BP.

Figure 4.2-2: 5GS providing access to EAS without UL CL/BP for non-roaming scenario

Figure 4.2-3 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-3: 5GS providing access to EAS with UL CL/BP for LBO roaming scenario

Figure 4.2-4 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing without UL CL/BP.

Figure 4.2-4: 5GS providing access to EAS without UL CL/BP for LBO roaming scenario

Figure 4.2-5 depicts 5GS architecture for HR-SBO roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-5: 5GS providing access to EAS with UL CL/BP for HR-SBO roaming scenario

NOTE 1:	Only some of the 5GS NFs are shown in the above reference architecture figures. In the above figures, the split between the UPF acting as UL CL/BP and the UPF acting as local PSA is illustrative.

NOTE 2:	Only the control plane of EASDF is depicted in the figure, the user plane between the EASDF and the UPF (i.e. over which the DNS messages are exchanged) is part of N6. Additionally, the EADSF may have direct connectivity with the local parts of one or more Data Networks.

NOTE 3:	For the HR-SBO roaming scenario, there can be other UPF(s) located in VPLMN between the UPF acting UL CL/BP and the UPF acting as remote PSA in HPLMN.

NOTE 4:  For Local Offloading Management scenario, it is I-SMF that selects and configures EASDF.

# * * * * next change * * * *

### 6.2.1	General

In Edge Computing deployment, an application service may be served by multiple Edge Application Servers typically deployed in different sites. These multiple Edge Application Servers that host service may use a single IP address (anycast address) or different IP addresses. To start such a service, the UE needs to know the IP address(es) of the Application Server(s) serving the service. The UE may do a discovery to get the IP address(es) of a suitable Edge Application Server (e.g. the closest one), so that the traffic can be locally routed to the Edge Application Server and service latency, traffic routing path and user service experience can be optimized.

EAS discovery is the procedure by which a UE discovers the IP address(es) of a suitable Edge Application Server(s) using Domain Name System (DNS). EAS Re-discovery is the EAS Discovery procedure that takes place when the previously discovered Edge Application Server cannot be used or may have become non-optimal (e.g. at edge relocation).

The DNS server to be used for EAS (re-)discovery may be deployed in different locations in the network as Central DNS (C-DNS) server or as Local DNS (L-DNS) resolver/server.

NOTE 1:	The C-DNS servers and/or L-DNS resolvers/servers can use an anycast address.

NOTE 2:	The C-DNS servers or L-DNS resolvers/servers can contact any other DNS servers for recursive queries, which is out of scope of this specification.

NOTE 3:	This specification describes the discovery procedure based on 5GS NFs as to ensure the UE is served by the application service closest to the UE's point of attachment. However, this does not exclude other upper layer solution that can be adopted by operator or service provider, like the EAS Discovery procedure defined in TS 23.558 [5], or other alternatives shown in Annex A and Annex B. How those other solutions work, or whether they are able to guarantee the closest application service for the UE, is out of the scope of this specification.

In order to provide a translation of the FQDN of an EAS into the address of an EAS as topologically close as possible to the UE, the Domain Name System may use following information:

-	The source IP address of the incoming DNS Query; and/or,

-	an EDNS Client Subnet (ECS) option (as defined in RFC 7871 [6]).

NOTE 4:	UE IP address can be subject to privacy restrictions, which means that it is not to be sent to Authoritative DNS / DNS Resolvers outside the network operator within EDNS Client Subnet option or as Source IP address of the DNS Query. UE source IP address can be protected by using NAT mechanism.

EAS (re-)discovery procedures described in this specification should use the top level domains (TLDs) in the public namespace by default.

If a private namespace is used, an Edge Computing Service Provider (ECSP) can provision DNS information in the EAS Deployment information via AF request with its Application Identifier, or DNN and NSSAI. Since private namespaces do not have a common root server or naming, the DNS information for each ECSP should be stored individually to prevent any overwriting of resolution entries.

NOTE 5:	The DNS information provided by ECSP in the EAS Deployment Information can be used to select the DNS settings for a PDU Session mainly if the PDU Session is specific for the ECSP services.

If the UE applications want to discover/access EAS by using the mechanisms defined in this TS, the UE shall support receiving DNS settings in PCO during PDU Session Establishment and PDU Session Modification, and the DNS Queries generated by the UE for these applications shall be sent to the DNS server/resolver (e.g. EASDF) indicated by the SMF. To ensure this, the application in the UE either requests the EDC functionality to send a DNS Query or, alternatively, uses the EDC functionality to get the configuration of the DNS Server (IP address and, conditionally, DNS security information of the EASDF/DNS Server/ Resolver; see TS 24.501 [11] and TS 33.501 [12]) indicated by the SMF (see clauses 5.2.1 and 6.2.4) then resolves the FQDN by its own DNS mechanism.

For EASDF, the DNS security information may be provided by EASDF to SMF. The DNS security information of the EASDF/Local DNS Server/Resolver may also be locally configured in the SMF.

NOTE 6:	The DNS security information of Local DNS Server/Resolver can be locally configured in the SMF if they belong to same security domain.

NOTE 7:	It is the decision of the application in the UE whether to use the EDC functionality or not to resolve the FQDN. If it does not use the EDC functionality, the usage of the EAS (re-)discovery procedures defined in clause 6.2 cannot be ensured.

The case of EAS (Re-)discovery over Distributed Anchor connectivity model is described in clause 6.2.2. For Multiple PDU Sessions connectivity model, the description in clause 6.2.2 also applies to the PDU Session(s) with Local PSA. The case of EAS (Re-)discovery over Session Breakout connectivity model is described in clause 6.2.3. When I-SMF based Local Offloading Management applies, the SMF in EAS (Re-)discovery over Session Breakout connectivity model described in 6.2.3 is replaced by I-SMF.

In this release, the I-SMF based Local Offloading Management does not cover common DNAI/EAS selection. Hence, replacing the SMF with I-SMF does not apply to clauses 6.2.3.2.4, 6.2.3.2.5, 6.2.3.2.6, and 6.2.3.2.7.

# * * * * next change * * * *

## 6.X	Support of the Local Offloading Management

### 6.X.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. For Session Breakout connectivity model, AMF may select/re-select an I-SMF to support Local Offloading Management in case Local Offloading Management is allowed in the subscription data. For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message handling, as defined in clauses 6.2.1 and 6.2.3, instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF as described in clause 6.3.23 of TS 23.501 [1].

-	Management and retrieval of EDI from NEF as described in 6.2.3.4.

NOTE X:	The SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Information from PCF via SMF, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE Y:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, i.e., the inserted I-SMF providing Local Offloading allowed indication to SMF, the SMF does not discover, select and configure EASDF as well as DNS message handling for the locally offloaded traffic towards the local part of DN.

### 6.X.2	Procedure

#### 6.X.2.1	General

This clause describes the I-SMF insertion and EAS (re)discovery procedure for Local Offloading Management.

#### 6.X.2.2	I-SMF insertion during the PDU Session establishment

This clause defines procedure for triggering I-SMF insertion for Local Offloading Management. The procedure described in clause 4.23.5.1 of TS 23.502 [3] is performed with some modifications. The details of these modifications are shown in Figure 6.X.2.2-1 and further described below.

Figure 6.X.2.2-1: SMF triggered I-SMF selection for Local Offloading Management during PDU session establishment procedure

- Steps from 1 to 10 in PDU session establishment procedure for non-roaming scenario defined in Figure 4.3.2.2.1-1 of TS 23.502 [3] are followed with the following enhancements:

- Step 4: as part of Session Management (SM) Subscription data retrieval from UDM, SMF retrieves Local Offloading Management allowed indication for the DNN/S-NSSAI of the PDU session.

- Step 7/9: As part of SM Policy Association Establishment/Modification, PCF provides PCC rules. The PCC rules may indicate a target DNAI or a list of DNAIs that is for Local Offloading Management.

- Based on the SM subscription data and PCC rules, the SMF determines whether I-SMF for Local Offloading Management should be triggered by considering:

- whether the UE has a Local Offloading Management allowed indication in SM Subscription data;

- whether the UE capability supports EDC functionality described in clause 5.2; and

- whether there is target DNAI(s) that is for Local Offloading Management.

NOTE:	The SMF determines the target DNAI which is applicable to the current UE location and which can be based on the common DNAI (if applicable) as defined in clause 5.6.7 of TS 23.501 [2], clauses 4.3.6 and 4.23.5.4 of TS 23.502 [3], and clause 6.2.3.2.2.

If all the given conditions above are true, SMF determines that there is a need to trigger I-SMF insertion for Local Offloading Management, and the SMF proceeds with step 3. Otherwise, the remaining steps in this Figure are not followed. Instead, the procedure given in Figure 4.3.2.2.1-1 of TS 23.502 [3] from step 11 and onwards is followed.

- The SMF invokes a Nsmf_PDUSession_SMContextStatusNotify service as in Step 2a of Figure 4.23.5.4-1 of TS 23.502 [3] with the following enhancements:

- the target DNAI information includes a target DNAI and Local Offloading Management allowed indication. This is to trigger the AMF to select a suitable I-SMF for the PDU Session. The target DNAI and the Local Offloading Management allowed indication are used for selecting an I-SMF, which supports Local Offloading Management and controls UPF connecting to the target DNAI, to offload edge computing related information management.

- Steps from 2 to 5 in Figure 4.3.2.2.2-1 of TS 23.502 [3] are followed with the following enhancements:

- Step 2: During SMF selection, the AMF queries NRF to retrieve a list of SMF(s) profiles, and determine the I-SMF that can serve the target DNAI, UE location and support Local Offloading Management.

NOTE:		The target DNAI is not supported by the anchor SMF but may be supported by multiple I-SMFs.

In case there is no I-SMF that supports the selection criteria considered by the AMF, the remaining steps in this Figure are not followed. Instead, the procedure given in Figure 4.3.2.2.1-1 of TS 23.502 [3] from step 11 and onwards is followed.

- Step 3: AMF sends Nsmf_PDUSession_CreateSMContext request including Local Offloading Management allowed indication.

- Step 4: Before I-SMF performs UPF selection., I-SMF retrieves EAS Deployment Information from NEF, as described in clause 6.2.3.4.3.

- Based on the Local Offloading Allowed indication, the selected I-SMF proceeds with EASDF selection and retrieves EASDF information including EASDF IP address and DNS security information.

- The I-SMF invokes Nsmf_PDUSession Create request to SMF. The request includes the EASDF information.

- Step 12 in Figure 4.3.2.2.2-1 of TS 23.502 [3] is followed.

- When SMF responds to Nsmf_PDUSession Create request, SMF provides Local Offloading Management policy to I-SMF.

- The I-SMF generates Local Offloading Management information based on the policy received from SMF and configures EASDF based on EAS Deployment Information and Local Offloading Management information.

- The remainder of the steps from 13a onwards in Figure 4.3.2.2.2-1 of TS 23.502 [3] are followed.

#### 6.X.2.3	I-SMF Insertion After PDU Session Establishment

This clause defines procedure for triggering I-SMF insertion or removal for Local Offloading Management after a PDU session is established with the anchor SMF and/or an I-SMF is already inserted when the UE is outside the anchor SMF service area. The procedure described in clause 4.23.5.4 of TS 23.502 [3] is performed with some modifications. The details of these modifications are shown in Figure 6.X.2.3-1 and further described below.

Figure 6.X.2.3-1: SMF triggered I-SMF insertion or removal for Local Offloading Management after PDU session establishment

- During the PDU Session Establishment procedure in clause 4.23.5.1 of TS 23.502 [3], SMF receives Local Offloading Management allowed indication from UDM.

- Similar to step 1 of Figure 4.23.5.4-1 of TS 23.502 [3], the PCF sends to the SMF, PCC rule(s) applicable to the ongoing PDU session by invoking Npcf_SMPolicyControl_UpdateNotify service operation. The PCC rule(s) includes a target DNAI or a list of DNAIs that is/are for Local Offloading Management.

- Similar steps 2 and 3 of Figure 6.X.Y.1-1 with the following enhancement in Step 2:

  - if an I-SMF is already in use for the PDU Session and the SMF decides that the SMF itself can serve the target DNAI that is not allowed for local offloading management, the existing I-SMF can be removed

- Similar to step 4 of Figure 6.X.Y.1-1 with the following enhancement:

For the I-SMF removal, AMF removes the existing I-SMF if:

  - the AMF receives from SMF Nsmf_PDUSession_SMContextStatusNotify without including target DNAI;

  - the AMF identifies that the SMF can serve the target DNAI that does not have Local Offloading Management allowed indication; or

  - the existing I-SMF cannot serve the UE location (e.g. due to mobility).

- Similar to step 4 of Figure 4.23.5.4-1 in TS 23.502 [3] with the following enhancement:

Nsmf_PDUSession_CreateSMContext Request to the new I-SMF includes Local Offloading Management allowed indication (step 5a of Figure 6.X.Y.2-1).

- Similar to step 5 of Figure 4.23.5.4-1 in TS 23.502 [3] with the following enhancements:

Based on the Local Offloading Allowed indication, the selected I-SMF:

retrieves EAS Deployment Information from NEF, as described in clause 6.2.3.4.3, and proceeds with UPF selection; and

proceeds with EASDF selection and retrieves EASDF information including EASDF IP address and DNS security information.

The I-SMF sends PDU session create request to SMF including the EASDF information and Local Offloading Management support indication.

Based on the I-SMF request, SMF retrieves Local Offloading Management policy from PCF and provides the policy as part of PDU session create response.

I-SMF generates Local Offloading Management information based on the policy received from SMF.

- I-SMF proceeds with EASDF configuration based on EAS Deployment Information and Local Offloading Management information.

- The step 6 of Figure 4.23.5.4-1 in TS 23.502 [3] is followed.

#### 6.X.2.4	EAS Discovery Procedure with EASDF for Local Offloading Management

I-SMF

EASDF

UE

DNS server

UPF

ULCL/BP

UPF

L-PSA

1. DNS query

2. The step 8 to 15 of the procedure in clause 6.2.3.2.2 by replacing SMF with I-SMF

UPF

L-PSA

SMF

3. ULCL/BP insertion with interaction between I-SMF and SMF

4. The step 17 to 18 of the procedure in clause 6.2.3.2.2 by replacing SMF with I-SMF

5. DNS response

Figure 6.X.2.3-1: Procedure for EAS Discovery with EASDF for Local Offloading Management PDU session

Editor’s Note: The details of differences compared to clause 6.7.2.3 are FFS.

# * * * * next change * * * *

## 7.1	EASDF Services

### 7.1.1	General

The following table illustrates the EASDF Services and Service Operations.

Table 7.1.1-1: NF services provided by the EASDF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Neasdf_DNSContext | Create | Request/Response | SMF/I-SMF |
|  | Update | Request/Response | SMF/I-SMF |
|  | Delete | Request/Response | SMF/I-SMF |
|  | Notify | Subscribe/Notify | SMF/I-SMF |
| Neasdf_BaselineDNSPattern | Create | Request/Response | SMF/I-SMF |
|  | Update | Request/Response | SMF/I-SMF |
|  | Delete | Request/Response | SMF/I-SMF |

# * * * * End of changes * * * *