3GPP TSG-WG SA2 Meeting #166	S2-2412290

# Orlando, USA, 18th Nov – 22th Nov, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5876 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR Femto access to local services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to support local services, a UPF can be deployed in the local network and used to access the local services. The selection of this local UPF can be based on dedicated S-NSSAI/DNN pair Therefore, no other enhancement is needed for the local services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 5.50.1 to clarify the support of local services Add a new annex to clarify how the exsiting mechanism support local services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functions and inaccurate description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.50.1, Annex X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.50.1	Overview

This clause provides an overview of 5GS functionalities and architecture to support 5G Femto deployments.

Editor's note:	The architecture and impacts will depend on the RAN WG3 output.

The existing CAG concept defined in the clause 5.30.3 for PNI-NPN is used for 5G Femto access control. The Femto cell may broadcast one or multiple CAG Identifiers as specified in the clause 5.30.3 if access control is needed. The enhancements enable the CAG owner or an authorized administrator to control/provision subscribers to allow accessing their 5G Femto cells.

For the support of NR Femto local services, a dedicated S-NSSAI/DNN pair can be used.

NOTE X:	Whether an dedicate UPF is needed for local service is depend on implementation.

# * * * * Next changes * * * *

Annex X (informative):
NR Femto

This annex provide information on how the NF Femto may be deployed and used by means of functionalities defined in clause 5.50 and existing functionalities.

## A.1 Access to local services

There are two scenarios of local services:

-	Corporate scenario: Several NR Femto and optionally a local UPF;

-	Home scenario: One NR Femto and optionally a local UPF, the UPF can be collocated with the NR Femto.

A dedicated S-NSSAI/DNN pair can be used for allowing the selection of the UPF deployed locally with NR Femto and to establish a PDU session toward such UPF to acces local services. The PCF provides the corresponding URSP rules to the UE. The Route Selection Validation Criteria of URSP may also be configured to limit the validity of the URSP rule fo rteh dedicated S-NSSAI/DNN in the are where the NR Femto is deployed, i.e the dedicated S-NSSAI/DNN pair is only supported by the Femto cell.

The operator configures the SMF to store the association between the dedicated S-NSSAI/DNN pair and the local UPF. When the UE requests a PDU Session for local service by using the dedicated S-NSSAI/DNN pair based on the URSP rules, the SMF shall select the corresponding UPF based on the configuration.

In  case of residential deployment where the IP connectivity is provided via fixed access network, i.e. the Femto is conncted via a RG. IN such scenario often the Femto is connected to the Hone local network where a private Ip addressis used an dteh RG support a NAT, furthermore the WAN IP address assigned to the RG is also dynamic. In order to to decouple the networking IP of the underlying fixed access network form the 5GC the architecture defined in clause 5.50.2 us define where the deployment of a SeGW and the establishment of IP tunnel from the NF femto and the UPF allows to use as inner tunnel information IP address belonging to the 5GC.

# * * * * End of changes * * * *