3GPP TSG-SA WG2#166	S2-2412314

Orlando, FL, USA, 18-22 November 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	UE Detach for S&F Mode Access

Document for:	Discussion/Approval

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3_ARCH/Rel.19

Abstract of the contribution: Discusses support of a UE Detach following UE access to a satellite in S&F mode

# 1.	Discussion

In the SA2#165 meeting it was not possible to complete and approve the 23.401 CR3801 on 5GSAT_ARCH_Ph3 (S2-2410989) which was consequently postponed. One omission in this CR is support of an optional detach for a UE after access to a satellite in S&F mode is lost.

Conclusions of TR 23.700-29 in clause 8.2 state the following (with text related to UE detach highlighted):

The following option is agreed for supporting Store and Forward operation with a full CN onboard the satellite with the following (informative) principles: - The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. - The implementation of the proxies and the interface between them is out of 3GPP scope. - The UE attaches, transfers data (e.g. SMS, MO and MT data, etc.) and detaches from each satellite as required and as determined by the monitoring list. NOTE 5: MT traffic is delivered to the UE after it performs an ATTACH. NOTE 6: After mobility from S&F operation, MT traffic could be stuck in the ground proxy and will only be retrieved once the UE goes back to S&F operation. For MT traffic, the UE attaches to a satellite and to allow delivery of MO traffic from the user or applications on the UE. A UE, based on implementation, could first wait for an indication from the user or from an application on the UE of pending MT traffic or could wait based on knowledge of when MT traffic may arrive, before performing the attach. - Depending on the deployment and implementation (i.e. outside the scope of 3GPP in this release), the HSSs on the satellites may be populated with subscription data either for only the UEs that may access satellite or all UEs that may access the satellite. - Depending on the deployment, the UE may have a USIM enhanced for IOPS, or a USIM dedicated to the satellite network. NOTE 7: The solution does not support the roaming architectures defined in TS 23.501 [2] or TS 23.401 [5].

Observation 1: In TR 23.700-29 conclusions, a UE is allowed to Detach from a satellite for the full EPC solution.

A detach may be performed by the onboard MME shortly before the UE will lose coverage.  Alternatively, if coverage is lost before the detach has occurred, the UE may detach locally as defined in TS 23.401. However, the UE would need to know that a detach is needed in order for this to occur because, for the split MME solution, a UE does not detach and the UE probably does not know whether the satellite uses the full EPC solution or split MME solution. An indication of the need for a detach can be conveyed to the UE by the satellite in a previous NAS procedure – e.g. an Attach, TAU or SR.

Observation 2: The UE Detach can be performed by the onboard MME or locally by the UE after coverage is lost. However for the UE to know that a detach is required, the onboard MME needs to send an indication of this to the UE in a previous NAS procedure.

Proposal: To enable a UE to detach locally when coverage is lost, it is proposed to enable an onboard MME to send an indication of this to the UE in a previous NAS procedure.

# 2.	Text proposal

It is proposed to agree the following addition to 23.401 CR3801 on 5GSAT_ARCH_Ph3 (S2-2410989).

# >>>>BEGINNING OF CHANGES<<<<

### 4.13.x	Support of Store and Forward Satellite Operation

#### 4.13.x.1	General

. . .

. . .

Before a UE that is attached in S&F mode loses coverage, the MME may initiate a detach procedure with the UE. If the UE loses coverage before the detach procedure occurs and if the MME has indicated in a previous NAS procedure that a detach is needed, the UE shall detach locally, as defined in TS 24.301 [46]. The UE may later perform a new Attach for the same PLMN according to the S&F Monitoring List and S&F Wait Timer.

NOTE x:	Detaching the UE after loss of coverage avoids having to synchronise a new NAS security state for the UE among other satellites.

Editor's note:	CT1 needs to define NAS support in TS 24.301 [46] for detaching a UE either before or after losing coverage in S&F mode.

# >>>>END OF CHANGES<<<<