3GPP TSG-SA2 Meeting #166	S2-2412353

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1175 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Updating FL to HFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Exisiting Procedure in the present TS using terminology of FL are based on Horizontal Federated learning (HFL). The addition of Vertical Federated Learning might cause confusion among readers |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change FL to HFL, Federated Learning to Horizontal Federaed learning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear which FL procedures relate to HFL of VFL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3, 6.2C, 6.2F, 7.10, 7.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 5.3	Horizontal Federated Learning (HFL) among multiple NWDAFs

Federated learning among multiple NWDAFs is a machine learning technique in core network that trains an ML Model across multiple decentralized entities holding local data set, without exchanging/sharing local data set. This approach stands in contrast to centralized machine learning techniques where all the local datasets are uploaded to one server, thus allowing to address critical issues such as data privacy, data security, data access rights.

NOTE 1:	Horizontal Federated Learning is supported among multiple NWDAFs, which means the local data set in different HFL client NWDAFs have the same feature space for different samples (e.g. UE IDs).

For Horizontal Federated Learning supported by multiple NWDAFs containing MTLF, there is one NWDAF containing MTLF acting as HFL server (called HFL server NWDAF for short) and multiple NWDAFs containing MTLF acting as HFL client (called HFL client NWDAF for short), the main functionality includes:

HFL server NWDAF:

-	discovers and selects HFL client NWDAFs to participant in an HFL procedure

-	requests HFL client NWDAFs to do local model training and to report local model information.

-	generates global ML Model by aggregating local model information from HFL client NWDAFs.

-	sends the global ML Model back to HFL client NWDAFs to perform an additional training iteration if needed.

HFL client NWDAF:

-	locally trains ML Model as tasked by the HFL server NWDAF with the available local data set, which includes the data that may not be allowed to be shared with other HFL client NWDAFs due to e.g. data privacy, data security, data access rights.

-	reports the trained local ML Model information to the HFL server NWDAF.

-	receives the global ML Model from HFL server NWDAF and perform an additional training iteration if needed.

HFL server NWDAF or HFL client NWDAF register to NRF with their HFL capability information as described in clause 5.2.

The NWDAF containing MTLF determines to train an ML Model either based on local configuration or when it receives a request from NWDAF containing AnLF. The NWDAF containing MTLF further determines whether the ML Model should be trained via HFL mechanism based on Analytic ID, Service Area/DNAI or when data can not be obtained directly from data producer NF (e.g. due to data privacy, data security). The NWDAF containing AnLF is not aware whether the ML Model is trained based on HFL or not.

If the NWDAF containing MTLF can act as an HFL server for the ML Model training, then HFL procedure is initiated by the NWDAF containing MTLF as HFL server NWDAF directly.

If the NWDAF containing MTLF determines to train an ML Model based on local configuration and the HFL mechanism is required, but the NWDAF containing MTLF can't act as an HFL server, the NWDAF containing MTLF should discover an HFL server NWDAF as described in clause 5.2 and request the HFL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The HFL server NWDAF may determine to initiate HFL procedure before providing the ML Model.

If the ML Model training is triggered by the request from NWDAF containing AnLF, the NWDAF containing MTLF determines the HFL mechanism is required but it can not act as an HFL server, the NWDAF containing MTLF should discover an HFL server NWDAF as described in clause 5.2 and request the HFL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The Notification Target Address and the Notification Correlation ID from the NWDAF containing AnLF is provided in the request message sent to the HFL server NWDAF. The HFL server NWDAF may determine to initiate HFL procedure before providing the ML Model. The HFL server NWDAF sends the ML Model information to the notification endpoint (e.g. the NWDAF containing AnLF) after the ML Model training success.

NOTE 2:	The security procedure on authorizating HFL server to initiate HFL procedure on the HFL client(s) is described in Annex X, clause X.9 of TS 33.501 [49]. The security procedure authorizing an MTLF to request ML Models on behalf of an AnLF to another MTLF (e.g., HFL server NWDAF) is described in Annex X, clause X.10 of TS 33.501 [49].

Before HFL procedure is initiated by HFL server NWDAF, appropriate HFL client NWDAFs should be discovered by HFL server NWDAF as described in clause 5.2.

When starting an HFL procedure, the HFL server NWDAF is to provide an initial model to each HFL client NWDAF, and then each HFL client NWDAF is to perform local model training using its local data set. The detailed procedure for HFL among Multiple NWDAFs is described in clause 6.2C.

2nd change

## 6.2C	Horizontal Federated Learning among Multiple NWDAFs

### 6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Federated Learning technique to train an ML Model.

### 6.2C.2	Procedures

#### 6.2C.2.1	Registration and Discovery procedure for Federated Learning

HFL Server NWDAF

NRF

NWDAF Registration

HFL Client NWDAF 1

...

HFL Client NWDAF N

1b. Nnrf_NFManagement_NFRegister Request (HFL Client NWDAF profile)

3b. Nnrf_NFManagement_NFRegister Response

1a. Nnrf_NFManagement_NFRegister Request (HFL Server NWDAF profile)

3a. Nnrf_NFManagement_NFRegister Response

7. HFL preparation request

9. Response to HFL Server NWDAF

8. Decide to join or not

10. Select HFL Client NWDAF(s)

2. Store HFL Server NWDAF and HFL Client NWDAF profile

5. Authorize NF Service Discovery

NWDAF Discovery

4. Nnrf_NFDiscovery_Request Request

6. Nnrf_NFDiscovery_Request Response (one or multiple HFL Client NWDAF instances, i.e., HFL Client NWDAF 1, …, N)

Figure 6.2C.2.1-1: Registration and Discovery procedure for Horizontal Federated Learning

Steps 1 to 3 are the NWDAF registration procedure.

1-3.	NWDAF containing MTLF as HFL Server NWDAF or HFL Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, HFL capability type information (i.e. HFL server and/or HFL client) and Time interval supporting HFL as described in clause 5.2.

Steps 4 to 6 are the NWDAF Discovery procedure.

4-6.	NWDAF containing MTLF determines ML Model requires HFL based on operator policy (e.g. pre-configured list of ML Models), Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as HFL Server NWDAF, the MTLF first discovers and selects HFL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML Model required, Model filter information as defined in clause 6.2A.2, HFL capability Type (i.e. HFL server), Time Period of Interest, Service Area.

Once the HFL Server NWDAF (the requested or the selected one) is determined, the HFL Server NWDAF discovers and selects other NWDAF(s) containing MTLF as HFL Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML Model required, HFL capability Type (i.e. HFL client), Service Area, NF type(s) of data sources from which the HFL Client NWDAF is able to collect data for local ML Model training, Time Period of Interest, ML Model Interoperability Indicator.

7.	HFL Server NWDAF sends Federated Learning preparation request to the HFL Client NWDAF(s), using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag, to check if the HFL Client NWDAF(s) can meet the ML Model training requirement (e.g. Analytics ID, ML Model Interoperability information, Data Availability requirement, HFL Availability time requirement (time span needed for the HFL process), etc.). Data Availability requirement includes a list of Event IDs of the local data for training, and may also include the dataset statistical properties, the time window of the data samples and the minimum number of data samples.

NOTE:	Federated Learning preparation procedure (i.e. steps 7-9) can be skipped if the HFL Server NWDAF can decide that the HFL Client NWDAF(s) supports the HFL procedure to be performed, e.g. based on information acquired from previous HFL procedures or from the NRF, or based on local configuration.

8.	HFL Client NWDAF(s) checks if it can meet the ML Model training requirement and/or can successfully download the model if the model information is provided in the request and decides whether to join the Federated Learning process based on operator policy (e.g. pre-configured list of ML Models) and/or implementation. Example criteria used by HFL Client NWDAF(s) may be based on its data availability and time availability, computation and communication capability and ML Model Interoperability information.

9.	HFL Client NWDAF(s) invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTraining_Subscribe response or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate to the HFL Server NWDAF whether it will join the HFL procedure and may include the reason in the response message if it cannot join the HFL process.

10.	HFL Server NWDAF determines the HFL Client NWDAFs to be involved in the HFL procedures based on the information received in step 6 and other information received in step 9 (if available).

#### 6.2C.2.2	General procedure for Horizontal Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.2-1: General procedure for Horizontal Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF or NWDAF containing MTLF) sends a subscription request to HFL server NWDAF to retrieve an ML Model, using Nnwdaf_MLModelProvision including Analytics ID, ML Model Monitoring information as defined in clause 7.5.2, desired ML Model metric (e.g. ML Model Accuracy).

NOTE 1:	The ML Model Accuracy threshold can be used to indicate the target ML Model Accuracy of the training process and the HFL server NWDAF may stop the training process when the ML Model Accuracy threshold is achieved during the training process.

If the consumer (i.e. the NWDAF containing AnLF or NWDAF containing MTLF) provides the Time when the ML Model is needed, the HFL Server NWDAF can take this information into account to decide the maximum response time for its HFL Client NWDAF(s).

1.	HFL Server NWDAF selects NWDAF(s) containing MTLF (FL Client NWDAF(s)) as described in clause 6.2C.2.1.

2.	HFL Server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF(s) containing MTLF (HFL Client NWDAF(s)), which participate in the Federated learning to perform the local model training and determine the interim local ML Model information based on the input parameter in the request from HFL Server NWDAF. The request includes the desired ML Model metric and initial ML Model and also includes the maximum response time, the HFL Client NWDAF has to report the interim local ML Model information to the HFL Server NWDAF before the maximum response time elapses.

3.	[Optional] Each HFL Client NWDAF collects its local data by using the current mechanism in clause 6.2 if the Client NWDAF has not local data available already.

4.	During Horizontal Federated Learning training procedure, each HFL Client NWDAF further trains the ML Model provided by the HFL Server NWDAF based on its local data and reports the interim local ML Model information to the HFL Server NWDAF in Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response. The Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response may also include the Status report of HFL training that includes local ML Model metric value (and optionally the used metric) computed by the HFL Client NWDAF and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension of data, etc.) in the HFL Client NWDAF. The Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Response also includes the global ML Model metric (value (and optionally the used metric) when the ML Model Accuracy Check Flag was included in the Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request (as described in step 7), the global ML Model metric value is calculated by the HFL Client NWDAF using the local training data as the testing dataset.

NOTE 2:	The parameters in characteristics of local training dataset are up to the implementation.

The local ML Model, which is sent from the HFL Client NWDAF(s) to the HFL Server NWDAF during the HFL training process, is the information needed by the HFL Server NWDAF to build the aggregated model.

If the HFL Client NWDAF is not able to complete the training of the interim local ML Model within the maximum response time provided by the HFL Server NWDAF, the HFL Client NWDAF shall send the Delay Event Notification that include the delay event indication, an optional cause code (e.g. local ML Model training failure, more time necessary for local ML Model training) and the expected time to complete the training if available to the HFL Server NWDAF before the maximum response time elapses.

4a.	[Optional]If HFL Server NWDAF receives notification/response that the HFL Client NWDAF is not able to complete the training within the maximum response time, the HFL Server NWDAF may send to the HFL Client NWDAF a new maximum response time in Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request, before which the HFL Client NWDAF has to report the interim local ML Model information to the HFL Server NWDAF. Otherwise, the HFL Server NWDAF may indicate HFL Client NWDAF to skip reporting for this iteration. HFL Server NWDAF includes the current iteration round ID in the message to indicate that the request is to modify the training parameters of the current iteration round.

Alternatively, the HFL Server NWDAF may inform the HFL Client NWDAF to cease the ML Model training by sending termination request and to report back the current local ML Model updates.

5.	The HFL Server NWDAF aggregates all the local ML Model information retrieved at step 4, to update the global ML Model. The HFL Server NWDAF may also compute the global ML Model metric value, e.g. based on the local ML Model metric value(s) provided by the HFL Client NWDAF(s) or by applying the global model on the validation dataset (if available). The HFL Server NWDAF may update the global ML Model each time a HFL Client NWDAF provides updated local ML Model information, or the HFL Server NWDAF may decide to wait for local ML Model information from all HFL Client NWDAFs before updating the global ML Model.

If the HFL Server NWDAF provides the maximum response time for the HFL Client NWDAF(s) to provide the interim local ML Model information in step 2, or the new maximum response time in step 4a, the HFL Server NWDAF decides either to wait for the HFL Client NWDAF(s) which have not yet provided their interim local ML Model within the new maximum response time or to aggregate only the retrieved local ML Model information instances to update global ML Model. The HFL Server NWDAF makes this decision, considering the notification/response from the HFL Client NWDAF or, if the notification is not received, based on local configuration.

6a.	[Optional] Based on the consumer request in step 0, the HFL Server NWDAF sends a Nnwdaf_MLModelProvision_Notify message to update the global ML Model metric value to the consumer periodically (e.g. a certain number of training rounds or every 10 min) or dynamically when some pre-determined status is achieved (e.g. the ML Model Accuracy threshold is achieved or training time expires).

6b.	[Optional] The consumer decides whether the current model can fulfil the requirement, e.g. global ML Model metric value is satisfactory for the consumer and determines to stop or continue the training process. The consumer re-invokes Nnwdaf_MLModelProvision_Subscribe service operation as used in step 0 to continue the training process or invokes Nnwdaf_MLModelProvision_Unsubscribe service operation to stop the training process.

6c.	[Optional] Based on the subscription request sent from the consumer in step 6b, the HFL Server NWDAF updates or terminates the current HFL training process.

If the HFL Server NWDAF received a request in step 6b to stop the Horizontal Federated Training process, steps 7 and 8 are skipped.

7.	If the HFL procedure continues, HFL Server NWDAF may determine HFL Client NWDAF as described in clause 6.2C.2.3 and sends Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request that includes the aggregated ML Model information to selected HFL Client NWDAF(s) for next round of Horizontal Federated Training. The request may also include the ML Model Accuracy Check Flag, that indicates the HFL Client NWDAF(s) to use the local training data as the testing dataset to calculate the Model Accuracy of the global ML Model provided by the HFL Server NWDAF.

8.	Each HFL Client NWDAF updates its own ML Model based on the aggregated ML Model information distributed by the HFL Server NWDAF at step 7.

NOTE 3:	The steps 3-8 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

When the Federated Training procedure is complete, the HFL Server NWDAF requests the HFL client NWDAF(s) to terminate the HFL procedure by invoking Nnwdaf_MLModelTraining_Unsubscribe service with a cause code that the HFL process has finished and optionally with the final aggregated ML Model information. Then the HFL client NWDAF(s) terminate the local model training and if the final aggregated ML Model information is received from the HFL server NWDAF, the HFL client NWDAF(s) can store it for further use.

9.	After the training process is complete, the HFL Server NWDAF may send Nnwdaf_MLModelProvision_Notify that includes the globally optimal ML Model information to the consumer.

#### 6.2C.2.3	Procedures for Maintaining Horizontal Federated Learning Processes

This clause specifies how to maintain a Horizontal Federation Learning process in HFL execution phase, including HFL Server NWDAF triggers reselection, addition, or removal of HFL Client NWDAF(s), discovers new HFL Client NWDAF(s) via NRF and HFL Client NWDAF(s) joins or leaves Horizontal Federated Learning process dynamically.

In Horizontal Federated Learning execution phase, HFL Server NWDAF monitors the status changes of HFL Client NWDAF(s) and may reselects HFL Client NWDAF(s) based on the updated status, availability and/or capability, etc.

NOTE 1:	HFL Server NWDAF checks if there is a need to carry on the HFL execution phase and then reselects HFL members for the next iteration if needed.

Figure 6.2C.2.3-1: Procedure of HFL Server NWDAF reselects HFL Client NWDAF(s), HFL Client NWDAF(s) Join or Leave Horizontal Federated Learning Process Dynamically in Horizontal Federated Learning execution phase

The procedure for HFL Server NWDAF reselecting HFL Client NWDAF(s), HFL Client NWDAF(s) joining or leaving Horizontal Federated Learning process dynamically is as follows:

1a.	HFL Server NWDAF may get the updated status of current HFL Client NWDAF(s) via NRF by using Nnrf_NFManagement service (as in clause 5.2.7.2 of TS 23.502 [3]) in the Horizontal Federated Learning execution phase.

HFL Server NWDAF may subscribe to NRF for notifications of status changes of the current NWDAF(s) (HFL Client NWDAFs 1…N) by invoking an Nnrf_NFManagement_NFStatusSubscribe service operation. NRF notifies the HFL Server NWDAF the status changes of the current HFL Client NWDAF(s) by invoking Nnrf_NFManagement_NFStatusNotify service operation(s).

The status of a current HFL Client NWDAF could be availability changes, capability changes (e.g. it will not support HFL anymore, etc.).

1b.	The current HFL Client NWDAF(s) may inform HFL Server NWDAF that it is leaving the Horizontal Federated Learning process by invoking Nnwdaf_MLModelTraining_Notify service operation with Termination Request and cause code (reason for leaving, e.g. high NF load, time availability changes).

1c.	HFL Server NWDAF may get the information of the new HFL Client NWDAF(s) dynamically via NRF by subscribing to the event that a new HFL Client NWDAF registers (Nnrf_NFManagement_NFStatusSubscribe service as in clause 5.2.7.2 of TS 23.502 [3]).

1d.	NWDAF may subscribe for NF load analytics of the HFL Client NWDAF(s).

1e.	HFL Client NWDAF(s) may send Status report of HFL training and Global ML Model Accuracy Information by invoking Nnwdaf_MLModelTraining_Notify service.

2.	HFL Server NWDAF checks HFL Client NWDAF(s) status based on the received information and may determine whether reselection of HFL Client NWDAF(s) for the next round(s) of Horizontal Federated Learning is needed based on the received information from step 1.

NOTE 2:	Several examples of the factors that the HFL Server NWDAF can consider to reselect the HFL Client NWDAF(s) are updated status of HFL Client NWDAF reported by NRF is different than the criteria were initially used for selecting the client; characteristics of local training dataset is different than global validation dataset owned by HFL Server NWDAF and/or the metric value of the global model calculated using the local training dataset is much different from that calculated by other HFL Client NWDAFs; the metric value of the global model calculated using the local training dataset is lower than the metric value calculated using the global validation dataset owned by HFL Server NWDAF; the metric value of the global model calculated using the local training dataset is lower than ML Model metric value received in Nnwdaf_MLModelMonitor_Notify when HFL Server NWDAF using AnLF-assisted MTLF ML Models Accuracy Monitoring; the load of the HFL Client NWDAF (from the NF load Analytics or from the HFL Client NWDAF directly) is high; the HFL Server NWDAF receives leave request from the HFL Client NWDAF; the HFL Client NWDAF did not report the status of HFL Training within the maximum response time.

3.	[If re-selection is needed as judged in step 2] If step 1c is not performed, HFL Server NWDAF may discover new candidate HFL Client NWDAF(s) via NRF by using Nnrf_NFDiscovery services as in clause 5.2.7.3 of TS 23.502 [3]. HFL Server NWDAF reselects HFL Client NWDAF(s) from the current HFL Client NWDAF(s) and the new candidate HFL Client NWDAF(s) (found in steps 1c or 3). For the new candidate HFL Client NWDAF(s), the interaction between HFL Server NWDAF and HFL Client NWDAF(s) is same as the selection procedure described in clause 6.2C.2.1. The adding / deleting HFL Client NWDAF(s) may happen at the end of each iteration.

4.	HFL Server NWDAF sends termination request by invoking Nnwdaf_MLModelTraining_Unsubscribe service operation or Nnwdaf_MLModelTrainingInfo_Request service operation with Termination Request to the HFL Client NWDAF(s), optionally indicating the reason, e.g. HFL Client NWDAF is unselected by the HFL Server NWDAF for the HFL process, or the HFL process is suspended, etc. And HFL server may also send the updated global ML Model information to the unselected HFL client NWDAF. HFL Client NWDAF(s) terminates operations for the Horizontal Federated Learning process if receive termination request from the HFL Server NWDAF and may perform further action to be qualified in participation of HFL training in the next cycles.

NOTE 3:	In the case of high load, the HFL Client NWDAF can e.g. decline new service request. In the case of low accuracy, the HFL Client NWDAF can gather new local training data.

3rd change

## 6.2F	Procedure for ML Model Training

### 6.2F.1	ML Model Training Subscribe/Unsubscribe

The procedure in Figure 6.2F.1-1 is used by an NWDAF service consumer, i.e. an NWDAF containing MTLF to subscribe to another NWDAF, i.e. an NWDAF containing MTLF, for a trained ML Model based on the ML Model file or ML Model information as described in clause 6.2F.2 provided by the NWDAF service consumer. The service may be used by an NWDAF containing MTLF to enable e.g. Horizontal Federated Learning or to update ML Model. The service is also used by an NWDAF service consumer to request an NWDAF containing MTLF to prepare training ML Model or modify existing ML Model training subscription.

Figure 6.2F.1-1: Procedure for ML Model Training subscribe/unsubscribe

1.	The NWDAF service consumer may subscribe or unsubscribe for training an ML Model by invoking the Nnwdaf_MLModelTraining_Subscribe/ Nnwdaf_MLModelTraining_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2F.2.

In order to enable Horizontal Federated Learning, NWDAF Service consumer act as HFL Server NWDAF can subscribe to multiple NWDAFs containing MTLF act as HFL Client NWDAFs, which are selected by the HFL Server NWDAF.

The HFL server NWDAF may use the request to check if an NWDAF can meet the ML Model training requirement (e.g. ML Model Interoperability information, Analytics ID, Serving Area and/or availability of data and time). In such case, the HFL server NWDAF includes an ML Preparation Flag. When the ML Preparation Flag presents in the request, the service provider NWDAF only checks if it can meet the ML Model training requirement (e.g. ML Model Interoperability information, Analytics ID, Serving Area and/or availability of data and time) and / or can successfully download the model if the model information is provided.

The HFL server NWDAF may use the request to get the Model Accuracy information of the global ML Model calculated by the HFL Client NWDAFs. In such cases, the service consumer NWDAF includes a Model Accuracy Check Flag. When the Model Accuracy Check Flag is present in the request, the service provider NWDAF uses the local training data as the testing dataset to calculate the Model Accuracy information of the ML Model provided by the service consumer NWDAF.

When NWDAF service consumer determine to further update the ML Model, NWDAF service consumer modifies the subscription by invoking Nnwdaf_MLModelTraining_Subscribe service operation including Subscription Correlation ID with ML Model Information (as defined in clause 6.2A.2).

2.	The NWDAF containing MTLF trains ML Model provided at step 2 by collecting new data or re-use the data that it owns. If the ML Model file is not provided in step 1, the NWDAF containing MTLF shall first get the ML Model using the information indicated at step 1.

3.	When the NWDAF containing MTLF completes ML Model training, the NWDAF containing MTLF notifies the NWDAF service consumer with ML Model Information (as defined in clause 6.2A.2) of updated ML Model by invoking the Nnwdaf_MLModelTraining_Notify service operation. The parameters that can be provided by the NWDAF containing MTLF as service provider is specified in clause 6.2F.2.

If the NWDAF containing MTLF determines to terminate the ML Model training, i.e. NWDAF containing MTLF will not provide further notifications related to this request, then the NWDAF containing MTLF may notify the NWDAF Service consumer a Terminate Request indication with cause code (e.g. NWDAF overload, not available for the HFL process anymore, etc.) by invoking the Nnwdaf_MLModelTraining_Notify service operation.

In order to enable Federated Learning, NWDAF containing MTLF acting as HFL Client NWDAF can notify NWDAF Service consumer acting as HFL Server NWDAF the local ML Model information and status report of HFL training including accuracy information of local model and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.).

If the Model Accuracy Check Flag is present in the Nnwdaf_MLModelTraining_Subscribe, the service provider NWDAF acting as HFL Client NWDAF may notify the NWDAF Service consumer acting as HFL Server NWDAF the Model Accuracy information of the global ML Model.

### 6.2F.2	Contents of ML Model Training

The consumers of the ML Model training services (i.e. an NWDAF containing MTLF) may provide the input parameters in Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service operations as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be trained.

-	ML Model Interoperability Information as defined in clause 6.2A.2.

-	(Only for Nnwdaf_MLModelTraining_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the NWDAF containing MTLF with the subscription.

-	[OPTIONAL] ML Model Information (as defined in clause 6.2A.2).

-	[OPTIONAL] ML Model file.

NOTE 1:	It is up to NWDAF implementation to determine whether to include ML Model file in input parameters considering ML Model file size, etc.

-	[OPTIONAL] ML Model identifier: identifies the provided ML Model.

-	[OPTIONAL] ML Preparation Flag: identifies whether the request is for preparing Federated Learning or executing Federated Learning.

-	[OPTIONAL] ML Model Accuracy Check Flag: identifies that the request is for using the local training data as the testing dataset to calculate the Model Accuracy of the global ML Model provided by the NWDAF service consumer acting as the HFL Server NWDAF.

-	[OPTIONAL] ML Correlation ID: identifies the Horizontal Federated Learning procedure for training the ML Model. This parameter is included when the service is used for Federated Learning.

-	[OPTIONAL] Data Availability requirement. This is the requirement on data availability for the ML Model training. e.g. HFL Server NWDAF sends the requirement in preparation request to a HFL Client NWDAF for selecting the HFL Client NWDAF which can meet the data availability requirement. The following may be included:

-	Event ID list to be collected for local model training.

-	Dataset statistical properties as defined in clause 6.1.3.

-	Time window of the data samples.

-	Minimum number of data samples.

-	[OPTIONAL] HFL Availability time requirement. This is the requirement on availability time for the ML Model training, e.g. HFL Server NWDAF sends the requirement in preparation request to HFL Client NWDAF for selecting the HFL Client NWDAF which is available in the required time for training ML Model.

-	[OPTIONAL] Training Filter Information: enables to select which data for the ML Model training is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the Training Filter Information are the same as or subset of parameter types in the ML Model Filter Information which are defined in clause 6.2A.2.

-	[OPTIONAL] Target of Training Reporting: indicates the object(s) for which data for ML Model training is requested, i.e. group of UEs identified by a list of Internal-Group-Ids or any UE (i.e. all UEs).

-	[OPTIONAL] Use case context: indicates the context of use of ML Model.

-	[OPTIONAL] Training Reporting Information with the following parameters:

-	Maximum response time: indicates maximum time for waiting notifications (i.e. model training results).

-	[OPTIONAL] Iteration round ID: indicates the iteration round number of current ML Model training.

-	[OPTIONAL] Expiry time.

-	[OPTIONAL] Indication of skipping the current HFL round.

The NWDAF containing MTLF provides to the consumer of the ML Model training service operations as described in clause 7.10 and clause 7.11, the output information in notification or response as listed below:

-	(Only for Nnwdaf_MLModelTraining_Notify) The Notification Correlation Information.

-	[OPTIONAL] ML Model Information (as defined in clause 6.2A.2).

-	[OPTIONAL] ML Model identifier: identifies the provisioned ML Model.

-	[OPTIONAL] Global ML Model Accuracy information: The model metric value of the global ML Model and optionally the used metric, which is calculate by the HFL Client NWDAF using the local training data as the testing dataset.

[OPTIONAL] Status report of HFL training: Accuracy information of local model and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.), which are generated by the HFL Client NWDAF during HFL procedure.

NOTE 2:	The parameters in Training Input Data Information are up to the implementation.

-	[OPTIONAL] ML Correlation ID. This parameter may be included when the service is used for Horizontal Federated Learning.

-	[OPTIONAL] Iteration round ID: indicates the iteration round number of ML Model training indicated by the HFL Server NWDAF.

-	[OPTIONAL] Delay Event Notification with the following parameters:

-	delay event indication: this parameter indicates that HFL Client NWDAF is not able to complete the training of the interim local ML Model within the maximum response time provided by the HFL Server NWDAF.

-	[OPTIONAL] cause code (e.g. local ML Model training failure, more time necessary for local ML Model training, etc.).

-	[OPTIONAL] Expected time to complete the training: Indicates to the HFL Server NWDAF that expected remaining training time and may be provided with Delay Event Notification.

### 6.2F.3	ML Model Training Information Request

The procedure in Figure 6.2F.3-1 is used by an NWDAF service consumer, i.e., an NWDAF containing MTLF to request another NWDAF, i.e., an NWDAF containing MTLF, for the information about ML Model training based on the ML Model file or ML Model information as described in clause 6.2F.2 provided by the NWDAF service consumer. The service may be used by an NWDAF containing MTLF to enable e.g. Federated Learning.

Figure 6.2F.3-1: Procedure for ML Model Training Information Request

1.	The NWDAF service consumer may request the NWDAF containing MTLF to get the information about the ML Model training based on the ML Model file or ML Model information as described in clause 6.2F.2 provided by the service consumer by invoking the Nnwdaf_MLModelTrainingInfo_Request service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2F.2.

In order to enable Horizontal Federated Learning, NWDAF Service consumer acting as HFL Server NWDAF requests to get ML Model Training Information from multiple NWDAF containing MTLF acting as HFL Client NWDAFs, which are selected by the HFL Server NWDAF. The details are specified in clause 6.2C.

The NWDAF service consumer may use the request to check if an NWDAF can meet the ML Model training requirements (e.g. ML Model Interoperability information, Analytics ID, Service Area/DNAI and/or availability of data and time). In such cases, the NWDAF service consumer includes an ML Preparation Flag.

The NWDAF service consumer may use the request to get the Model Accuracy of the ML Model provided by the service consumer using local training data in the NWDAF containing MTLF as the testing dataset. In such cases, the service consumer NWDAF includes a Model Accuracy Check Flag.

2.	When the ML Preparation Flag is present in the request, the NWDAF containing MTLF only checks whether it can meet the ML Model training requirement and/or can successfully download the model if the model information is provided. Based on the check result, the NWDAF containing MTLF gets a successful return code or failure cause code (e.g. NWDAF does not meet the ML training requirements) as the information about the ML Model training.

When the Model Accuracy Check Flag is present in the request, the NWDAF containing MTLF uses the local training data as the testing dataset to calculate the Model Accuracy information of the ML Model provided by the service consumer. The NWDAF containing MTLF includes the Model Accuracy information into the information about the ML Model training.

When the NWDAF containing MTLF is ongoing ML Model training based on the ML Model file or ML Model information as described in clause 6.2F.2 provided by the NWDAF service consumer, the NWDAF containing MTLF gets a failure cause code (e.g. ML training is not complete) as the information about the ML Model training.

When the NWDAF containing MTLF completes ML Model training based on the ML Model file or ML Model information as described in clause 6.2F.2 provided by the NWDAF service consumer, the NWDAF containing MTLF gets a successful return code and the ML Model Information of the trained ML Model as the information about the ML Model training.

3.	The NWDAF containing MTLF replies to the NWDAF service consumer with the information about the ML Model training by invoking the Nnwdaf_MLModelTrainingInfo_Request response service operation.

4th change

## 7.10	Nnwdaf_MLModelTraining Service

### 7.10.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe/notify/modify for ML Model training.

NOTE:	In this release of the specification, the service provider and consumer are limited to NWDAF containing MTLF.

When used for Horizontal Federated Learning, this service enables HFL server NWDAF to enable Horizontal Federated Learning(HFL) while providing global ML Model information to HFL Client NWDAF and getting local ML Model information and status report of HFL training as defined in clause 6.2C.2.3 from the HFL Client NWDAF.

This service may also be used by the consumer (i.e. HFL Server NWDAF) to check if the service provider (i.e. FL Client NWDAF) can meet the ML Model training requirement as described in clause 6.2F.1.

This service may also be used by the consumer (i.e. HFL Server NWDAF) to request the service provider (i.e. HFL Client NWDAF) to calculate and provide Model Accuracy of the global ML Model as described in clause 6.2F.1.

### 7.10.2	Nnwdaf_MLModelTraining_Subscribe service operation

Service operation name: Nnwdaf_MLModelTraining_Subscribe

Description: Subscribes to NWDAF ML Model training with specific parameters.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	ML Model Interoperability information;

-	Notification Target Address (+ Notification Correlation ID).

Inputs, Optional:

-	ML Model identifier: identifies the provided ML Model.

-	ML Model Information (as defined in clause 6.2A.2);

-	ML Model file;

-	Subscription Correlation ID (in the case of modification of the ML Model Training subscription);

-	ML Training Information, i.e. data availability requirement, time availability requirement.

-	ML Preparation Flag;

-	ML Model Accuracy Check Flag;

-	ML Correlation ID;

-	Training Filter Information;

-	Target of Training Reporting;

-	Training Reporting Information as defined in clause 6.2F.2;

-	Use case context;

-	Iteration round ID;

-	Expiry time.

-	Indication of skipping the current HFL round.

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the ML training requirements, ML training is not complete, NWDAF overload, not available for the FL process anymore, etc.).

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: ML Correlation ID (e.g. confirm of the subscription for this HFL process).

### 7.10.3	Nnwdaf_MLModelTraining_Unsubscribe service operation

Service operation name: Nnwdaf_MLModelTraining_Unsubscribe

Description: Terminate NWDAF ML Model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code (e.g. HFL Client NWDAF is unselected by the HFL Server NWDAF for the HFL process, or the HFL process is suspended or finished, etc.). Final aggregated ML Model information (if HFL has finished) or updated aggregated ML Model information (if HFL is suspended).

### 7.10.4	Nnwdaf_MLModelTraining_Notify service operation

Service operation name: Nnwdaf_MLModelTraining_Notify

Description: NWDAF notifies the consumer instance of the trained ML Model that has subscribed to the specific NWDAF service. The NWDAF can also use this service to indicate to consumer it will terminate the ML Model training.

Inputs, Required:

-	Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during ML Model training.

Inputs, Optional:

-	Set of the tuple (Analytics ID, ML Model Information as defined in clause 6.2F.2;

-	ML Correlation ID, when for Federated Learning;

-	Corresponding Use case context;

-	Termination Request: this parameter indicates that NWDAF requests to terminate the ML Model training, i.e. NWDAF will not provide further notifications related to this request, with cause code (e.g. NWDAF overload, not available for the FL process anymore, etc.);

-	ML Model identifier: this parameter identifies the provisioned ML Model;

-	Global ML Model Accuracy information: The model accuracy of the global ML Model, which is calculate by the HFL Client NWDAF using the local training data as the testing dataset;

-	Status report of HFL training: local ML Model accuracy information and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.), which are generated by the HFL Client NWDAF during HFL procedure;

-	Delay Event Notification: as defined in clause 6.2F.2;

-	Iteration round ID.

NOTE:	The detail reasons in the cause code are up to stage 3.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

5th change

## 7.11	Nnwdaf_MLModelTrainingInfo Service

### 7.11.1	General

Service Description: This service enables the consumer to request for the information about ML Model training based on the ML Model file or ML Model information as described in clause 6.2F.2 provided by the consumer.

NOTE:	In this release of the specification, the service provider and consumer are limited to NWDAF containing MTLF.

When used for Horizontal Federated Learning, this service enables HFL server NWDAF to enable Horizontal Federated Learning while providing global ML Model information to HFL Client NWDAF and getting local ML Model information from the HFL Client NWDAF.

### 7.11.2	Nnwdaf_MLModelTrainingInfo_Request service operation

Service operation name: Nnwdaf_MLModelTrainingInfo_Request

Description: Request information about NWDAF ML Model training with specific parameters.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2.

-	ML Model Interoperability information.

Inputs, Optional:

-	ML Model identifier: identifies the provided ML Model.

-	ML Model Information (as defined in clause 6.2A.2).

-	ML Model file.

-	ML Training Information (i.e. data availability requirement, time availability requirement).

-	Training Reporting Information as defined in clause 6.2F.2.

-	ML Preparation Flag.

-	ML Model Accuracy Check Flag.

-	ML Correlation ID.

-	Termination Request, when terminating the Federated Learning identified by the ML Correlation ID and optionally indicating the reason, e.g. FL Client NWDAF is unselected by the FL Server NWDAF for the FL process, or the FL process is suspended, etc.

-	Training Filter Information.

-	Target of Training Reporting.

-	Use case context.

-	Indication of skipping the current FL round.

Outputs Required: When the request is accepted: Operation execution result indication. When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the ML training requirements, ML training is not complete, NWDAF overload, not available for the FL process anymore, etc.).

NOTE:	The detail reasons in the cause code are up to stage 3.

Outputs, Optional:

-	ML Model identifier.

-	Set of the tuple (Analytics ID, ML Model Information (as defined in clause 6.2A.2)).

-	ML Correlation ID, when for Federated Learning.

-	Corresponding Use case context.

-	Global ML Model Accuracy information: The model accuracy of the global ML Model, which is calculate by the HFL Client NWDAF using the local training data as the testing dataset.

-	Status report of HFL training: local ML Model Accuracy Information and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension of data, etc.), which are generated by the HFL Client NWDAF during HFL procedure.

-	Delay Event Notification as defined in clause 6.2F.2.

End of changes