3GPP TSG-SA2 Meeting #166	S2-2411335r6

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1104 | rev | 9 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Some corrections of the AIML_CN KI#4 Signalling Storm |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SK Telecom, Verizon, Samsung, AT&T, China Mobile, ETRI, KPN N.V., LG Uplus, Rakuten Mobile, ZTE, Huawei, HiSilicon, vivo, NTT DOCOMO, OPPO, Nokia, [China Telecom, Ericsson, Lenovo, KDDI] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the approved revision 8 of the S2-2410976 (SA2#165), in this revision 9, some duplicatations/additions, clean ups including typos are corrected. - In the input, the NWDAF can also collect NRF related information from the NRF directly because the information is reported by NRF to OAM. Therefore, when information is available the NRF can serve as the data source of related information from NRF. - In the output, the received signalling analytics is related to the time slot. For example, the existing Dispersion analytics also included the time slot. Therefore, time slot entry information is added in the output. - Signaling information is removed because it contains only the received signling analysis. It is sufficient to include only the received signling analysis in the information. - SCP is not mentioned explictly which may lead misunderstanding. - Some input parameters agreed in the TR are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed OAM and MDAF/MDAS from analytics consumer. - Deleted some dulplicate information, clean ups including typos. - Deleted Target indicent ID as it is represented in the Cause in the output analytics. - Updated Target Analytics report to include SUPI/GPSI range. - Added NRF as the data source of NRF related information. - Added time slot entry in the output and deleted bullet of the optional signaling information. - Mentioned the SCP in the general description and the procedure description. - Deleted EN of Additional Analytic Filter information. - Missing parameters are added. - Updated the 6.x.4.-1 Figure to be Visio-editable. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It can mislead and result in inconsistent specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.x.1 (new), 6.x.2 (new), 6.x.3 (new), 6.x.4 (new), 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revision from the S2-2410976 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new)* * * *

6.x	Signalling Storm Analytics

6.x.1	General

This clause specifies how NWDAF supports analytics for network abnormal behaviours (i.e. signalling storm) mitigation and prevention.

The signalling storm analytics provides analytics information (statistics or predictions) regarding the abnormal signalling. Abnormal signalling is a significant deviations from normal conditions indicating a potential signalling storm. The analytics of signalling storm requires the identification of whether the signalling storm is due to signalling from a NF or massive signalling from UE based on the requested analytics ID.

The NWDAF provides statistics and/or prediction analytics to consumer NFs, e.g. AMF, SMF, UDM, PCF, NRF, SCP and AF.

The consumer of these analytics may indicate in the request or subscription:

-	Analytics ID = "Signalling Storm";

-	Target of Analytics Reporting: a list of NFs, or one UE, or group of the UE(s) identified by internal/external Group ID or SUPI/GPSI range;

-

Analytics Filter Information:

-	NF ID List: instance IDs or NF Set IDs of target NF that signalling storm analytics should be provided for.

-	Area of Interest (AoI) and/or S-NSSAI which restricts the scope of signalling storm analytics to the specific area and/or slice;

-	Analytics Reporting Information:

-	Expected Report Time: indicates the time limit by which the analytics results need to be received., e.g. 10 seconds, 5 minutes, etc;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-

Signalling frequency threshold: indicates the frequency of the received signalling within the time period;

-	UE number threshold: indicates a threshold of the amount of UEs of which the number of one type of request messages meets the frequency of UE requests threshold. For example, if the number of UEs that initiate registration requests exceeds the pre-set rate threshold (10 times per hour) and the number of such UEs exceeding the pre-set rate threshold exceeds 100 (the UE number threshold is set to 100), the NWDAF will provide the analytics.

6.x.2	Input data

The NWDAF collects the signalling feature data, NF context, AF data and MDAF data as listed in Table 6.x.2-1, 6.x.2-2, 6.x.2-3, 6.x.2-4, 6.x.2-5 and 6.x.2-6.

The NWDAF also collects UE behavioural information per group of UEs as specified in clause 6.7.5.2.

The NWDAF may use output of abnormal UE behaviour analytics and UE dispersion analytics as specified in clause 6.7.5 and 6.10.3 as input data for analytics on signalling storm. This information may be collected from a different NWDAF (e.g. distributed architecture).

The NWDAF may collect the subgroup of input data based on implementation.

Table 6.x.2-1: UE related Context Data collection

| Information | Source | Description |
| --- | --- | --- |
| UE ID |  | Identifies a single UE. |
| Signalling feature data |  | NF procedures containing signalling exchange information related to a particular UE or session from the Connection, Registration, Mobility and Session Managements procedures. (NOTE 1) |
| > Request type and number from UE/RAN (0..max) | AMF | Request type of N1 or N2 interface (such as received Initial Registration Request, Mobility and Periodic Registration Request, Service Request, etc) and the number of requests corresponding to the request type. |
| >> Time duration from receiving request from UE/RAN to response to UE/RAN | AMF | Time duration between the request from UE/RAN and response to UE/RAN. |
| >> Number of successful responses of UE/RAN | AMF | Number of successful responses associated to their initial requests, such as Registration Response, etc. |
| >> Number of failed responses of UE/RAN | AMF | Number of failed responses associated to their initial requests, such as Registration Reject, Service Reject, etc. |
| >> Reason of failed responses of UE/RAN | AMF | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| >> A posterior Type of requests of UE/RAN (0..max) | AMF | A posterior Request types triggered from UE/RAN, for NF Service request, or request to UE/RAN. |
| > Request type and number from NF (0..max) | NF, SCP | Request type received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc., as well as the number of requests received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc. |
| >> Time duration from receiving request from NF to response to NF | NF, SCP | Time duration between the request from NF and response to NF. |
| >> Number of successful responses of NF | NF, SCP | Number of successful responses associated to their initial requests. |
| >> Number of failed responses of NF | NF, SCP | Number of failed responses associated to their initial requests. |
| >> Reason of failed responses of NF | NF, SCP | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| >> Number of redundant signalling of NF | NF, SCP | Number of received redundant signalling. The redundant signalling means the signalling which is transmitted in multiple times. |
| >> A posterior Type of requests of NF (0..max) | NF, SCP | A posterior Request types triggered from NF, for NF Service request. |
| > Public Warning information | AMF | Public Warning information as defined in the TS 23.041[X], such as WRITE-REPLACE WARNING REQUEST, etc. |
| > Frequent Mobility Registration Update | AMF | The number of Mobility Registration Updates N within a period M may be an indication for abnormal ping-pong behaviour, where N and M are operator’s configurable parameters. |
| > Number of receiving Session Report from UPFs | SMF | Number of receive Session Report from UPF triggered by DL packet in case of PDU Session is in 5GCM-idle state. |
| UE Context in NF |  |  |
| > State transition information | AMF, SMF | UE related state transition information such as transition type, frequency of CM state changes etc. State transition identifier: - “Access Type change to 3GPP access”; - “Access Type change to non-3GPP access”; - “RM state change to RM-DEREGISTERED”; - “RM state change to RM-REGISTERED “; - “CM state change to CM-IDLE”; - “CM state change to CM-CONNECTED”; - “Handover”; or - “Mobility Registration Update”. - “Frequent Mobility Registration Update”(table 6.7.2.2-1) Or, PDU Session related state transition information such as transition type, frequency SM state changes, etc. State transition identifier: - “PDU Session Establishment”; - “PDU Session Release”; - “Communication failure”; or - “PLMN change”. It can be reported for a group of UEs(e.g., the number of total transitions or percentage of the group UEs who have transitions). |
| > timer information | AMF, SMF | Timer information which has been set for the UE, such as timer type, duration. |
| NOTE 1: NWDAF can optionally provide transaction dispersion analytic information for MM and SM transactions. |  |  |

Table 6.x.2-2: NF Context Data collection

| Information | Source | Description |
| --- | --- | --- |
| NF ID |  | NF instance ID |
| NF profile | NRF | NF Profile information such as allowed NF information for the NF and NF Service(s). |
| > NF load status information | NRF | Load information indicates the current load of the NF and NF Service(s), e.g. CPU, memory, and/or percentage of load information. (NOTE 1) |
| > Capacity and priority information of NFs and NF Services | NRF, SCP | Capacity and priority information of the registered NF and NF Service(s). (NOTE 1) |
| NF heart-beat related information | NF | NF heart-beat related information such as responding time, Number of retransmissions, heart-beat intervals, |
| > Unexpected operational event | NF, OAM | Unexpected operational status based on operator defined threshold, e.g. operating temperatures, energy consumptions, axiality processing unit (xPU) usages, thread/process creation rate, I/O latency, transaction rate, etc. beyond a preconfigured threshold(s) detected by the NF. (NOTE2) |
| NOTE 1: If this information is not available in the NRF, it can be obtained from the OAM. NOTE 2: Generation of this event depends on the implementation and deployment of the NF. |  |  |

Table 6.x.2-3: NF Specific Data collection

| Information | Source | Description |
| --- | --- | --- |
| NF ID |  | NF instance ID |
| > Usage information of UE IP address resources | SMF | Usage information of UE IP address resources (dynamic and static, V4, V6, etc.) for CP or UP allocation, such as number, usage, number of UE IPs, which prohibit allocation during certain time interval, etc. This parameter is valid for SMF only. |
| > Load information of connected UPFs | SMF | Load information of connected UPFs such as using PFCP Load Control Information. This parameter is valid for SMF only. |
| SCP Signalling statistics | SCP | The number of different types of signalling received and sent by SCP during a target time period, etc. This parameter is valid for SCP only. |

Table 6.x.2-4: Application activation time information

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | Identifies the application providing this information. |
| User activation time information (1…max) | AF | Information of activation time for the users (e.g. IoT users) per application. |
| > Number of UEs | AF | The total number of UEs |
| > Active Time | AF | The time stamp of the users per application switch to active, or the start and end time of the users activity per application. |
| > Inactive Time | AF | The time stamp of the users per application switch to inactive, or the start and end time of the users inactivity per application, if applicable. |
| > UE type ID | AF | Identifies a group of UEs, e.g. external group ID, or a list of UE IDs. |

Table 6.x.2-5: Data collection from NRF/OAM

| Information | Source | Description |
| --- | --- | --- |
| NRF ID |  | NRF instance ID. |
| > Received time | NRF, OAM | Time stamp that the related request is received at the NRF. |
| Number of NF service registration requests | NRF, OAM | Number of registration request received at the NRF |
| > Number of successful NF service registrations | NRF, OAM | Number of successful registrations |
| > Number of failed NF service registrations due to encoding error of NF profile | NRF, OAM | Number of failed registrations |
| > Number of failed NF service registrations due to NRF internal error | NRF, OAM | Number of failed registrations |
| Number of NF service update requests | NRF, OAM | Number of update request received at the NRF |
| > Number of successful NF service updates | NRF, OAM | Number of successful updates |
| > Number of failed NF service updates due to encoding error of NF profile | NRF, OAM | Number of failed updates |
| > Number of failed NF service updates due to NRF internal error | NRF, OAM | Number of failed updates |
| Number of NF discovery requests | NRF, OAM | Number of discovery request received at the NRF |
| > Number of successful NF discoveries | NRF, OAM | Number of successful discovery attempts |
| > Number of failed NF service discoveries due to unauthorized NF Service consumer | NRF, OAM | Number of failed discovery attempts |
| > Number of failed NF discoveries due to input errors | NRF, OAM | Number of failed discovery attempts |
| > Number of failed NF discoveries due to NRF internal error | NRF, OAM | Number of failed discovery attempts |

Table 6.x.2-6: Data collection from MDAF/MDAS of control plane congestion analysis

| Information | Source | Description |
| --- | --- | --- |
| affectedObject | MDAF | Indication of 5GC NFs where congestion issues occurred or potentially may occur. |
| cPCongestionIssueID | MDAF | This field holds the ID of the control plane congestion issue which is reported. |

6.x.3	Output analytics

The output analytics of signalling storm provided by NWDAF is defined in Table 6.x.3-1, and Table 6.x.3-2. The Table 6.x.3-3 gives examples of NF actions for solving each risk. The final mitigation or prevention operations are based on operator's policy/configuration and NF implementation.

The statistics and predictions are provided with a Validity Period, as defined in clause 6.1.3.

Table 6.x.3-1: Signalling storm statistics

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm statistics. |
| > Target NF ID | A list of impacted NFs of signalling storm detected by NWDAF. |
| > Cause of the signalling storm | The potential cause of NF Abnormality (i.e. massive signalling from UE or NF abnormal signalling). |
| > Source UE/NF | group of the UE(s) identified as served by slice ID or internal Group ID or NF(s) identified as NF list which is causing the signalling storm. |
| > Time slot entry (1..max) | List of time slots during the Analytics target period. |
| >> Time slot start | Time slot start within the Analytics target period. |
| >> Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| >> Received Signalling Analytics | Information of signalling received by the target NF(s). |
| >>> Total number of received signalling | Indicates the statistics on the number of signalling messages received by the target NF(s) in the time slot. |
| >>> Growth rate of received signalling | Difference between the number of signalling messages received by the NF in the time slot and the number of signalling messages received by the target NF(s) in the previous time slot. |
| >>> Signalling analytics of UE | Indicates the statistics on the number of signalling messages received from UEs within the time slot. NOTE1 |
| > OPTIONAL Timer List | The list of timer information per source UE(s) (NOTE 1) |
| >> Type of timer | The type of timer which has been set. |
| >> Timer duration | The timer duration that has be selected for the source UE(s). |
| NOTE 1: Only available when Cause of signalling storm is massive signalling from UEs, and there exists Source UE(s). |  |

Table 6.x.3-2: Signalling storm predictions

| Information | Description |
| --- | --- |
| Report (1..max) | List of predicted signalling storm analytics. |
| > Target NF ID | A list of impacted NFs signalling storm predicted by NWDAF. |
| > Cause of the signalling storm | The potential cause of NF Abnormality (i.e. massive signalling from UE or NF abnormal signalling). |
| > Source UE/NF | group of the UE(s) identified as served by slice ID or internal Group ID, or NF(s) identified as NF list which is causing the signalling storm. |
| > Time slot entry (1..max) | List of time slots during the Analytics target period. |
| >> Time slot start | Time slot start within the Analytics target period. |
| >> Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| >> Reference point | The information of the reference point impacted. (e.g. N1, N2) |
| >> Service operation(s) | The information of the service operation(s) impacted. (e.g. Namf_Communication(UEContextTransfer), Nsmf_PDUSession(UpdateSMContext)) |
| >> Received Signalling Analytics | Information of signalling received by the target NF(s). |
| >>> Received number of signalling | Received number of signalling of the specific signalling type. |
| >>> Growth rate of received signalling | Difference between the number of signalling messages received by the NF in the time slot and the number of signalling messages received by the target NF(s) in the previous time slot. |
| >>> Signalling analytics of UE | Indicates the statistics on the number of signalling messages received from UEs within the time slot. (NOTE 1) |
| > OPTIONAL Timer List | The list of timer information per Source UE(s) (NOTE 1) |
| >> Type of timer | The type of timer. |
| >> Timer duration | The timer duration that commonly is selected for the Source UE(s). |
| > Priority | Priority (relative to other NFs of the same type) of candidate NFs as defined in the TS 29.510 [18]. |
| > Capacity | Candidate NF capacity information, expressed as a weight relative to other NF instances of the same type as defined in the TS 29.510 [18]. |
| > Confidence | Confidence of this prediction. |
| NOTE 1 Only available when Cause of signalling storm is massive signalling from UEs, and there exists Source UE(s). |  |

NOTE 1:	The statistics and predictions are provided with a Validity Period, as defined in clause 6.1.3.

Table 6.x.3-3: Example mechanisms to mitigate and prevent the signalling storm

| Cause of the signalling storm | Actions of NFs |
| --- | --- |
| massive signalling from UE | AMF sets MM NAS related timer (e.g. back-off, T3512) with suggested time range for a selected set of UEs. |
| massive signalling from UE | SMF sets SM NAS related timer (e.g. back-off) with suggested time range for a selected set of Sessions. |
| massive signalling from UE | AMF/SMF sets suggested N1/N2 interface related ingress/egress threshold, or AMF triggers RAN to initiate overload control for a selected set of UEs in specific slice or priority as defined in clause 8.7.7 of TS 38.413 [8] to start overload control. The AMF may assign a slice under signalling storm conditions to the UEs and initiate overload control for the specific slice. |
| NF abnormal signalling | NRF configures the local policy (e.g. to prevent the source NF with abnormal signalling from being discovered or discovering others, to update the NF profile(s)(e.g. Priority, Capacity) of NF(s) based on the signalling storm statistics and/or predictions and send the updated NF profiles(s) to NF service consumer(s) of the NF(s)(e.g. subscriber of the NF profile as defined in clause in 4.17.7 of TS 23.502)). |
| NF abnormal signalling | Source NF configures to (re)select other NFs instead of NF with abnormal signalling and may unsubscribes the NF with abnormal signalling. |
| NF abnormal signalling | Source NF configures to deprioritize the NFs/Services with abnormal signalling from being selected. |
| NF abnormal signalling | Source NF triggers UE Reregistration and/or Session Reestablishments to avoid NF with abnormal signalling. |
| NF abnormal signalling | SCP blocks or throttles messages originating from an NF/service with abnormal signalling. |
| NF abnormal signalling | SCP redirects requests towards an NF service producer with abnormal signalling towards another service producer |
| NF abnormal signalling | SCP notifies communication peers of an NF with abnormal signalling about abnormal condition, e.g. high load |

6.x.4	Procedures

The NWDAF can provide information on network signalling storm as follows. (Figure 6.x.4-1).

Figure 6.x.4-1: Procedure for NWDAF-assisted Network Signalling Storm Mitigation and Prevention

1.	The consumer NF or SCP subscribes to or sends a request to NWDAF for the signalling storm analytics using either Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation. The request additionally includes thresholds such as confidence level and accuracy of detection.

The Analytics ID is set to "Signalling Storm". The target for analytics reporting is set to be any NF/SCP or any UE. Analytic filters may be provided as shown in clause 6.x.1.

The consumer NF/SCP can request statistics or/and predictions for a given Analytics target period.

The consumer NF/SCP may also subscribe to “NF load analytics” as depicted in clause 6.5 and trigger the signalling storm analytics based on output of the NF load analytics, e.g. CPU usage is over 80%.

The consumer NF/SCP may also subscribe to “Abnormal behaviour” analytics as depicted in clause 6.7.5 and trigger the signalling storm analytics based on the suspicion of DDoS attack.

The consumer NF/SCP may also subscribe to “Dispersion Analytics” analytics as depicted in clause 6.10 and trigger the signalling storm analytics when the transaction dispersion is above the expected transaction dispersion configurable threshold.

2.	The NWDAF retrieves Input data from NFs/SCP, using Nnf_EventExposure_Subscribe, such as UE and/or NF releated context data, NF context, AF data, MDAF data and UE behavioural information to derive the required analytics as depicted in clause 6.x.2. The NWDAF may optionally request additional data aligned with the analysis target identified in step 1.

3.	The NWDAF derives the required analytics with the collected data described in clause 6.x.2.

4.	The NWDAF invokes Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response or response to the consumer NF for the Output data analytics as depicted in clause 6.x.3.

5.	The consumer upon receiving the detection and/or prediction and/or mitigation may execute based on the example mechanisms as depicted in clause 6.x.3.

# * * * * Second change * * * *

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, NRF, SCP, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, NRF, SCP, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, NRF, SCP, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, NRF, SCP, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-NWDAF, V-NWDAF |
|  | Unsubscribe |  | H-NWDAF, V-NWDAF |
|  | Notify |  | H-NWDAF, V-NWDAF |
|  | Request | Request / Response | H-NWDAF, V-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-NWDAF, V-NWDAF |
|  | Unsubscribe |  | H-NWDAF, V-NWDAF |
|  | Notify |  | H-NWDAF, V-NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Editor´s note:	It is FFS whether separate Roaming Analytics and Roaming data services for inbound and outbound roaming scenarios are required.

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| E2E data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behavior | Analytics ID: Movement Behavior | Statistics or predictions on movement behavior for an applicable area |
| Signalling Storm | Analytics ID: Signalling storm | Statistics or predictions on controlling signalling storm for network abnormal behaviour mitigation or prevention. |

# * * * * End of changes * * * *