SA WG2 Meeting #166	S2-2411401

# Orlando, USA, 18 – 22 November, 2024                        (revision of S2-24xxxxx)

Source: 	NEC

Title: 	AIoT conclusion for D2R message size on Key issue#3

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes the AIoT conclusion for D2R message size on Key issue#3.

1	Discussion

RAN2 sends a LS (S2-2411311) to SA2 to ask the following questions:

RAN2 kindly requests SA2 to answer the following questions:

Q1: Can the CN provide, to the reader, an estimate of the expected size of the following D2R message(s) in response to the service request?

Q2: If such information (see Q1) can be provided by CN to the reader, can it be provided only in some cases or in all cases?

During the AIoT service, the service layer assistance information is provided by AF to AIoTF. The service layer D2R message size can be included in the service layer assistance information if AF can provide this parameter. AIoTF maps the service layer D2R message size to the network layer D2R message size and provides the network layer D2R message size to the Reader.

AF provides the assistance information to AIoTF in the AIoT service request. When receiving the AIoT service request from AF, if the assistance information is not included, the AIoT Function requests the assistance information from AF.

Based on the discussion above, it is suggested to answer RAN2 as below:

SA2 answers the following questions:

RAN2 Q1: Can the CN provide, to the reader, an estimate of the expected size of the following D2R message(s) in response to the service request?

SA2 Answer 1: Yes, CN can provide to the reader an estimate of the expected size of the following D2R message(s) in response to the service request.

RAN2 Q2: If such information (see Q1) can be provided by CN to the reader, can it be provided only in some cases or in all cases?

SA2 Answer 2: Such information can be provided only in some cases where there are SLA(Service Layer Agreement) between the AF and the core network, and the AF has the information and capability to provide such information.

2	Proposal

It is proposed to update TR 23.700-13 as following.

# * * * * First change* * * *

## 8.3	Conclusion on Key Issue #3

### 8.3.1	General

The clause concludes the following aspects:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices;

NOTE:	The above aspect includes studying whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

Editor's note:	Additional conclusions are FFS.

AIoTF can provide to the reader an estimate of the expected size of the following D2R message(s) in response to the service request, which is based on the AF provided service layer D2R message size if it is available.

# * * * * End of Changes * * * *