3GPP TSG-WG SA2 Meeting #166	S2-2411404

# 18 - 22 November, 2024, Orlando, USA			                                 (revision of S2-2411072)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5628 | rev | 8 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Energy Efficiency and Energy Saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Rakuten Mobile, SK Telecom, China Unicom, NEC, Tencent, TOYOTA MOTOR CORPORATION, Samsung, ZTE, LG Electronics, ETRI, [Orange, Interdigital, NTT DOCOMO, KPN, KDDI,Deutsche Telekom, vivo, Huawei, HiSilicon, Telefonica, Nokia,] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This paper provides the basic framework and function design to support energy related information collection, energy consumption information calculation, and exposure. New NF EIF is introduced to support the above functionality. The methods of calculation for the Energy consumption information of different granularities are provided. The Energy Consumption information of the different granularities are derived from user plane traffic transmission, i.e. the combination of Energy Consumption information in NG-RAN, I-UPF(s) and PSA-UPF(s), and the data volume is the main contributor. The Energy Consumption Assisted information include: Node-level energy consumption information, Node-level data volume, and data volume of the required granularities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new NF EIF for architecture and function design to support energy consumption information collection and calculation. Adding further information or detailed description to clarify the EIF service and interaction. Removing the Ens |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The energy saving features cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 3.2; 5.X;6.2.5;6.2.10; Annex XX |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TS 22.261: “Service requirements for next generation new services and markets; Stage 1”.

[3]	3GPP TS 23.502: “Procedures for the 5G System; Stage 2”.

[4]	3GPP TS 23.203: “Policies and Charging control architecture; Stage 2”.

[5]	3GPP TS 23.040: “Technical realization of the Short Message Service (SMS); Stage 2”.

[6]	3GPP TS 24.011: “Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3”.

[7]	IETF RFC 7157: “Ipv6 Multihoming without Network Address Translation”.

[8]	IETF RFC 4191: “Default Router Preferences and More-Specific Routes”.

[9]	IETF RFC 2131: “Dynamic Host Configuration Protocol”.

[10]	IETF RFC 4862: “Ipv6 Stateless Address Autoconfiguration”.

[11]	ITU-T Recommendation I.130: “Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN”.

[12]	ITU-T Recommendation Q.65: “The unified functional methodology for the characterization of services and network capabilities”.

[13]	3GPP TS 24.301: “Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3”.

[14]	Void.

[15]	3GPP TS 23.228: “IP Multimedia Subsystem (IMS); Stage 2”.

[16]	3GPP TS 22.173: “IMS Multimedia Telephony Service and supplementary services; Stage 1”.

[17]	3GPP TS 23.122: “Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode”.

[18]	3GPP TS 23.167: “3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions”.

[19]	3GPP TS 23.003: “Numbering, Addressing and Identification”.

[20]	IETF RFC 7542: “The Network Access Identifier”.

[21]	3GPP TS 23.002: “Network Architecture”.

[22]	3GPP TS 23.335: “User Data Convergence (UDC); Technical realization and information flows; Stage 2”.

[23]	3GPP TS 23.221: “Architectural requirements”.

[24]	3GPP TS 22.153: “Multimedia priority service”.

[25]	3GPP TS 22.011: “Service Accessibility”.

[26]	3GPP TS 23.401: “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

[27]	3GPP TS 38.300: “NR; NR and NG-RAN Overall Description”.

[28]	3GPP TS 38.331: “NR; Radio Resource Control (RRC); Protocol Specification”.

[29]	3GPP TS 33.501: “Security architecture and procedures for 5G system”.

[30]	3GPP TS 36.300: “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”.

[31]	3GPP TS 37.340: “Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2”.

[32]	3GPP TS 23.214: “Architecture enhancements for control and user plane separation of EPC nodes; Stage 2”.

[33]	3GPP TS 22.101: “3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles”.

[34]	3GPP TS 38.413: “NG-RAN; NG Application Protocol (NGAP)”.

[35]	3GPP TS 33.126: “Lawful Interception Requirements”.

[36]	3GPP TS 23.682: “Architecture enhancements to facilitate communications with packet data networks and applications”.

[37]	3GPP TS 22.280: “Mission Critical Services Common Requirements (MCCoRe); Stage 1”.

[38]	3GPP TS 23.379: “Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2”.

[39]	3GPP TS 23.281: “Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2”.

[40]	3GPP TS 23.282: “Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2”.

[41]	3GPP TS 32.240: “Charging management; Charging architecture and principles”.

[42]	3GPP TS 38.401: “NG-RAN Architecture description”.

[43]	3GPP TS 23.402: “Architecture enhancements for non-3GPP accesses”.

[44]	IETF RFC 4960: “Stream Control Transmission Protocol”.

[45]	3GPP TS 23.503: “Policy and Charging Control Framework for the 5G System”.

[46]	3GPP TS 23.041: “Public Warning System”.

[47]	3GPP TS 24.501: “Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3”.

[48]	3GPP TS 24.502: “Access to the 5G System (5GS) via non-3GPP access networks; Stage 3”.

[49]	3GPP TS 29.500: “5G System; Technical Realization of Service Based Architecture; Stage 3”.

[50]	3GPP TS 38.304: “NR; User Equipment (UE) procedures in idle mode”.

[51]	3GPP TS 36.331: “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification”.

[52]	3GPP TS 36.304: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode”.

[53]	Void.

[54]	IETF RFC 4861: “Neighbor Discovery for IP version 6 (Ipv6)”.

[55]	3GPP TS 23.271: “Functional stage 2 description of Location Services (LCS)”.

[56]	3GPP TS 23.060: “General Packet Radio Service (GPRS); Service description; Stage 2”.

[57]	IETF RFC 4555: “IKEv2 Mobility and Multihoming Protocol (MOBIKE)”.

[58]	3GPP TS 29.510: “5G System: Network function repository services; Stage 3”.

[59]	3GPP TS 29.502: “5G System: Session Management Services: Stage 3”.

[60]	IETF RFC 7296: “Internet Key Exchange Protocol Version 2 (IKEv2) “.

[61]	3GPP TS 23.380: “IMS Restoration Procedures”.

[62]	3GPP TS 24.229: “IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3”.

[63]	3GPP TS 23.292: “IP Multimedia Subsystem (IMS) centralized services; Stage 2”.

[64]	3GPP TS 23.222: “Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs”.

[65]	3GPP TS 29.244: “Interface between the Control Plane and the User Plane Nodes; Stage 3”.

[66]	3GPP TS 32.421: “Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements”.

[67]	3GPP TS 32.290: “5G system; Services, operations and procedures of charging using Service Based Interface (SBI)”.

[68]	3GPP TS 32.255: “5G Data connectivity domain charging; Stage 2”.

[69]	3GPP TS 38.306: “NR; User Equipment -UE) radio access capabilities”.

[70]	3GPP TS 36.306: “Evolved Universal Terrestrial Radio Access -E-UTRA); User Equipment -UE) radio access capabilities”.

[71]	3GPP TS 29.518: “5G System; Access and Mobility Management Services; Stage 3”.

[72]	Void.

[73]	IETF RFC 2865: “Remote Authentication Dial In User Service (RADIUS)”.

[74]	IETF RFC 3162: “RADIUS and Ipv6”.

[75]	3GPP TS 29.281: “General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)”.

[76]	3GPP TS 26.238: “Uplink streaming”.

[77]	3GPP TR 26.939: “Guidelines on the Framework for Live Uplink Streaming (FLUS)”.

[78]	International Telecommunication Union (ITU), Standardization Bureau (TSB): “Operational Bulletin No. 1156”; http://handle.itu.int/11.1002/pub/810cad63-en (retrieved October 5, 2018).

[79]	3GPP TS 28.533: “Management and orchestration; Architecture framework”.

[80]	3GPP TS 24.250: “Protocol for Reliable Data Service; Stage 3”.

[81]	IETF RFC 8684: “TCP Extensions for Multipath Operation with Multiple Addresses”.

[82]	IETF RFC 8803: “0-RTT TCP Convert Protocol”.

[83]	IEEE Std 802.1CB-2017: “IEEE Standard for Local and metropolitan area networks-Frame Replication and Elimination for Reliability”.

[84]	3GPP TS 23.316: “Wireless and wireline convergence access support for the 5G System (5GS)”.

[85]	WiFi Alliance Technical Committee, Hotspot 2.0 Technical Task Group: “Hotspot 2.0 (Release 2) Technical Specification”.

[86]	3GPP TS 23.288: “Architecture enhancements for 5G System (5GS) to support network data analytics services”.

[87]	3GPP TS 23.273: “5G System (5GS) Location Services (LCS); Stage 2”.

[88]	3GPP TS 23.216: “Single Radio Voice Call Continuity (SRVCC); Stage 2”.

[89]	CableLabs DOCSIS MULPI: “Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification”.

[90]	BBF TR-124 issue 5: “Functional Requirements for Broadband Residential Gateway Devices”.

[91]	BBF TR-101 issue 2: “Migration to Ethernet-Based Broadband Aggregation”.

[92]	BBF TR-178 issue 1: “Multi-service Broadband Network Architecture and Nodal Requirements”.

[93]	BBF TR-456 issue 2: “AGF Functional Requirements”.

[94]	BBF WT-457: “FMIF Functional Requirements”.

Editor’s note:	The reference to BBF WT-457 will be revised when finalized by BBF.

[95]	Void.

[96]	Void.

[97]	IEEE Std 802.1AB-2016: “IEEE Standard for Local and metropolitan area networks – Station and Media Access Control Connectivity Discovery”.

[98]	IEEE Std 802.1Q-2022: “IEEE Standard for Local and metropolitan area networks–Bridges and Bridged Networks”.

[99]	3GPP TS 38.423: “NG-RAN; Xn Application Protocol (XnAP)”.

[100]	3GPP TS 36.413: “Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)”.

[101]	3GPP TS 29.274: “Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3”.

[102]	3GPP TS 23.632: “User Data Interworking, Coexistence and Migration; stage 2”.

[103]	3GPP TS 29.563: “5G System (5GS); HSS services for interworking with UDM; Stage 3”.

[104]	IEEE Std 802.1AS-2020: “IEEE Standard for Local and metropolitan area networks–Timing and Synchronization for Time-Sensitive Applications”.

[105]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains”.

[106]	IEEE Std 802.11-2012: “IEEE Standard for Information technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements – Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications”.

[107]	IEEE Std 1588-2008: “IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems”.

[108]	3GPP TS 28.552: “Management and orchestration; 5G performance measurements”.

[109]	3GPP TS 24.193: “Access Traffic Steering, Switching and Splitting; Stage 3”.

[110]	3GPP TS 24.526: “User Equipment (UE) policies for 5G System (5GS); Stage 3”.

[111]	3GPP TS 22.186: “Enhancement of 3GPP support for V2X scenarios; Stage 1”.

[112]	3GPP TR 38.824: “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”.

[113]	IEEE: “Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)”, https://standards.ieee.org/content/dam/ieee-standards/standards/web/documents/tutorials/eui.pdf.

[114]	3GPP TS 32.256: “Charging Management; 5G connection and mobility domain charging; Stage 2”.

[115]	3GPP TS 33.210: “Network Domain Security (NDS); IP network layer security”.

[116]	3GPP TS 38.415: “PDU Session User Plane Protocol”.

[117]	3GPP TS 24.535: “Device-side Time-Sensitive Networking (TSN) Translator (DS-TT) to network-side TSN Translator (NW-TT) protocol aspects; Stage 3”.

[118]	3GPP TS 32.274: “Charging Management; Short Message Service (SMS) charging”.

[119]	3GPP TS 23.008: “Organization of subscriber data”.

[120]	3GPP TS 38.314: “NR; Layer 2 measurements”.

[121]	3GPP TS 23.287: “Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services”.

[122]	3GPP TS 29.503: “5G System; Unified Data Management Services; Stage 3”.

[123]	3GPP TS 32.254: “Charging management; Exposure function Northbound Application Program Interfaces (APIs) charging”.

[124]	3GPP TS 33.535: “Authentication and Key Management for Applications based on 3GPP credentials in the 5G System (5GS)”.

[125]	3GPP TS 38.410: “NG-RAN; NG general aspects and principles”.

[126]	IEEE Std 1588: “IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems”, Edition 2019.

[127]	ST 2059-2:2015: “SMPTE Standard – SMPTE Profile for Use of IEEE-1588 Precision Time Protocol in Professional Broadcast Applications”.

[128]	3GPP TS 23.304: “Proximity based Services (ProSe) in the 5G System (5GS)”.

[129]	3GPP TS 23.247: “Architectural enhancements for 5G multicast-broadcast services”.

[130]	3GPP TS 23.548: “5G System Enhancements for Edge Computing; Stage 2”.

[131]	IEEE Std 802.3: “Ethernet”.

[132]	3GPP TS 29.561: “5G System; Interworking between 5G Network and external Data Networks; Stage 3”.

[133]	3GPP TS 29.513: “Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3”.

[134]	3GPP TS 23.558: “Architecture for enabling Edge Applications (EA)”.

[135]	3GPP TS 26.501: “5G Media Streaming (5GMS); General description and architecture”.

[136]	3GPP TS 23.256: “Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2”.

[137]	GSMA NG.116: “Generic Network Slice Template”.

[138]	IETF RFC 3948: “UDP Encapsulation of Ipsec ESP Packets”.

[139]	3GPP TS 24.539: “5G System (5GS); Network to TSN translator (TT) protocol aspects; Stage 3”.

[140]	3GPP TS 33.220: “Generic Authentication Architecture (GAA); Generic bootstrapping architecture”.

[141]	3GPP TS 33.223: “Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function”.

[142]	3GPP TS 23.540: “Technical realization of Service Based Short Message Service; Stage 2”.

[143]	3GPP TS 38.321: “NR; Medium Access Control (MAC) protocol specification”.

[144]	3GPP TS 29.525: “5G System; UE Policy Control Service; Stage 3”.

[145]	3GPP TS 29.505: “5G System; Usage of the Unified Data Repository Services for Subscription Data; Stage 3”.

[146]	IEEE Std 802.1Qdj-2024: “IEEE Draft Standard for Local and metropolitan area networks – Bridges and Bridged Networks – Amendment XX: Configuration Enhancements for Time-Sensitive Networking”.

[147]	Void.

[148]	3GPP TS 28.557: “Management and orchestration; Management of Non-Public Networks (NPN)”.

[149]	3GPP TS 28.541: “Management and orchestration; 5G Network Resource Model (NRM)”.

[150]	IETF RFC 8655: “Deterministic Networking Architecture”.

[151]	IETF RFC 8343: “A YANG Data Model for Interface Management”.

[152]	IETF RFC 8344: “A YANG Data Model for IP Management”.

[153]	IETF RFC 7224: “ IANA Interface Type YANG Module”.

[154]	IETF draft-ietf-detnet-yang: “Deterministic Networking (DetNet) YANG Model”.

Editor’s note:	The reference to draft-ietf-detnet-yang will be revised to RFC when finalized by IETF.

[155]	IETF RFC 6241: “Network Configuration Protocol (NETCONF)”.

[156]	IETF RFC 8040: “RESTCONF Protocol”.

[157]	IETF RFC 8939: “Deterministic Networking (DetNet) Data Plane: IP”.

[158]	IETF RFC 5279: “A Uniform Resource Name (URN) Namespace for the 3rd Generation Partnership Project (3GPP)”.

[159]	IETF RFC 9330:”Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture”.

[160]	IETF RFC 9331: “Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S)”.

[161]	IETF RFC 9332: “Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)”.

[162]	IETF RFC 6603: “Prefix Exclude Option for DHCPv6-based Prefix Delegation”.

[163]	IETF RFC 8415: “Dynamic Host Configuration Protocol for Ipv6 (DHCPv6)”.

[164]	ITU-T Recommendation G.810: “Definitions and terminology for synchronization networks”.

[165]	3GPP TS 38.470: “NG-RAN; F1 general aspects and principles”.

[166]	IETF RFC 9000: “QUIC: A UDP-Based Multiplexed and Secure Transport”.

[167]	IETF RFC 9001: “Using TLS to Secure QUIC”.

[168]	IETF RFC 9002: “QUIC Loss Detection and Congestion Control”.

[169]	IETF RFC 9221: “An Unreliable Datagram Extension to QUIC”.

[170]	IETF RFC 9298: “Proxying UDP in HTTP”.

[171]	IETF RFC 9114: “Hypertext Transfer Protocol Version 3 (HTTP/3)”.

[172]	IETF RFC 9297: “HTTP Datagrams and the Capsule Protocol”.

[173]	IETF RFC 9220: “Bootstrapping WebSockets with HTTP/3”.

[174]	draft-ietf-quic-multipath: “Multipath Extension for QUIC”.

Editor’s note:	The above document cannot be formally referenced until it is published as an RFC.

[175]	3GPP TS 28.530: “Management and orchestration; Concepts, use cases and requirements”.

[176]	3GPP TS 28.531: “Management and orchestration; Provisioning”.

[177]	3GPP TS 23.434: “Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows”.

[178]	IEEE Std 802.1CBdb-2021: “Amendment 2: Extend Stream Identification Functions”.

[179]	3GPP TS 26.522: “5G Real-time Media Transport Protocol Configurations”.

[180]	3GPP TS 23.586: “Architectural Enhancements to support Ranging based services and Sidelink Positioning”.

[181]	3GPP TS 23.542: “Application layer support for Personal IoT Network”.

[182]	IETF RFC 8415: “Dynamic Host Configuration Protocol for Ipv6 (DHCPv6)”.

[183]	3GPP TS 29.571: “5G System; Common Data Types for Service Based Interfaces; Stage 3”.

[184]	3GPP TS 23.289: “Mission Critical services over 5G System; Stage 2”.

[185]	IETF RFC 3550: “RTP: A Transport Protocol for Real-Time Applications”.

[186]	IETF RFC 3711: “The Secure Real-time Transport Protocol (SRTP)”.

[187]	IETF RFC 6184: “RTP Payload Format for H.264 Video”.

[188]	IETF RFC 7798: “RTP Payload Format for High Efficiency Video Coding (HEVC) “.

[189]	IETF RFC 8285: “A General Mechanism for RTP Header Extensions”.

[190]	3GPP TS 28.405: “Quality of Experience (QoE) measurement collection; Control and configuration”.

[191]	3GPP TS 37.355: “ LTE Positioning Protocol (LPP)”.

[192]	3GPP TS 32.422: “Telecommunication management; Subscriber and equipment trace; Trace control and configuration management”.

[193]	IETF RFC 3168: “The Addition of Explicit Congestion Notification (ECN) to IP”.

[194]	3GPP TS 33.503: “Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)”.

[195]	3GPP TS 38.414: “NG-RAN; NG data transport”.

[196]	3GPP TS 38.799: “Study on Additional Topological Enhancements for NR”.

[Y]	3GPP TS 28.554: “Management and orchestration; 5G end to end Key Performance Indicators (KPI)”.

[x]	3GPP TS 28.310: “Management and orchestration; Energy efficiency of 5G”.

# * * * * Second change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5G DDNMF	5G Direct Discovery Name Management Function

5G LAN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-AN PDB	5G Access Network Packet Delay Budget

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G GM	5G Grand Master

5G NSWO	5G Non-Seamless WLAN offload

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G VN	5G Virtual Network

5QI	5G QoS Identifier

ADRF	Analytics Data Repository Function

AF	Application Function

AI/ML	Artificial Intelligence/Machine Learning

AKMA	Authentication and Key Management for Applications

AnLF	Analytics Logical Function

AMF	Access and Mobility Management Function

AoI	Area of Interest

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CH	Credentials Holder

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

CQRCI	Clock Quality Reporting Control Information

DAPS	Dual Active Protocol Stacks

DCCF	Data Collection Coordination Function

DCS	Default Credentials Server

DetNet	Deterministic Networking

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

EAC	Early Admission Control

EIF	Energy Information Function

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FL	Federated Learning

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GBA	Generic Bootstrapping Architecture

GEO	Geostationary Orbit

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network Selection

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HMTC	High-Performance Machine-Type Communications

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IMEI/TAC	IMEI Type Allocation Code

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

I-UPF	Intermediate UPF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LEO	Low Earth Orbit

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

L4S	Low Latency, Low Loss and Scalable Throughput

MBS	Multicast/Broadcast Service

MBSF	Multicast/Broadcast Service Function

MBSR	Mobile Base Station Relay

MBSTF	Multicast/Broadcast Service Transport Function

MB-SMF	Multicast/Broadcast Session Management Function

MB-UPF	Multicast/Broadcast User Plane Function

MEO	Medium Earth Orbit

MFAF	Messaging Framework Adaptor Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MINT	Minimization of Service Interruption

ML	Machine Learning

MPQUIC	Multi-Path QUIC

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

MTLF	Model Training Logical Function

MWAB	Mobile gNB with wireless access backhauling

N3IWF	Non-3GPP InterWorking Function

N3QAI		Non-3GPP QoS Assistance Information

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NAT	Network Address Translation

NCR	Network Controlled Repeater

NCR-MT	NCR Mobile Termination

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NS-AoS	Network Slice Area of Service

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

NSAG	Network Slice AS Group

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-specific and SNPN Authentication and Authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NSSRG	Network Slice Simultaneous Registration Group

NSWO	Non-Seamless WLAN offload

NSWOF	Non-Seamless WLAN offload Function

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

ONN	Onboarding Network

ON-SNPN	Onboarding Standalone Non-Public Network

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PDV	Packet Delay Variation

PEGC	PIN Element with Gateway Capability

PEI	Permanent Equipment Identifier

PEMC	PIN Element with Management Capability

PER	Packet Error Rate

PFD	Packet Flow Description

PIN	Personal IoT Network

PINE	PIN Element

PLR	Packet Loss Rate

PNI-NPN	Public Network Integrated Non-Public Network

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PSDB	PDU Set Delay Budget

PSER	PDU Set Error Rate

PSIHI	PDU Set Integrated Handling Information

PTP	Precision Time Protocol

PVS	Provisioning Server

QFI	QoS Flow Identifier

QMC	QoE Measurement Collection

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

IAN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

RTT	Round Trip Time

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SF	Service Function

SFC	Service Function Chain

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SO-SNPN	Subscription Owner Standalone Non-Public Network

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TA	Tracking Area

TAI	Tracking Area Identity

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAC	TSC Assistance Container

TSCAI	Traffic Assistance Information

TSCTSF	Time Sensitive Communication and Time Synchronization Function

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TSS	Timing Synchronization Status

TT	TSN Translator

TWIF	Trusted WLAN Interworking Function

UAS NF	Uncrewed Aerial System Network Function

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

# * * * *Third change (all new text) * * * *

5.X	Support of Energy Efficiency and Energy Saving

5.X.1	General

The 5GS supports some features aimed at Energy saving described in the following clauses.

5.X.2	Energy Consumption Information collection, calculation and exposure

5.X.2.1	General

The Energy Information Function (EIF) is defined to collect the Energy Consumption and Related information, calculate the Energy Consumption information at UE, PDU Session and/or QoS flow granularity, store the Energy Consumption information if applicable and expose the Energy Consumption information to the authorized consumer NF(s) (AF or 5GC NF (e.g., NWDAF, PCF)) subject to operator’s policy.

The collected Energy Consumption and Related information includes Node-level energy consumption information, Node-level data volume, and data volume of the required granularities (e.g. QoS flow or PDU session).

The calculated Energy Consumption information is at UE, PDU session and/or QoS flow granularities.

NOTE 1: The EIF can be deployed standalone or co-located with NWDAF.

5.X.2.2	Energy Consumption information collection

The Energy Information Function (EIF) collects the Node-level energy consumption information, Node-level data volume from OAM and data volume of the required granularities from UPF (directly or via SMF).

The EIF invokes the OAM services to retrieve the Node-level energy consumption information (i.e. NG-RAN and UPF(s)) and Node-level data volume (i.e. UPFs) with providing the NF Node ID. OAM provide the information requested by EIF subscription as specified in TS28.554[X]. The EIF gets the gNB ID from AMF and retrieve the UPF ID(s) from SMF.When the gNB and/or the (I-)UPF(s) which serving the UE are changed, the serving gNB ID and UPF ID will be updated to the EIF through AMF/SMF as described in clause 4.3.5.X in TS 23.502[x].

The Node-level Energy Consumption information received from OAM could be used by EIF for all the UEs serving by the NF Node.

For the data volume from UPF of the required granulairities e.g., QoS flow or PDU session, the EIF should send a request to UDM with providing the UE SUPI, and the DNN, S-NSSAI of this PDU session to find the SMF which serving the PDU session. And EIF subscribe the required data volume with invoking Nsmf_EventExposure service, and SMF request the UPF to report the data volume to SMF or EIF directly.

5.X.2.3	 Energy Consumption information calculation

EIF calculates the Energy Consumption information of the required granularies, based on Node-level energy consumption information, Node-level data volume, and data volume of the required granularities based on some formulas or by operator implementation. Some example formulas are described in Annex XX.

5.x.2.4	Energy Consumption information exposure

AF/NEF or 5GC NF (e.g., NWDAF, PCF) may subscribe the EIF for Energy consumption information with providing assisted parameters.

The NEF or 5GC NF discover the EIF as described in clause 5.xxx.

The assisted parameters may include one or more of e.g., UE GPSI/SUPI, UE address, DNN/S-NSSAI, Flow description(s), Area of Interest (AOI), Application ID, Energy related criteria (e.g., maximum/minimum/average /variance value for Energy Consumption), reporting frequency, reporting threshold.

Editor’s NOTE: the other assisted parameters are FFS.

Editor’s NOTE:	 Whether and how the renewable energy is exposed is FFS.

# * * * * Forth change  * * * *

6.2.5	NEF

6.2.5.0	NEF functionality

The Network Exposure Function (NEF) supports the following independent functionality:

-	Exposure of capabilities and events:

NF capabilities and events may be securely exposed by NEF for e.g. 3rd party, Application Functions, Edge Computing as described in clause 5.13.

NEF stores/retrieves information as structured data using a standardized interface (Nudr) to the Unified Data Repository (UDR).

-	Secure provision of information from external application to 3GPP network:

It provides a means for the Application Functions to securely provide information to 3GPP network, e.g. Expected UE Behaviour, 5G-VN group information, time synchronization service information and PDU Set handling service specific information. In that case the NEF may authenticate and authorize and assist in throttling the Application Functions.

-	Translation of internal-external information:

It translates between information exchanged with the AF and information exchanged with the internal network function. For example, it translates between an AF-Service-Identifier and internal 5G Core information such as DNN, S-NSSAI, as described in clause 5.6.7.

In particular, NEF handles masking of network and user sensitive information to external AF's according to the network policy.

-	Redirecting the AF to a more suitable NEF/L-NEF e.g. when serving an AF request for local information exposure and detecting there is a more appropriate NEF instance to serve the AF's request.

-	The Network Exposure Function receives information from other network functions (based on exposed capabilities of other network functions). NEF stores the received information as structured data using a standardized interface to a Unified Data Repository (UDR). The stored information can be accessed and "re-exposed" by the NEF to other network functions and Application Functions, and used for other purposes such as analytics.

-	A NEF may also support a PFD Function: The PFD Function in the NEF may store and retrieve PFD(s) in the UDR and shall provide PFD(s) to the SMF on the request of SMF (pull mode) or on the request of PFD management from NEF (push mode), as described in TS 23.503 [45].

-	A NEF may also support a 5G-VN Group Management Function: The 5G-VN Group Management Function in the NEF may store the 5G-VN group information in the UDR via UDM as described in TS 23.502 [3].

-	Support management of ECS Address Information.

-	Support management of relationship between DNAI and EAS Address Information.

-	Exposure of analytics:

NWDAF analytics may be securely exposed by NEF for external party, as specified in TS 23.288 [86].

-	Retrieval of data from external party by NWDAF:

Data provided by the external party may be collected by NWDAF via NEF for analytics generation purpose. NEF handles and forwards requests and notifications between NWDAF and AF, as specified in TS 23.288 [86].

-	Support of Non-IP Data Delivery:

NEF provides a means for management of NIDD configuration and delivery of MO/MT unstructured data by exposing the NIDD APIs as described in TS 23.502 [3] on the N33/Nnef reference point. See clause 5.31.5.

-	Charging data collection and support of charging interfaces.

-	Support of Member UE selection assistance functionality:

-	NEF may provide one or more list(s) of candidate UE(s) (among the list of target member UE(s) provided by the AF) and additional information to the AF based on the parameters contained in the request from the AF as described in clause 5.46.2. NEF supports the translation of the member UE selection filtering criteria parameters received from the AF to the corresponding event or analytics filters that can be understood by the 5GC NFs for events or analytics related data collection. NEF interacts with 5GC NFs using existing services in order to collect the corresponding data and then derive the list(s) of candidate UE(s) and other assistance information as described in clause 4.15.13 of TS 23.502 [3].

-	Support of Multi-member AF session with required QoS for a set of UEs identified by a list of UE addresses:

-	Details are specified in clause 4.15.6.13 of TS 23.502 [3].

-	Support of UAS NF functionality:

Details are defined in TS 23.256 [136].

-	Support of EAS deployment functionality:

Details are defined in TS 23.548 [130].

-	Support of SBI-based MO SM transmit for MSISDN-less MO SMS:

Details are defined in TS 23.540 [142].

-	Support PDU Set Handling as defined in clause 5.37.5.

-	Support management of common EAS and common DNAI:

Details are defined in TS 23.548 [130].

-	Support request and exposure of energy related information as defined in clause 5.x.

A specific NEF instance may support one or more of the functionalities described above and consequently an individual NEF may support a subset of the APIs specified for capability exposure.

NOTE:	The NEF can access the UDR located in the same PLMN as the NEF.

The services provided by the NEF are specified in clause 7.2.8.

For external exposure of services related to specific UE(s), the NEF resides in the HPLMN. Depending on operator agreements, the NEF in the HPLMN may have interface(s) with NF(s) in the VPLMN.

When a UE is capable of switching between EPC and 5GC, an SCEF+NEF is used for service exposure. See clause 5.17.5 for a description of the SCEF+NEF.

6.2.5.1	Support for CAPIF

When an NEF is used for external exposure, the CAPIF may be supported. When CAPIF is supported, an NEF that is used for external exposure supports the CAPIF API provider domain functions. The CAPIF and associated API provider domain functions are specified in TS 23.222 [64].

# * * * * Eighth change * * * *

6.2.10	AF

The Application Function (AF) interacts with the 3GPP Core Network in order to provide services, for example to support the following:

-	Application Function influence on traffic routing (see clause 5.6.7);

-	Application Function influence on Service Function Chaining (see clause 5.6.16.2);

-	Accessing Network Exposure Function (see clause 5.20);

-	Interacting with the Policy and charging control framework (see clause 5.14);

-	Time synchronization service (see clause 5.27.1.8);

-	IMS interactions with 5GC (see clause 5.16).

-	Support PDU Set Handling as defined in clause 5.37.5.

-	Support request and exposure of energy related information as defined in clause 5.x.

Based on operator deployment, Application Functions considered to be trusted by the operator can be allowed to interact directly with relevant Network Functions.

Application Functions not allowed by the operator to access directly the Network Functions shall use the external exposure framework (see clause 7.3) via the NEF to interact with relevant Network Functions.

The functionality and purpose of Application Functions are only defined in this specification with respect to their interaction with the 3GPP Core Network.

# * * * * Ninth change * * * *

Annex XX (informative):
Network Energy consumption information calculation

# XX.1	General

This Annex provides examples on how EIF calculate the energy consumption for required granularities.

The energy consumed at a UPF (EUPF)or a gNB (EgNB) is known over a time window T. And the EIF is aware of the data volume (DV) consumed at a UPF by a UE (DVUE,UPF), UE PDU session (DVSession,UPF) and UE QoS flow (DVFlow,UPF) over the time interval T. The EIF is aware of the overall data volume at a gNB (DVgNB), as well as at a UPF (DVUPF) which serving the UE. Then based on the below example fomular, the required granularities Energy consumption information can be obtained.

To derive the of energy consumed at a gNB over a period of time T by a UE, PDU session, QoS flow, the formulas are:

EUE,gNB=EgNBDVUE,gNBDVgNB

ESession,gNB=EgNBDVSession,gNBDVgNB

EFlow,gNB=EgNBDVFlow,gNBDVgNB

To derive the energy consumed at a UPF over a time period T by a UE, session, QoS flow the formulas are:

EUE,UPF=EUPFDVUE,UPFDVUPF

ESession, UPF=EUPFDVUE,UPFDVUPF

EFlow,UPF=EUPFDVFlow,UPFDVUPF

To derive the energy consumed in the network in the time interval T by a UE (EUE), Session (ESession), QoS flow (EFlow) the formula is:

EUE=gNBi∈gNBUETEUE, gNBi+ UPFi∈UPFUETEUE, UPFi

where gNBUET are all gNBs used by the UE in an interval T, UPFUET are all UPFs used by the UE in an interval T.

ESession=gNBi∈gNBSessionTESession, gNBi+ UPFi∈UPFSessionTESession, UPFi

where gNBSessionT are all gNBs used by the PDU Session in an interval T, UPFSessionT are all UPFs used by the PDU Session in an interval T.

EFlow=gNBi∈gNBFlowTEFlow, gNBi+ UPFi∈UPFFlowTEFlow, UPFi

where gNBFlowT are all gNBs used by the QoS Flow in an interval T, UPFFlowT are all UPFs used by the QoS Flow in an interval T.