3GPP TSG-SA2 Meeting #166	S2-2411429

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0486 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for multihop L3 U2N relay reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to clause 6.5.3, U2N relay reselection procedure is supported with Remote UE’s monitoring on PC5 link. But for multihop U2N relay connection, relay reselection may be triggered by Remote UE or Intermediate Relay when they detect link quality degradation or link failure. The U2N relay reselection procedure needs to be updated to support multihop U2N relay connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Multihop L3 U2N Relay reselection procedure is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multihop L3 U2N Relay does not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

### 6.5.3	5G ProSe UE-to-Network Relay reselection

After being connected to the 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE keeps performing the measurement of the signal strength of PC5 unicast link with the 5G ProSe UE-to-Network Relay for relay reselection.

The 5G ProSe UE-to-Network Relay discovery procedures defined in clause 6.3.2.3 are used to discover available 5G ProSe UE-to-Network Relays for 5G ProSe UE-to-Network Relay reselection.

#### 6.5.3.X 5G ProSe multihop UE-to-Network Relay reselection

After being connected to the 5G ProSe UE-to-Network Relay, the 5G ProSe Remote UE and 5G ProSe Intermediate UE-to-Network Relay keep performing the measurement of the signal strength of PC5 unicast link with its parent Relay (Intermediate Relay or U2N relay) for relay reselection.

When a 5G ProSe Intermediate UE-to-Network Relay detect the link failure or link quality degradation, the 5G ProSe Intermediate UE-to-Network Relay informs the 5G ProSe Remote UE the link failure or link quality degradation of the PC5 link so that the 5G ProSe Remote UE may initiate multihop UE-to-Network Relay reselection procedure. The 5G ProSe Intermediate UE-to-Network Relay may include alternative multihop path to U2N Relay if it is available.

The 5G ProSe Remote UE may perform the 5G ProSe UE-to-Network Relay discovery procedures defined in clause 6.3.2.3 to discover available 5G ProSe UE-to-Network Relays or 5G ProSe multi-hop UE-to-Network Relay Discovery procedures defined in clause 6.3.2.5 to discover available multihop path to the 5G ProSe UE-to-Network Relays for 5G ProSe (multihop) UE-to-Network Relay reselection.

The 5G ProSe Remote UE may perform U2N Relay Communication setup procedure defined in clause 6.5.1.

After 5G ProSe multihop U2N relay reselection procedure, if the same 5G ProSe UE-to-Network Relay is used for connection with a 5G ProSe Remote UE, the existing IP address of the 5G ProSe Remote UE is reused. The 5G ProSe UE-to-Network Relay may update the routing context for the remote UE with its new child 5G ProSe Intermediate UE-to-Network Relay and the 5G ProSe UE-to-Network Relay may inform IP address of the 5G ProSe Remote UE to its child 5G ProSe Intermediate UE-to-Network Relay(s).

During 5G ProSe multihop U2N relay reselection procedure, an Intermediate UE-to-Network Relay may decide to reuse existing connection with its parent Relay UE (an Intermediate UE-to-Network Relay or the UE-to-Network relay) for a 5G ProSe Remote UE based on path information in the received DCR or LMR. The Intermediate UE-to-Network Relay may send a DCR or a LMA to its child Intermediate UE-to-Network Relay without sending DCR or LMR to its parent Relay (Intermediate UE-to-Network Relay or UE-to-Network Relay).  The 5G ProSe Intermediate UE-to-Network Relay may update the routing context for the 5G ProSe Remote UE with its new child 5G ProSe Intermediate UE-to-Network Relay and the Intermediate UE-to-Network Relay may inform the IP address of Remote UE to its child Intermediate 5G ProSe UE-to-Network Relay(s).

* * *End of Changes * * *