SA WG2 Meeting #166	S2-2411450

# Orlando, USA, 18 – 22 November, 2024                        (revision of S2-24xxxxx)

Source: 	NEC

Title: 	AIoT conclusion for Simple AIoT operation in certain deployment on Key issue#2

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes AIoT conclusion for no security on Key issue#2.

1	Discussion

In the TR 23.700-13 v1.1.0, the solution 41: Simple AIoT device and AIoT solution in the certain deployment describes as below:

One typical AIoT use case is inventory in the warehouse, where the environment is closed and controllable by the warehouse owner (as the 3rd party/AF) and the AIoT network operator.

For the AIoT device with very limited capabilities and power can access to the network, the AIoT device operation including security and privacy can be compromised if it is agreed by the AIoT network operator and the 3rd party.

In this solution, the AIoT device does not need to have security or privacy capabilities internally. For the AIoT device having security or privacy capabilities, either the warehouse owner or the network operator configures the AIoT device without security or privacy enabled in this case. From the AIoT network and 3rd party’s perspectives, the device only needs to store its device ID, and responds to a reader with its device ID when the reader requests for inventory. Based on the agreement between AF and AIoT device management, AIoT controller authorizes the simple AIoT operation without security and privacy.

Figure 6.41.2-1: Inventory procedure

The solution can be concluded in the Conclusion on Key Issue #2 in TR 23.700-13.

2	Proposal

It is proposed to update TR 23.700-13 as following.

# * * * * Start of Change* * * *

8.2	Conclusion on Key Issue #2

8.2.x	Simple AIoT operation in certain deployment

When the AIoT operation environment is closed and controllable by the environment administrator and the AIoT network operator, the AIoT device operation including security and privacy can be compromised if it is agreed by them. The AIoT core network authorizes the simple AIoT operation without security and privacy between the AIoT device and the Reader.

# * * * * End of Changes * * * *