3GPP TSG-SA WG2 Meeting #166	S2-2411464

# Orlando, USA, 18th Nov – 22nd Nov, 2024								(revision of S2-2410987)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1492 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call setup of UE-Satellite-UE communication in IMS with IMS-AGW on board |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Huawei, HiSilicon, China Telecom, CATT], NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on 5GSAT_Ph3_ARCH WID work task, the functionality and procedure of UE-satellite-UE communication via IMS-AGW deployed on satellite should be captured in TS 23.228. For call setup: When the originating P-CSCF receives the initial INVITE request from the originating UE using satellite access, it is not able to determine whether the UE-Satellite-UE communication can be activated before negotiating with the terminating side. The originating P-CSCF selects an IMS-AGW on ground for the originating UE by default after receiving the INVITE message and triggers media negotiation by sending SDP offer to the terminate side. If the terminating side determines that it is possible to activate UE-Satellite-UE communication, then, a re-selection/re-negotiation procedure is needed for the originating P-CSCF to re-select the IMS-AGW on satellite for the originating UE to enable the UE-Satellite-UE communication. Revision 4: Using AF (P-CSCF) subscription to user plane management event notifications instead of Access Network Information to retrieve satellite identifier. This would mean a change to 23.503 (CR#1429) to add satellite identifier in 23.503 is not required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of support of UE-Satellite-UE communication in IMS, including: - Call setup procedures of UE-SAT-UE communication. Revision 4: P-CSCF subscription to user plane management event notifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3_ARCH work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AX.5 (new), AX.5.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented after TS 23.228 CR 1428. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change (All following text are new) ---

# AX.5	Procedures for optimized media routing

## AX.5.1	At call setup

When the originating P-CSCF receives the initial INVITE request from the originating UE using regenerative NR satellite access, it selects the IMS-AGW on ground and triggers media negotiation by sending SDP offer to the terminating side indicating that satellite access is used on the originating side. If the terminating P-CSCF activates UE-satellite-UE communication, the originating P-CSCF re-selects an IMS-AGW on satellite. See Figure AX.5.1-1 that describes the procedure.

NOTE X: To support the UE-satellite-UE communication, P-CSCFs (AF) need to support N5 interface.

Editor's note: 	Whether and how to take UE subscription aspect into account for UE-satellite-UE communication activation is FFS.

Editor's note: The call flow presented below may be revised e.g., based on whether there is any unnecessary impact on H.248 [18] or based on discussion on satellite change procedure.

Editor's note: It is FFS whether early media traffic can be supported or not for the UE-Sat-UE communication during call setup.

Editor's note: It is FFS how P-CSCF subscribes to satellite change events from SMF, e.g., based on UP Path change event or Access Network Information PCRT.

Figure AX.5.1-1:	Session establishment procedure with activation of optimized media routing

1.	UE A sends a SIP INVITE request, containing an initial SDP offer, to P-CSCF A. The access information in the SIP INVITE request indicates that the UE is accessing from NR satellites access as defined in TS 24.229 [10a].

2.	After receiving the SIP INVITE request indicating NR satellite access, if UE A is in the non-roaming scenario and the P-CSCF A supports UE-Satellite-UE communication, it may perform following actions to determine whether UE-satellite-UE communication is possible:

- 	P-CSCF A subscribes to user plane management event notifications as described in clause 4.3.6.3 in TS 23.502 [94] using the Npcf_PolicyAuthorization service The P-CSCF includes the list of DNAIs to the subscription.

- 	The SMF selects a local UPF based on the UE's current serving satellite identifier and the corresponding target DNAI. The SMF notifies the P-CSCF for the UP path change event (Early Notification) with the target DNAI and UE's Satellite ID.

The P-CSCF determines whether the UE A is using regenerative satellite access or not based on whether a serving satellite identification is obtained as part of the Early Notification.

3.	The P-CSCF A allocates an IMS-AGW A' on the ground as default. P-CSCF A interacts with IMS-AGW A' to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 4-20 are performed if the UE-Satellite-UE communication is possible based on the decision in step 2 (i.e., UE A is using NR generate satellite access and in non-roaming scenario). Otherwise, SIP message routing procedures with IMS-AGW on ground should be performed after P-CSCF A selecting a ground IMS-AGW for usage (details have been omitted from the diagram).

4.	P-CSCF A updates the SIP INVITE request inserting the identification of the satellite serving UE A and updates SDP offer with the IMS-AGW transport addresses allocated in step 3. Then, P-CSCF A forwards the updated SIP INVITE request to the terminating side via IMS core (details have been omitted from the diagram).

5.	After receiving the SIP INVITE request which contains a satellite identification indicating that UE A is using regenerative satellite access, if UE B is not in roaming, P-CSCF B subscribes to user plane management event notifications as described in clause 4.3.6.3 in TS 23.502 [94] using the Npcf_PolicyAuthorization service. If the UE B is also using the regenerative satellite access, the serving satellite identification is contained in the notification.

6.	When UE B is not in roaming, and the two satellites identified by the two satellite identifications received in step 4 and 5 are same or connected with ISL(s), P-CSCF B determines to activate the UE-Satellite-communication.

If UE-Satellite-UE communications is not activated, P-CSCF B selects an IMS-AGW B' on ground for UE B; otherwise, P-CSCF B selects an IMS-AGW B on satellite for UE B.

NOTE Y:	How the terminating P-CSCF uses the satellite identifications to derive whether the two satellites are connected with ISL(s) is not standardized in this release and depends on operators’ policy e.g., by querying transport network system or based on node level information reported by the IMS-AGW deployed on the satellites which presents that the two satellite are connected or not.

6a-6b. Based on the decision in step 6, P-CSCF B interacts with IMS-AGW B on satellite or IMS-AGW B' on ground to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 7-20 are performed if the UE-Satellite-UE communication is activated. Otherwise, SIP message routing procedures with IMS-AGW on the ground should be performed after P-CSCF B selecting a ground IMS-AGW B' for usage (details have been omitted from the diagram).

7-9. P-CSCF B forwards the updated SIP INVITE request to UE B. UE B returns a 18X response with SDP Answer.

9a-9b. After receiving the SDP Answer, P-CSCF B interacts with the IMS-AGW B to allocate transport resources based on the mechanism defined in TS 23.334 [74].

10.	P-CSCF B instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service operation.

To indicate SMF to insert UL CL/BP and L-PSA UPF for IMS PDU session to enable UE-Satellite-UE communication, P-CSCF B should send following parameters via the request based on Application Function influence on traffic routing mechanism described in clause 5.6.7 of TS 23.501 [93] and clause 4.3.6 of TS 23.502 [94]:

-	A flow description information which contains the IP address of the IMS-AGW B;

-	A DNAI associated with the IMS-AGW B on satellite and corresponding N6 traffic routing information.

11.	P-CSCF B updates the 18X response to carry the identifier of the satellite serving UE B to indicate that UE-Satellite-UE communication is activated. Then, P-CSCF B returns the updated 18X response to originating side via IMS core (details have been omitted from the diagram).

12.	Based on the identification of the satellite serving UE B in step 11, P-CSCF A determines to perform UE-satellite-UE communication and select an IMS-AGW A on satellite.

12a-12c. P-CSCF A interacts with IMS-AGW A on satellite to allocate transport resources for both IMS access and core network sides. P-CSCF A also releases the IMS-AGW A' on ground.

13. 

P-CSCF sends a positive acknowledgement for the Early Notification received in Step 2 and SMF inserts the local UPF into the satellite.

P-CSCF A instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service operation.

To indicate SMF to insert UL CL/BP and L-PSA UPF for IMS PDU session to enable UE-Satellite-UE communication, P-CSCF B should send following parameters via the request based on Application Function influence on traffic routing mechanism described in clause 5.6.7 of TS 23.501 [93] and clause 4.3.6 of TS 23.502 [94]:

-	A flow description information which contains the IP address of the IMS-AGW A;

-	A DNAI associated with the IMS-AGW A on satellite and corresponding N6 traffic routing information.

14.	P-CSCF A updates the SDP answer in the 18X response with the IMS-AGW transport addresses allocated in step 12, and forwards the updated 18X response to UE A.

15.	UE A acknowledges the Response and sends the Response Confirmation to P-CSCF A.

16-17. P-CSCF A forwards the Response Confirmation to P-CSCF B. The Response Confirmation includes SDP offer modified/generated by P-CSCF A which contains the IMS-AGW transport address obtained in step 12. Upon receiving the SDP offer, P-CSCF B updates the allocated IMS-AGW on the satellite for UE B with the transport address in the SDP offer.

18.	P-CSCF B sends the Response Confirmation to UE B.

19.	Procedure continues to setup the call as defined in clause 5.6 and 5.7.

## ---End of the Change---