3GPP SA WG2 Meeting #166	S2-2411524

# Orlando, FL,USA, 18-22 November 2024								                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2139 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | URSP rules applied for non-3GPP devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support non-3GPP devices connecting behind a 5G-RG, based on the conclusions in clause 8.4 of TR 23.700-32, S2-2411020 has been agreed in SA2#165, which shows the example of PDU Sessions based on Connectivity Group ID for the traffic of non-3GPP devices connecting behind a 5G-RG. But how the Connectivity Group ID in URSP is correlated with non-3GPP devices is not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifiy the correlation between Connectivity Group and non-3GPP devices connecting behind a 5G-RG. Updating the UE access selection and PDU Session selection related policy information for non-3GPP device connecting behind a 5G-RG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of clarification on how the existing URSP parameters are used for non-3GPP device connecting behind a 5G-RG. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10b, 9.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR refers to TS 23.501 clause 5.X as specified in the CR #5750 (S2-2411269). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

## 4.10b	Differentiated services for NAUN3 devices behind 5G-RG

NAUN3 devices cannot be authenticated by 5GC but may e.g. be locally authenticated by the 5G-RG using e.g. pre-shared secret. Differentiated services (QoS, network slicing) may be provided for NAUN3 devices as defined in this clause.

"Connectivity Group IDs" may be defined on the 5G-RG where each Connectivity Group ID corresponds to a separate physical or virtual port on the 5G-RG. These ports could, for example, refer to separate physical ethernet ports and/or to separate WLAN SSIDs and/or to a separate VLAN and/or to a group of non-3GPP devices with traffic subject to QoS differentiation as defined in TS 23.501 [2] clause 5.X. The devices that connect to a certain logical port are considered part of the same Connectivity Group ID. How this configuration on the 5G-RG is done is out of scope of this specification.

Each Connectivity Group ID is then mapped to a separate PDU Session that is established by the 5G-RG based on the procedures defined in clause 7. The overall architecture is illustrated in Figure-4.10b-1.

Figure 4.10b-1: Example scenario for NAUN3 devices behind 5G-RG based on connectivity groups

The 5G-RG is configured with the (virtual) port information (e.g. VLANs and SSIDs) based on TR-69 [18], TR-360 and TR-181 [46]. URSP rules can be provided to the 5G-RG to indicate how to map Connectivity Group ID to the parameters of the PDU Session used to carry the traffic of corresponding devices e.g. DNN, S-NSSAI, etc.

NOTE:	In addition, the mapping between a "virtual port" and DNN/S-NSSAI can be configured via TR-69 [18]/TR-181 [46].

Whether and how the NAUN3 devices are configured to use a specific SSID or connect to a certain Ethernet port on the 5G-RG is out of scope of this specification.

Differentiation of charging and QoS may be provided via PCC rules (for different service flows) related with dedicated PDU Sessions for NAUN3 devices. Isolation of devices using a specific Connectivity Group ID into a specific network slice, i.e. with separate S-NSSAI may also be provided.

# >>>>NEXT CHANGE<<<<

#### 9.5.2.1	5G-RG

This clause specifies the delta related to UE policy distribution defined in TS 23.503 [4] clause 6.1.2.2 and related to URSP defined in TS 23.503 [4] clause 6.6. for 5G-RG.

Only URSP policies are applicable for a 5G -RG.

NOTE:	For example, ANDSP policies, ProSe Layer-3 UE-to-Network Relay Offload Policy are not applicable to 5G-RG,

If the PCF provides a URSP policy to the 5G-RG, the PCF should only provide the following URSP policy information:

-	Rule Precedence.

-	Traffic descriptor/Application descriptors, IP descriptors, Domain descriptors, Non-IP descriptors, DNN, Connection Capabilities.

-	Route Selection Descriptor Precedence.

-	Route selection components/ SSC Mode Selection, Network Slice Selection, DNN Selection, PDU Session Type Selection.

-	Route Selection Validation Criteria/ Time Window.

The 5G-RG shall ignore any of the parameters and policies not listed above, if received from the 5GC.

The 5G-RG shall use the URSP policy as specified in TS 23.503 [4], for example for the association of application and PDU session, slices, etc.

The URSP indicates for the application of Auto-Configuration Server (ACS) which PDU session type, NSSAI and/or DNN is to be used. The 5G-RG establishes the connectivity to the management entity (e.g. ACS) via user plane connection on a PDU session according to the URSP.

UE Policy procedures defined in clause 6.1.2.2 of TS 23.503 [4] are applicable as follows:

-	Roaming is not applicable to W-5GAN access in this release of specification.

-	USRP rule enforcement reporting is not defined for 5G-RG.

In the case of an Authenticable Non-3GPP (AUN3) device behind a 5G-RG only URSP policies are applicable. Each AUN3 device is managed in 5G-RG as a separate device with its own NAS and states, therefore from point of view of 5GC, the AUN3 device is seen as a single device and the URSP received in the NAS dedicated to an AUN3 device shall apply to this AUN3 device only the 5G-RG shall apply URSP received within NAS for AUN3 device acting on behalf of AUN3 device. Therefore to support the case when AUN3 devices may be connected via 5G-RG, specific URSP rules may be configured by the PCF for the SUPI associated with the AUN3 device.

UE Route Selection Policy information targeting an AUN3 device (i.e. sent to a 5G-RG in the NAS connection corresponding to an AUN3 device) follows the structure defined in clause 6.2.2 of TS 23.503 [4] with the following difference:

-	As an AUN3 can have only one PDU Session, its URSP shall contain a match all TD.

USRP rule enforcement reporting is not defined for AUN3 devices.

In order to support the case when NAUN3 devices or non-3GPP devices with traffic subject to QoS differentiation as defined in TS 23.501 [2] clause 5.X may be connected via 5G-RG, specific URSP rules may be configured by the PCF on 5G-RG.

URSP rules for NAUN3 devices or non-3GPP devices with traffic subject to QoS differentiation as defined in TS 23.501 [2] clause 5.X connected to 5G-RG follow the structure defined in clause 6.6.2 of TS 23.503 [4] and may contain any combination of the following traffic descriptors:

-	IP Descriptors: For IP traffic from NAUN3 devices or non-3GPP devices with traffic subject to QoS differentiation connected to 5G-RG, IP descriptors are matched against header information contained in IP packets sent by NAUN3 devices or non-3GPP devices with traffic subject to QoS differentiation; IP descriptors are only applicable for traffic from NAUN3 devices or non-3GPP devices with traffic subject to QoS differentiation if network address translation (NAT) is performed for that traffic.

-	Non-IP descriptors: For Ethernet traffic from NAUN3 devices or non-3GPP devices with traffic subject to QoS differentiation connected to 5G-RG, Non-IP descriptors are matched against header information contained in Ethernet frames sent by NAUN3 devices or non-3GPP devices with traffic subject to QoS differentiation.

-	Connectivity Group ID: For traffic from a NAUN3 device or non-3GPP devices with traffic subject to QoS differentiation connected to 5G-RG, Connectivity Group ID in the URSP rule is matched against the Connectivity Group ID that the NAUN3 device or non-3GPP devices with traffic subject to QoS differentiation is associated with (see clause 4.10b).

# >>>>END OF CHANGES<<<<