3GPP TSG-WG SA2 Meeting #166	S2-2411599

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0269 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: UE mobility for PDU session supporting local offloading management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For a PDU session supporting local offloading management, when UE moves, the insertion/change/removal of I-SMF should be considered. Since the architecture and mechanism of local offloading management are similar with ETSUN, it is proposed to only describe the different part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause for UE mobility in local offloading management scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE mobility in local offloading management scenario cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.X.2.3	UE mobility for PDU session supporting Local Offloading Management

For a PDU session supporting Local Offloading Management, during mobility events, e.g. Hand-Over or AMF change, the AMF selects new I-SMF as follows:

- To support the local offloading management, the I-SMF selected need support Local Offloading Management, serves all DNAIs within its service area and the new UE location, DNN and S-NSSAI associated with the PDU session.

- If only SMF is serving the PDU session and the service area of the SMF does not include the new UE location, the AMF selects and inserts an I-SMF.

- If the existing I-SMF cannot serve the UE location, then the AMF selects a new I-SMF and initiates I-SMF change.

- If the existing I-SMF cannot serve the new UE location, but the service area of SMF include the new UE location and the SMF can serve all DNAIs within its service area, the AMF removes the I-SMF and interfaces with the SMF directly.

For I-SMF insertion and relocation case, the target I-SMF and SMF interacts as follows:

- The target I-SMF sends new selected EASDF IP address and Local Offloading Management request to the SMF.

The SM context for retrival includes Local Offloading Management authorization result.

- The SMF sends the new selected EASDF IP address to the UE via PCO.

For I-SMF removal case, the SMF performs as follows:

- A new EASDF is selected by the SMF and notified to the UE via PCO.

Except above difference on the I-SMF insertion/change/removal, the mobility handling is same as the existing respective mobility handling defined in clause 4.23 of TS23.502[3]

# * * * * End of changes * * * *