3GPP TSG-SA2 Meeting #166	S2-2411615

# 18 - 22 November, 2024, Orlando, USA										revision of S2-2410683

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1278 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Preparation Procedure for Vertical Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TR 23.700-84, the following principles have been agreed in clause 8.2: P#2.3: For sample alignment for VFL: P#2.3.1: For NWDAF acting as VFL Server, NWDAF triggers sample alignment, may query NWDAFs or AFs acting as VFL clients to query availability of samples, and generates the interclause of samples. P#2.3.2 In case of VFL between the NWDAF as VFL server and AF as VFL client, the NEF may be involved in sample alignment with mapping of information (e.g. internal versus external information) , then NWDAF as VFL server will determine the final list of participants supporting the samples for VFL training. P#2.3.3: For AF acting as VFL Server and NWDAFs acting as VFL clients, AF performs sample alignment, selects samples to be used in the training process and generates the interclause of samples. P#2.3.4: Feature description information may be registered to the NRF or locally configured in the NWDAF or the AF, negotiated between VFL server and VFL client when performing feature alignment. Feature alignment is optional in a VFL process. In this CR, procedure for Vertical Federated Learning preparation is introduced to support the agreed principles. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce procedure for Vertical Federated Learning preparation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreed principle for AIML_CN cannot be fully supported by 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.2 (new), 6.2H.2.2.0 (new), 6.2H.2.2.1 (new), 6.2H.2.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes ()* * *

#### 6.2H.2.2	Preparation procedure for Vertical Federated Learning

##### 6.2H.2.2.0	General

Preparation procedure is used to check if the VFL Client(s) can meet the ML Model training requirement. The procedure includes the negotiation, between server and client(s) to enable interoperability, sample alignment and optionally feature alignment.

NOTE 1: 	Features can be non-specified, privacy protected and in this case feature alignment is a simple alignment using info registered in NRF, e.g. data sources info or proprietary Feature ID).

Editor´s note: Possible refinement of the selection of features during the preparation phase is FFS.

NOTE 2:	Vertical Federated Learning preparation procedure can be skipped if the VFL Server can decide which VFL Client(s) support the VFL procedure to be performed, e.g. based on local configuration.

##### 6.2H.2.2.1	Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when NWDAF is the VFL Server

Editor's note:	Whether the VFL Interoperability information (e.g. type of supported VFL training method(s)) needs to be part of the NF profile registered in NRF is FFS.

Editor´s Note: Whether the sample IDs are included as part of the MLModelTraining or a separate Nnwdaf service is defined is FFS.

Editor´s Note: The preparation procedure when an untrustrd AF is part of the procedure is FFS.

Editor´s note:	For UEs as samples, additional discussion is needed on whether UE needs to be registerd or not.

1.	The VFL Server sends a Federated Learning preparation request to each of the VFL Client(s) with a VFL Preparation Flag. The Server may add a list of sample IDs, Analytics ID and optionally Feature ID.

For the VFL client NWDAF or VFL client AF, the VFL server invokes Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service.

For the VFL client untrusted AF, the VFL server sends the Federated Learning preparation request via NEF. The NEF may maps the internal IDs of the samples into external IDs before sending the Federated Learning preparation request to the untrusted AF VFL client.

Editor’s Note: the service operations deployed in step1 are FFS.

2.	Each VFL Client checks if it can meet the ML Model training requirement. If it cannot meet the requirements for any reason, it may decide which requirements it can agree to.

Editor´s note:	Clarifications which parameters are regarded as requirements and can be modified by the VFL client is required

3.	Each VFL Client sends Federated Learning preparation response to indicate to the VFL Server whether it accepts the requirements, whether it requests new requirements or whether it will not join. The client includes the requirements it can accept or may add a reason if it cannot join the FL process. The VFL Client also includes the sample ID(s) and optionally Feature ID(s) it can support.

For the VFL client NWDAF or VFL client AF, the VFL client invokes invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response service operation, based on the service operation used in step 1.

For the VFL client untrusted AF, the VFL client sends the Federated Learning preparation response to the VFL server via NEF. The NEF may maps the external IDs of the samples into internal IDs before sending the Federated Learning preparation response to the VFL server NWDAF.

If the notify or response includes new requested requirements, procedure from step 1 is repeated.

Editor´s note: It is ffs whether the VFL client can only reduce the requirements (e.g. samples) or also add new requirements. It is also ffs whether step 1 needs to be repeated if requirements have been reduced by the VFL client.

Editor’s Note: the service operations deployed in step 3 are FFS.

4.	The VFL Server NWDAF determines the VFL Client(s) to be involved in the VFL procedures based on theinformation received in step 6 in Figure 6.2H.2.1-1 (if performed) and other information (e.g. sample IDs and optional the feature IDs can be supported by each VFL client) received in step 3 (if available).

##### 6.2H.2.2.2	Preparation procedure for Vertical Federated Learning when the untrusted AF as the VFL server

This clause specifies the preparation (including sample alignment) procedure for untrusted-AF-initiated VFL scenarios between untrusted AF and NWDAF(s) within a single PLMN.

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL Server

Editor´s Note: Whether the sample IDs are included as part of the MLModelTraining or a separate Nnwdaf service is defined is FFS.

  - The untrusted AF as VFL server sends VFL preparation request to the candidate VFL client(s) NWDAF via NEF with a VFL Preparation Flag to check if the VFL client(s) can meet the ML Model training requirement (e.g. Analytics ID, Sample alignment requirement, VFL Availability time requirement (time span needed for the VFL process), optional feature requirement, etc.). Sample alignment requirement and feature alignment requirement (if any) include a list of sample IDs and feature IDs targeted for VFL training as defined by the AF, respectively.

Editor´s note:	It is ffs whether .the VFL server sends a separate message for each VFL client towards the NEF. Usage of temporary NWDAF IDs needs to be added.

  - The NEF maps the external UE IDs (e.g. GPSIs) to the internal UE IDs (e.g. SUPIs). The NEF sends VFL preparation request to each candidate VFL client using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service when multiple candidate VFL clients are involved with the same information as provided in step 1 in 6.2H.2.2.1.

Editor's note:	Whether to use ML Model interoperability information or introduce new identifiers for interoperability among NWDAF and AFs is FFS.

Editor’s Note:	The specific service name for the untrusted AF case is FFS.

  - Same as step 1 in 6.2H.2.2-1.

  - Same as step 2 in 6.2H.2.2-1.

  - Same as step 3 in 6.2H.2.2-1.

  - When multiple candidate VFL clients are involved, the NEF may aggregate the response from each candidate VFL client

The NEF determines the sample ID intersection among the VFL clients (if more than one VFL clients are involved) based on the information received in step 5. Then the NEF only maps the intersected sample IDs from internal IDs into external IDs and include the interested sample IDs into VFL Preparation response message to the untrusted AF..

  - The NEF sends VFL preparation response to VFL server to indicate the intersect sample IDs among the VFL clients and other received information.

  - The VFL Server AF determines the VFL Client(s) to be involved in the VFL procedures.

* * *Next Change (new text) * * *

* * *End of Changes * * *