3GPP TSG-WG SA2 Meeting #166	S2-2411628

18th – 22nd November, 2024, Orlando, US

Source:	China Mobile

Title:	Conclusions for KI#2 on Identifier and ID Management

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Conclusions for KI#2 are proposed.

1. Introduction/Discussion

With regard to the Identifier and Identification Management, we have the following concerns:

- As the write operation consumes more power, the coverage will be reduced when writing into Ambient IoT devices via radio interface. In addition, writing frequently may impact the performance of devices. Thus, it is better to avoid writing frequently by the network for device identifier management.

- The other concern is about the Editor’s note for operator allocated Identifier. For operator allocated Identifier, the authentication server of operator holds the credential. When the Ambient IoT devices connect with 5GC, the network can locate the credential holder based on the Network Identifier. Thus, there is no need to maintain the credential holder ID in Part1.

Proposal:

- The 5GS shall only support read-only operations to manage Device Identifiers in Ambient IoT Devices.

- Delete the Editor's note: For operator allocated Identifier, it is FFS whether further information (e,g. the credential holder ID) in Part1 is needed.

2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13.

FIRST CHANGE

8.2	Conclusion on Key Issue #2

8.2.1	Identifier and Identification Management

The following aspects and principles are considered and agreed for the interim conclusion on Identifier  and Identification Management:

The following principles are agreed for Identifier and Identification Management:

-	An Ambient IoT Device is configured with a permanent Ambient IoT Device Identifier which can be assigned by an operator or by a third party. The Identifier is used to identify Ambient IoT Device and locate the corresponding authentication server.

NOTE 1:	How to configure Ambient IoT Device with the permanent Ambient IoT Device Identifier is out of 3GPP scope.

-    The 5GS shall only support read-only operations to manage Device Identifiers in Ambient IoT Devices.

The permanent Ambient IoT Device Identifier includes the following information:

1)	Part1information:

-	The ID type, which is used to indicate the identifier is assigned by an operator or by a third party.

-	The operational entity, which is the entity managing and allocating the Identifier:

-	For operator allocated ID, it is a network identifier (i.e. MCC+MNC and/or NID).

-	For third party allocated ID, it is information used to identify a 3rd party.

Editor's note:	whether the operational entity is always needed is FFS.

2)	Part2 information

-	the information (e.g. EPC or others) used to distinguish different Ambient IoT Devices within the scope identified by the Part1 information.

NOTE 2:	If the information used to identify the 3rd party is the scheme and the company prefix of the EPC, Part2 information is not necessary to be the whole EPC but can be the leftover part of the EPC.

NOTE 3:	The coding for the above information is left to stage3.

Figure 8.2.1-1 The Operator allocated ID

For operator allocated Identifier, the network identifier is mandatory and can be used to index the authentication server or not.

Figure 8.2.1-2 The third party allocated ID

For third party allocated Identifier, the network identifier is not needed. The third party may be the credential holder or not. If it is not the crendetial holder, the network should be provided with third party related context including the information used to locate the authentication server.

Editor's note:	The length of Identifier is fixed or dynamical is FFS.

Editor's note:	The uniqueness of Identifier is FFS.

Editor's note:	Whether the temporary ID in the AIoT NAS layer is required for the privacy protection is FFS and is pending SA3 decision.

END OF CHANGES