3GPP TSG-WG SA2 Meeting #166	S2-2411640

18th – 22nd November 2024, Orlando FL USA

Source:	Ericsson

Title:	Discussion on VFL item 3 and 4

Document for:	Agreement

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract: In this paper item 3 and 4 in work plan is discussed

# Introduction

The items 3 and 4 discussion and observations are described below.

### 1.1 Item 4 - Interaction between the VFL server and the consumer

The description of item 4 according to the list of issues shared by the rapporteur is:

Relevant ENs in existing TS or approved CRs:

Editor's note:	Whether the case that the AnLF can not generathe the analytics output but can determines the VFL server for the requested analytics is exist is FFS. Whether and how such NWDAF is capable to determine the NWDAF as VFL server is FFS. (CR: 2411192 – OPPO; Clause: 6.2H.2.4.1)

Editor’s Note: Further extensions are needed to show the interaction between consumer and VFL server. For example, how the consumer (i.e., NWDAF containing AnLF) sends a subscription request to VFL server (CR: 2411194 – CMCC; Clause: 6.2H.2.3.1)

The key points to be discussed:

- Who can be the consumer of VFL server?

- Under which condition the consumer will interact with the VFL server?

- What (e.g. analytics output, ML model or other new info) and how to interact with VFL server by the consumer?

The consumer of NWDAF containing MTLF, when using MLModelProvosioning is believed to be an NWDAF containing AnLF. For training, a VFL server is either an NWDAF or an AF.

For inference, the server is already decided during training, and can be either an NWDAF or an AF. The consumer of NWDAF containing AnLF is believed to be an analytics consumer.

It should be preferred to minimize the impact to analytics consumers and NWDAF containing AnLF.

Observation #1: To keep the existing relations consumer of analytics shall request Analytics from an NWDAF containing AnLF. An NWDAF wanting to retrieve a model shall request that from an NWDAF containing MTLF.

In VFL operations no model is possible to be retrieved from a VFL server. In training the final output is that the training is done. And the NWDAF containing AnLF need to request Analytics from the VFL server to be able to support the analytics consumer.

It should be preferred to minimize the impact to analytics consumers and NWADF containing AnLF. An NWDAF containing AnLF does not Discover nor select VFL server only NWDAF containing MTLF. Therefore, NWDAF containing AnLF shall be notified that no model is possible to retrieve, but rather it shall request analytics when training is done.

Observation #2: The NWDAF containing MTLF communicates to the NWDF containing AnLF that no model will be possible to retrieve, but rather it shall request analytics when training is done.

If an NWDAF containing MTLF realizes that it cannot produce the model (e.g. because it will not receive the QoE from an AF) it decides the AF to be the VFL server to produce the output for the Analytics ID. This is similar to when an NWDAF containing MTLF decides it cannot perform as HFL server but rather request another NWDAF containing MTLF to be HFL server, and adds a Notification target of the analytics consumer. An NWDAF containing MTLF can request VFL training on behalf of NWDAF containing AnLF and adds the Notification target.

If no Notification target is added the NWDAF containing MTLF must be in the loop for all communication between NWDAF containing AnLF and VFL server. And if no AF ID is communicated to the NWDAF containing AnLF, it must after training is done, request Analytics from the NWDAF containing MTLF. Which goes against the existing relations and responsibilities. The NWDAF containing MTLF can add the AF ID in the communication to the NWDAF containing MTLF.

Observation #3: If NWDAF containing MTLF decides that an AF shall be VFL server, it may add the Notification target of the NWDAF containing AnLF in the request to the AF. If it does not do so it needs to communicate the AF ID to the NWDAF containing AnLF. In both cases the AF is known by the AnLF.

After training is done, the AF is known by the NWDAF containing AnLF. It can the request Analytics from the AF.

Observation #4: For inference the NWDAF containing AnLF can request analytics from VFL server.

### 1.2 Item 3 - VFL server capability registration

Relevant ENs in existing TS or approved CRs:

Editor’s Note：Whether NWDAF as VFL server registers to NRF with VFL capability information is needed is FFS. (CR: 2411193 – vivo; Clause: 5.2)

Editor’s Note：Whether AF as VFL server registers to NRF with VFL capability information is needed is FFS. (CR: 2411193 – vivo; Clause: 5.X)

Editor’s Note：Whether a discovery of an AF acting as VFL server is required and how it can be done is FFS. (CR: 2411193 – vivo; Clause: 6.2H.2.1.2)

Editor’s note: whether to register and discover VFL server is FFS. (CR: 2410096 – ICS; Clause: 6.3.13, TS 23.501)

The key points to be discussed: (depends on the issue 4, can be discussed together)

- Whether and which info should be registered to NRF by the VFL server, e.g. only supported analytics ID like other normal AnLF, or VFL capability info of VFL server?

In the discussion for item 4 it is obvious that an NWDAF containing MTLF must be able to discover and select a VFL server for training when AF is server. So the last three ENs can be resolved.

Observation #5: An AF VFL server registers VFL capability per Analytics ID.

For the first EN, whether the NWDAF must register VFL server capability is not obvious from the discussion for item 4.

First question to be asked is if e.g. an NWDAF containing MTLF or an AF shall be able to trigger VFL training, where NWDAF is VFL server?

Second question is whether it is useful if either NWDAF or AF as VFL server register when it has trained a model for a specific Analytics ID?

This is to be decided and can be kept as an EN for this meeting.

# 3. Conclusions and Proposals

To be decided if the observations are correct. Also the updates of the mentioned CRs above need to be reviewed.

The updated documents are:

S2-2411641 - Refinements for VFL feature

S2-2411644 - General training procedure for Vertical Federated Learning between NWDAF(s) and AF(s)

S2-2411645 - Update the general inference procedure for vertical federated learning to resolve ENs