SA WG2 Meeting #166	S2-2411648

November 18 – 22, 2024, Orlando, FL, USA

Source:	Ericsson, Apple, Cablelabs, Google

Title:	Capability handling for MPQUIC-IP and MPQUIC-E

Document for:	Approval

Agenda Item:	19.13.2

Work Item / Release:	MASSS / Rel-19

Abstract of the contribution: This contribution discusses the capability negotiation for the new steering functionalities.

# Background

For IP PDU Sessions in rel-16 – rel-18, there is no mandatory Steering functionality, but MPTCP and MPQUIC are bundled with ATSSS-LL Active-Standby. When MPTCP and/or MPQUIC is supported, the PCF generates a lowest precedence catch-all rule that uses ATSSS-LL with Active Standby. The technical reason is that MPTCP and MPQUIC can only handle TCP and UDP traffic and there is thus a need to have a way to handle non-MPTCP and non-MPQUIC traffic. However, with MPQUIC-IP there is no such technical reason.

For Ethernet PDU Sessions in rel-16 – rel-18, only a single steering functionality (ATSSS-LL) has so far been defined and in our understanding, this is the reason it was categorized as mandatory. However, with the introduction MPQUIC-E, ATSSS-LL is no longer the only option.

Release 19 introduced new MPQUIC based steering functionalities for both IP and Ethernet PDU which can be used for both IP and Ethernet traffic. This is why in this Release ATSSS-LL is not any longer the only steering functionality which can be used to handle either the IP and/or Ethernet traffic in its entirety.

Observation 1: There is no technical reason anymore in rel-19 for mandating ATSSS-LL (for Ethernet) or bundling MPQUIC-E or MPQUIC-IP with ATSSS-LL Active-Standby.

# Discussion

##### Mandatory steering functionalities

It was discussed at SA2#165 whether and what steering functionalities that should be considered mandatory in rel-19.

Mandating steering functionality may simplify interoperability but it also creates complexity. A basic simple mandatory steering functionality (e.g. ATSSS-LL with Active Standby) could be nice as a lowest common denominator between UE and NW, but if a mandatory feature is not of interest for a certain use case, it just causes implementation complexity and cost. In those cases, it may be preferable to reject a MA PDU Session request (or downgrade a MA PDU Session request to SA PDU Session) if the desired steering functionality(ies) is not supported.

Standards cannot force what an operator deploys/provides in the network when multiple options are available. Mandating a steering functionality in the network does not make sense since it is anyway an operator decision to decide what features to purchase and enable in a deployed network.

Furthermore, mandating a specific steering functionality like ATSSS-LL across multiple Releases when alternatives are available does not simplify neither the handset design nor the network. Such a requirement introduces an unnecessary burden for both handset and network vendors as well as on operators which would like to deploy such service starting from this Release on.

So far ATSSS has not seen a significant market adoption. High baseline requirements on ATSSS functionality will likely not increase the chances for ATSSS to being supported in the mainstream.

Observation 2: The mandated steering functionality should be kept to a minimum. Putting burdens on UE and NW implementations and deployment will not help getting it deployed….

It has been claimed that rel-19 needs to mandate ATSSS-LL for Ethernet to be backwards compatible. However, there is no rel-18 network out there and no rel-18 capable UEs. UE vendors and operators may decide to go directly to rel-19 for ATSSS. It can also be noted that ATSSS as such is an optional feature. A customer with a rel-18 phone supporting ATSSS may buy a rel-19 phone that does not support ATSSS. There is no guarantee that a network supports ATSSS in first place and no expectancy today that a legacy UE need to access ATSSS services. Much more as we indicated earlier, mandating both UE and network to support both ATSSS-LL as well as MPQUIC -E when the operator would like to support Rel-19 features unnecessarily increases both the UE and network implementation as well as potentially reduces the network efficiency over RAN when both steering functionalities are in use.

Observation 3: The discussion about backwards compatibility for Ethernet is theoretical.

Another question is how to handle MPQUIC-UDP in case the UE also supports MPQUIC-IP. Currently, MPQUIC-UDP support is bundled with ATSSS-LL with Active-Standby. As mentioned above, the reason for this is to be able to carry non-UDP traffic. However, if MPQUIC-IP is supported, MPQUIC-IP can carry non-UDP traffic, and it would technically speaking be no need to support ATSSS-LL with Active-Standby. Similar logic applies in case MPTCP and MPQUIC-IP is supported, although such combination may be a less likely implementation.

Observation 4: In case MPQUIC-UDP and MPQUIC-IP are supported, there is no need to mandate the support for ATSSS-LL Active-Standby.

It has also been discussed that rel-19 needs to allow a fallback to ATSSS-LL Active-Standby for IP-based PDU Sessions, in case there is no common support of MPTCP, MPQUIC-UDP, MPQUIC-IP between UE and NW. To enable that, the MPQUIC-IP capability should be bundled with ATSSS-LL Active-Standby.

Such fallback is however not possible in rel-16-18 and the handling of mismatch cases between UE and NW capabilities have not been addressed in previous releases. I.e. it is not used as a fallback in rel-16-18, it is just used as a way to handle non-MPTCP/non-MPQUIC traffic. Furthermore, even if fallback is possible, it may not be desirable.

Observation 5: Using ATSSS-LL with Active-Standby as a fallback option could be possible, but the use case and benefit are not so clear. If there is a mismatch, it may be better to simply reject the MA PDU Session or accept the session as a single-access PDU Session.

# Proposal

In our view (for rel-19):

- 	ATSSS-LL with any steering mode should not be mandated for Ethernet PDU Sessions.

- 	MPQUIC-E and MPQUIC-IP should be defined as “standalone” capabilities.

- 	In case MPQUIC-UDP and MPQUIC-IP are supported, there is no need to bundle MPQUIC-UDP with ATSSS-LL with Active-Standby. Similarly, in case MPTCP and MPQUIC-IP are supported, there is no need to bundle MPTCP with ATSSS-LL Active-Standby.

- 	In addition to the MPQUIC-E and MPQUIC-IP “standalone” capabilities, an option could be to also define capabilities that bundles MPQUIC-E and MPQUIC-IP with ATSSS-LL Active-Standby.

Please see CRs in S2-2411649 and S2-2411650.