3GPP SA WG2 Meeting #166	S2-2411667

Orlando, USA, November 18 - 22, 2024													(was S2-240xxxx)

Source:	Ericsson

Title:	KI#3 Conclusion on NEF Exposure

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution proposes NEF exposure aspect in the Key Issue #3 conclusion for Ambient IoT.

1. Introduction

This contribution discuss the NEF exposure aspect in the Key Issue#3 conclusion. The NEF exposure aspect covers the AIoT services provided towards the AF, which is equivelant to the information provided by the AF which had been discussed but not concluded in SA2#165.

From SA1 requirement and also RAN SID, in Rel-19, SA WGs and RAN WGs focus on inventory and command use cases.

[Proposal-1] It is proposed to expose Inventory and Command services towards AF in NEF.

No matter for Inventory use case or for Command use case, the target devices need to be identified within the AIoT service request from the AF.

- For individual devices, the target devices are identified by the permanent AIoT Device Identifier

- For a group of devices, there are two options in TR 23.700-13 for the ID used:

  - Option-1: Group ID

  - Option-2: Device ID MASK information (including EPC with MASK).

Group ID requires additional information stored in NVM of the devices, besides the individual device ID. Device ID MASK information allows the network to reach a group of devices by masking some bits in the device ID, so that only the Device ID needs to be stored in the NVM of the devices. For type 1 devices, the less storage requirement would be more important. That is, Option-2 is more suitable for type 1 devices.

On the other hand, the group ID bring more flexibility than the MASK. Using the MASK, the group size needs to be pre-determined. For example, if the AF uses the last 4 bits for a group of devices in the planning phase. In Device ID MASK information, the last 4 bits are masked. But after running for a period, there need to be the 17th device to be deployed, the MASK adjustment (e.g., needs to mask the last 5 bits) of this group may cause conflicts (e.g., some unintended devices respond the A-IoT paging due to the new MASK). For flexibility consideration, Option-1 is more suitable for type 2 (and also higher) devices.

[Proposal-2] It is proposed to let AF provide target device information, which can be permanent AIoT Device Identifier with or without MASK information, or a group identifier.

For inventory, in most cases, AF provides target area information to request the readers within the area to perform the inventory operation towards the target AIoT devices. As a complemenatary use case, in topology 2, it is also possible that some UE readers request AF over-the-top, so that AF triggers inventory via those UE readers.

[Proposal-3] For inventory, it is proposed to let AF provide target area information or UE reader ID information in topology 2.

For inventory, the NEF may also provide other optional parameters to enable the flexible usage of AIoT services towards the AF:

- Periodicity information: the AF may request the network to perform the inventory periodically, so that it does not have to send request to trigger the inventory each time. When the periodic inventory is performed, the AF may further decide to inventory all target AIoT devices, or only those new AIoT devices.

- Aggregation required information: the AF may want the network to aggregate the response for efficient transportation. The AF may also want a fast response, so that the network should forward the received response without waiting for the responses from other devices.

- Location information required: the AF may request the location information of the devices. It is up to RAN3 to determine how the location information is provided.

[Proposal-4] For inventory, it is proposed to expose the optional parameters about periodicity information, aggregation required information and location information required.

For command, AF should also provide target area information to request the readers within the area to perform the command operation towards the target AIoT devices. It is also required to provide the command (i.e., read, write, enable, disable) with the proper parameters.

[Proposal-5] For command, it is proposed to let AF provide target area information and the command information.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.1.0:

* * * Start of Change  * * * *

### 8.3.3	NEF exposure

NEF supports to expose Ambient IoT services towards the AF. The Ambient IoT services include Inventory and Command services.

For Inventory service, Inventory service operation is provided:

-  The target device information is required. It can be a permanent AIoT Device Identifier with or without MASK information, or a group identifier.

-  The target area information or UE reader ID information is required. UE reader ID information applies to topology 2 only.

-  Periodicity information is optional. It indicates that AF requests 5GC to perform inventory periodically.

- Aggregation required information is optional. It indicates that the AF requests 5GC to aggregate the response before sending back to the AF.

- Location information required is optional. It indicates that AF requests 5GC to provide location information additionally.

NOTE:	It is up to RAN3 to determine how the location information is provided.

For Command service, Read, Write, Enable, Disable service operations are provided:

-  The target device information is required. It can be a permanent AIoT Device Identifier with or without MASK information, or a group identifier.

-  The target area information is required.

- The parameters for the corresponding command are required.

- Aggregation required information is optional. It indicates that the AF requests 5GC to aggregate the response before sending back to the AF.

* * * End of Change * * * *