3GPP SA WG2 Meeting #166	S2-2411668

Orlando, USA, November 18 - 22, 2024													(was S2-240xxxx)

Source:	Ericsson

Title:	KI#3 Conclusion Update on Procedure Principles

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution proposes the procedure principles clause in KI#3 conclusion.

1. Introduction

In the interim conclusion of KI#3, there is clause 8.3.2 “AIoT services supported by the 5GC” and clause 8.3.3 “NEF exposure”. Those two clauses describe the service operations supported by 5GC and exposed by the NEF. To describe the procedures for inventory and command, therefore, a new clause is needed.

[Proposal-1] Introduce a new clause to describe procedure principles.

The procedure principles depend on the solution architecture which is to be concluded in KI#1, which decides how CN deliver the relevant information towards BS readers and UE readers.

[Proposal-2] Capture such dependency with an EN.

In both inventory and command procedures, after the AIOTF receives inventory request from the AF (optionally via NEF), it needs to distribute the request towards the proper readers.

-	In topology 1, it needs to distribute towards proper A-IoT RAN nodes, and it is A-IoT RAN node which decides the BS readers.

-	In topology 2, there are two different use cases in the inventory:

- 	As a main use case, the AF provides the area information towards the AIOTF: In this use case, The AIOTF needs to request AMFs to page CM-IDLE UEs, to ensure they are reachable and have UE contexts in the gNB.

Option-1: The AIOTF can distribute towards proper A-IoT-enabled gNBs based on area information, and gNBs can perform the UE reader selection, which aligns with the solution in topology 1. If the selection needs to be performed based on reader information (e.g., fixed UE readers), such information can be provided to gNB as a part of UE context information.

Option-2: The AIOTF makes a selection based on certain criterias to generate the candidate UE reader list. The AIOTF request the gNB to make a final decision to get the determined UE reader list.

- 	As a complementary use case, the AF provides the UE reader IDs towards the AIOTF: The AIOTF sends the UE reader IDs to the gNB, and gNB will use those UE reader IDs. There is no reader selection in this use case.

The A-IoT RAN/A-IoT-enabled gNB are deeply involved in the reader selection procedure. It is also related with how AIoT session resource setup information is provided to the gNB, which depends on RAN3 as well. It requires a cooperation with RAN3 on how the readers are selected.

[Proposal-3] It is proposed to capture the RAN3 dependency in an EN.

Inventory Procedure:

The A-IoT paging can target to one or more individual devices or one or more group of devices.

For the individual devices, it is up to SA3 to determine which kind of individual device identification can be used to address existing security/privacy requirements.

For A-IoT paging for a group of devices, there are two options in TR 23.700-13 for the ID used:

- Option-1: Group ID

- Option-2: Device ID MASK information (including EPC with MASK).

Group ID requires additional information stored in NVM of the devices, besides the individual device ID. Device ID MASK information allows the network to reach a group of devices by masking some bits in the device ID, so that only the Device ID needs to be stored in the NVM of the devices. For type 1 devices, the less storage requirement would be more important. That is, Option-2 is recommended for type 1 devices.

On the other hand, the group ID bring more flexibility than the MASK. Using the MASK, the group size needs to be pre-determined. For example, if the AF uses the last 4 bits for a group of devices in the planning phase. In Device ID MASK information, the last 4 bits are masked. But after running for a period, there need to be the 17th device to be deployed, the MASK adjustment (e.g., needs to mask the last 5 bits) of this group may cause conflicts (e.g., some unintended devices respond the A-IoT paging due to the new MASK). For flexibility consideration, Option-1 is recommended for type 2 devices.

And for both options, it is also up to SA3 to determine which kind of group identificaiton can address existing security/privay requirments.

[Proposal-4] It is proposed to capture the SA3 dependency in the EN for A-IoT paging

In inventory procedure, after the reader triggers A-IoT paging, the devices need to respond their device IDs or truncated device IDs, if they are within the group or match the device MASK information. Following the existing solutions in 5GS, the device ID needs to be concealed when there’s no mutual authentication between the network and the device. Are Ambient IoT devices able to perform the concealment, especially for type 1 devices? If not, is there a secured way to report the device ID towards the network? It is up to SA3 to determine.

[Proposal-5] It is proposed to capture the SA3 dependency in the EN for device ID response.

Command Procedure:

For command procedure, AF request network to deliver the command to one or multiple devices (in an area).

As discussed above, if the device can be registered to the network, the temporary ID can be used to reach the device and have the command delivery in “command-only” manner. There is no specific security/privacy conern.

Otherwise, the device ID needs to be used  to reach the device and have the command delivery in “inventory-command” manner. In this case, as discussed in Inventory Procedure, it is also up to SA3 to determine which kind of individual or group identificaiton can address existing security/privay requirments.

[Proposal-6] It is proposed to capture the SA3 dependency in the EN for A-IoT paging.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.1.0:

* * * Start of Change (ALL TEXTS ARE NEW) * * * *

## 8.3.X	Procedure Principles

Editor's note:	The procedure principles depends on the KI#1 architecture conclusions.

The following procedure aspects and principles for Ambient IoT services are agreed for the conclusion on the support of Ambient IoT Services, which apply to both topology 1 and topology 2:

-	The AF sends inventory or command request towards the AIOTF, optionally via NEF when the AF is outside trusted domain.

-	The AIOTF generates the AIoT device NAS message.

- 	The AIOTF sends the AIoT device NAS message, accompanied by the assistant information to AIoT RAN and readers.

Editor’s note: Terminology needs to be aligned with RAN3: in case of topology 1, “AIoT RAN” may serve one or several readers (“BS readers” in current SA2 terminology), in case of topology 2, “AIoT RAN” is an “AIoT enabled gNB” serves one or several “UE Readers”.

Editor's note:	It depends on KI#1 architecture conclusions to decide how AIOTF sends the AIoT device NAS message and the assistant information towards AIoT RAN.

Editor’s note: For topology 2, it is FFS and also depends on RAN3 on how AIoT session resource control information is  provided to A-IoT-enabled gNB.

Editor's note:	It requires cooperation with RAN3 on how AIoT RAN and the readers served by AIoT RAN are selected in topology 1 and topology 2, dependent on whether and how information is provided controlling the area within which an AIoT procedure shall be executed.

For Inventory:

-	A-IoT paging as triggered towards AIoT devices containing target device information. The target device information contains either one or several individual AIoT Device Identifiers or or one or several AIoT Device Group Identifiers.

Editor's note:	It is up to SA3 to determine which kind of individual / group identification are able to address existing security/privacy requirements.

-	Each AIoT device eligible to respond provide a device IDs within a response to the A-IoT paging request.

Editor's note:	It is up to SA3 to determine which kind of device ID contained in the response is able to address security/privacy requirements.

-	The AIOTF authenticates the device and allocates temporary ID in the AIoT NAS layer for the device.

Editor's note:	It is up to SA3 to determine whether the temporary ID in the AIoT NAS layer is necessary to address security/privacy requirements.

-	The AIoT RAN and the AIOTF may aggregate the responses from several devices; aggregation may be dependent on whether aggregation required information is provided by the AF.

-	The AIOTF sends the device ID(s) towards the AF, optionally via the NEF. Whether the AIOTF includes additionally AIoT device location information depends e.g. on the location information required from the AF.

NOTE:	It is up to RAN3 to determine how and which AIoT device location information is provided.

-	Whether the AIOTF triggers periodical Inventory depends on respective information provided by the AF but could also depend on conditions considered by the AIOTF itself.

For Command:

-	The Command procedure is performed in an "Inventory and Command" manner or in an (optimized) "Command-only" manner.  In “Inventory and Command”, the A-IoT paging is triggered towards either one or several individual AIoT Device Identifiers or one or several AIoT Device Group Identifiers. In “Command-only”, the A-IoT paging is triggered towards individual AIoT device using temporary ID in the AIoT NAS layer along with the AIoT device NAS message containing Command information.

Editor's note:	It is up to SA3 to determine, in “Inventory and Command”, which kind of individual / group identification are able to address are able to address existing security/privacy requirements.

Editor's note:	It is up to SA3 to determine whether the temporary ID in the AIoT NAS layer is necessary to address security/privacy requirements.

-	The AIoT device eligible to execute the command responds the result towards the AIOTF.

-	The AIoT RAN and the AIOTF may aggregate the responses from several devices;aggregation may be dependents on whether aggregation required information is provided by the AF.

-	The AIOTF sends the command results towards the AF, optionally via the NEF.

Editor's note:	Additional conclusions are FFS.

* * * End of Change * * * *