3GPP SA-WG2 Meeting #166	S2-2411695

# Orlando, USA 18 – 22nd November, 2024	  		 								(revision of S2-240)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5836 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Usage of QUIC-Aware-proxying for Handling of end-to-end encrypted XR flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA plenary approved conclusions in 23.700-70 clause 8.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a high level description of the QUIC-Aware-proxying mechanism that may be used to support Media Related Information detection by UPF when XR traffic is encrypted between UE and AS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | conclusions in 23.700-70 clause 8.2 are not supported by normative work |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR should be implemented after CR 5728 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE ALL text is new

##### 5.37.X.Y  Usage of connect-UDP in-band signaling protocol coupled with QUIC-Aware-proxying

Editor’s Note: the whole content of this clause is FFS and depends on IETF feedback

When connect-UDP in-band signaling protocol is used for XRM traffic using HTTP/3 QUIC transport, double encryption of the end-to-end traffic payloads on the connection between UPF and AS proxy may be avoided using QUIC aware proxying with HTTP defined in [xx2].

NOTE 1:	The double encryption stems from the end-to-end traffic being encrypted once via QUIC between UE and AS proxy and another time in connect-UDP related encapsulation / tunnel between the UPF and the AS proxy.

Clause 5.37.X.Y defines the usage of connect-UDP in-band signaling protocol coupled with QUIC-Aware-proxying and documents only the specific mechanisms to be supported then on top of those defined in clause 5.37.X.2.

By default, UPF and AS proxy use tunnelling mode to exchange XRM traffic. The UPF and AS proxy may use forwarded mode as defined in [xx3] when the end-to-end traffic flow is based on QUIC, if they both support this feature, and they have negotiated Virtual Connection ID using RFC 9297 [172] as defined in QUIC aware proxying with HTTP [xx2].

During the QUIC connection establishment between the UE and the AS proxy, the UPF may learn the end-to-end QUIC Connection IDs that are used in the end-to-end flows from UE to AS and those used in the flows from AS to UE. Based on the learnt Connection ID(s), the UPF may negotiate with AS proxy per [xx3] to get Virtual Connection IDs (VCID(s)) corresponding to the end-to-end Connection IDs. These virtual CIDs may be used by UPF and AS to support the forwarded mode.

When forwarded mode is active and possible for UL direction, the UPF sends UL QUIC packet towards the AS proxy using the Virtual Connection IDs negotiated via QUIC aware proxying with HTTP [xx3]; such UL traffic contains only the UDP payload received from the UE.

When forwarded mode is active and possible for DL direction, Media Related Information is included by the AS into the forwarded UDP payload packets via a packet transform defined by 3GPP. The UPF uses a 3GPP defined packet transformer [clause 5.3 of xx3] to retrieve the Media Related Information and the content of the original end-to-end QUIC packet.

During the negotiation between UPF and AS proxy to activate the forwarded mode, the UPF and the AS proxy need to agree on the 3GPP packet transform to apply in the DL direction.

NOTE 1: 	Since the connect-UDP client (in the UPF) is separate from the end-to-end QUIC client (in the UE) it cannot coordinate as tightly with the client QUIC stack as is assumed by the “QUIC-Aware Proxying Using HTTP” draft [xx2]. The connect-UDP client in the UPF can extract CIDs from e2e QUIC Initial packets and uses these values in REGISTER_CLIENT_CID and REGISTER_TARGET_CID capsules defined in [xx2]. Prior to forwarding QUIC packets with short header, the connect-UDP client (UL) and the proxy (DL) check whether the destination CID is identical to an already registered CIDs, if not, the packets are sent in tunneled-mode.

NOTE 2: The 3GPP packet transform is defined in stage-3 specifications.

NEXT CHANGE (2)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES