3GPP TSG-SA2 Meeting #166	S2-2411731

# Orlando, US, November 18 - 22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1280 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: VFL inference procedure when untrusted AF is acting as VFL server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - This paper adds inference procedure of Vertical Federated Learning when untrusted AF is acting as VFL server |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding new clause for inference procedure of Vertical Federated Learning when AF is acting as VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing documentation for VFL procedures with AF as server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.4.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.2H.2.4.2	Inference Procedure for Vertical Federated Learning when untrusted AF is acting as VFL Server

Figure 6.2H.2.4.2-1: Inference Procedure for Vertical Federated Learning when untrusted AF is acting as VFL Server.

1-2.	The inference procedure may be triggered by internal service logic of the AF acting as VFL server, or the consumer NF sends an subscribe event (Analytics ID, Target of Analytics Reporting= e.g., UE IDs) to the untrusted AF via NEF.

Editor's note:	Whether and how a 5GC NF can send a request to trigger the VFL server AF to perform inference is FFS. Whether the AF event exposure service can be reused is FFS

3.	Based on the information received in step 2 or internal service logic of AF, VFL server decides to initiate the VFL inference procedure with the VFL clients. VFL server selects client(s) using information stored in the VFL training process. The server may select some or no clients, e.g., depending on their contribution to the training result and the current status of the client.

For each VFL client, the VFL server NWDAF sends a Nnef_MLModelInference_Subscribe or Nnef_MLModelInferenceInfo_Request to NEF including the UE IDs, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used, and the temporary NWDAF ID obtained in the discovery procedure (see Clause 6.2H.2.1.1).

Editor's note:	It is FFS additional parameters are needed to send from the VFL server to VFL client.

Editor's note:	It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying.

4.	The NEF looks up stored information about VFL client during in the discovery procedure (see Clause 6.2H.2.1.1) and forwards the request to the VFL client. If the message in step 3 contains external identifiers, the NEF maps them to internal identifiers.

5.	Same as the step 3 defined in clause 6.2H.2.4.1.

6.	Same as the step 4 defined in clause 6.2H.2.4.1.

7-8.	Same as the step 5 defined in clause 6.2H.2.4.1 with following differences:

Each VFL Client sends the client intermediate local inference results to the VFL server via NEF.

9.	Same as the step 6 defined in clause 6.2H.2.4.1.

10-11.	If the inference was triggered by a request or subscription for information from a 5GC entity in step 1, the AF may report results derived by the inference process to the 5GC entity via NEF.

* * * * End of Changes * * * *