SA WG2 Meeting #S2-166	S2-2411825

18 - 22 November, 2024, Orlando, USA	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0624 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE's user plane availability status change for LCS-UPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT1 sends the LS (S2-2411289/C1-246023) to ask the scenario: The cause value "user plane not available" was introduced for UE to reject the network initiated user plane connection establishment procedure when the user plane is not available or release the established user plane connection when the user plane availability is changed to “not available”. However, currently there is no dedicated procedure specified to inform the LMF when the UE's user plane availability changes from "unavailable" to "available" It propoes to add a new clasue to clarify the User plane available notification to LMF. When the User plane is not available, the UE has sent the "User plane not available" to LMF in 24.572. When the User plane is available again, the UE sends the notification to LMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new clause to clarify, when the User plane is available, the UE sends the notification to LMF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how the handle the scenario, when the User plane is available again. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 6.18.X	User Plane Status Change notification to LMF

The figure 6.18.X-1 shows how a UE notifies the LMF User Plane Available when the UE user plane status changes from "user plane not available" to "user plane available".

Figure 6.18.X-1:  by UE

1.	UE has informed the LMF user plan not available, e.g. by rejecting the reject the network initiated user plane connection establishment procedure, or  releasing the established user plane connection when the user plane availability is changed to "not available" as defined in TS 24.572 [48].

2.	The UE user plane availability is changed from "not available" to "available".

3.	The UE sends "user plane available" to LMF through AMF to indicate user plane connection is available again.

4.	AMF sends the User Plane status received in step 3 to the LMF via Namf_N1messageNotify service.

5.	The LMF decides whether to use the positioning procedure via a user plane connection between UE and LMF and perform subsequent procedure as defined in the clause 6.18.1.

END OF CHANGES