3GPP TSG-SA2 Meeting #166	S2-2411832

# 18th – 22nd November 2024, Orlando FL. USA				             (revision of S2-2411193)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1198 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Refinements for VFL feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Vivo, ICS, LG Electronics, InterDigital, ZTE,] OPPO, [Nokia, Samsung] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Federated Learning are composed of Horizontal Federated Learning and Vertical Federated Learning. Horizontal Federated Learning have been supported in R18. In order to avoid any confusion, the majority agree to use the terminology Vertical Federated Learning e.g. feature naming, feature descrption,VFL model ID and capability indiation. One EN is capture. i.e Whether the VFL Interoperability information (e.g. type of supported VFL training method(s), vendor information, and feature related information) needs to be part of the NF profile registered in NRF is FFS. Vender information is needed. Different venders need to be interoperable between each other to understand the meaning of the VFL interoperatbilbiy information. VFL interoperatbilbiy information should include a list of Vender IDs to indicate which venders can support the interoperability to perform the VFL. Feature Information is needed. Considering the feature is sensitive information, the feature ID could be used to represent the feature info supported by different entities. Feature ID is a vender specific information and just reflects the feature type. (e.g., features from the different data sources.) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the existing teminology from FL to HFL; - Refine the HFL capability information; - Refine the VFL capablity information for NWDAF - Add VFL capablity information for AF; - Add VFL capability information registration for untrused AF via NEF. - Complete the general discovery procedure for VFL process with the case that the AF as the VFL server. - Vender ID and featuer ID are added as part of the VFL interoperability inforamtion and will be register to the NRF as the NF profile. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Result in confusing and inconsistency regading Horizontal Federated Learning and Vertical Federated Learning; The VFL registration and discovery procedure is not clear and not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.X(new), 6.2H, 6.2H.1, 6.2H.2, 6.2H.2.1, 6.2H.2.1.1 (new), 6.2H.2.1.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

-	If the analytics are related to UE(s) and if NWDAF instances indicate weights for TAIs in their NF profile (see clause 6.3.13 of TS 23.501 [2]), the NWDAF service consumer may use the weights for TAIs to decide which NWDAF to select.

-	If the NWDAF service consumer needs to discover an NWDAF containing an AnLF with analytics accuracy checking capability, the consumer may query NRF providing also the analytics accuracy checking capability in the discovery request.

-	If the NWDAF service consumer needs to discover an NWDAF containing AnLF which can use the Model provided by the specific NWDAF containing MTLF (e.g., in case of analytics context transfer), the consumer should discover the NWDAF(s) whose vendor ID is in the ML Model Interoperability indicator of the NWDAF containing MTLF.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types or to collect data from particular NWDAF Serving Area, the consumer may query NRF providing the NF Set IDs or NF types or Area of Interest in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources or NWDAF Serving Area information as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	When one or more trained ML Models are available for one or more Analytics ID(s) the NWDAF containing MTLF shall include the Analytics ID(s) that is(are) supported per service in the registration towards NRF. The NWDAF containing MTLF may wait to register in NRF the above services until at least one trained model is available. The NWDAF containing MTLF may provide to the NRF a list of Analytics IDs corresponding to the trained ML Models and possibly the ML Model Filter Information for the trained ML Model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML Model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. If the NWDAF containing MTLF supports ML Model interoperability, the NWDAF containing MTLF includes, in the registration to the NRF, an ML Model Interoperability indicator for each Analytics ID.

Editor's note:	How to indicate supporting model training for the LMF-based AI/ML positioning as defined in TS 23.273 [39] by the MTLF to NRF, e.g., using a specific analytics ID or a new indication is FFS.

Editor's note:	For model training for LMF-based AI/ML positioning, whether Positioning case information (e.g. Uplink/downlink, or case 2b, 3b) can be included in NF profile as an optional parameter is FFS.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML Models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML Models requested by the NWDAFs from the vendors in the list.

NOTE 4:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (e.g. an NWDAF containing AnLF, an NWDAF containing MTLF as FL server or FL client) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required and Vendor ID. The NRF returns one or more candidate instances of NWDAF containing MTLF to the NF consumer and each candidate instance of NWDAF containing MTLF includes the Analytics ID(s), possibly the ML Model Filter Information for the available trained ML Models and ML Model Interoperability indicator, if available.

-	If the NWDAF service consumer needs to discover an NWDAF containing an MTLF with ML Model accuracy checking capability, the consumer may query NRF also providing the ML Model accuracy checking capability in the discovery request.

In order to discover an NWDAF containing MTLF with Horizontal Federated Learning (HFL) capability via NRF, in addition to the procedures described above for discovering NWDAF containing MTLF:

-	An NWDAF containing MTLF supporting HFL as a server shall additionally include FL capability type (i.e. HFL server), and may include Time interval supporting HFL as HFL capability information during the registration in NRF.

-	An NWDAF containing MTLF supporting HFL as a client shall additionally include FL capability type (i.e. HFL client), and may include Time interval supporting HFL as HFL capability information during the registration in NRF, and it may also include, NF type(s) and NWDAF Serving Area information and/or NF set ID(s) of the data source(s) where data can be collected as input for local model training.

NOTE 5:	An NWDAF containing MTLF may indicate to support both HFL server and HFL client in the HFL capability for specific Analytics ID. The FL capability type only applies for HFL but continues to be termed FL capability type to maintain backward compatibility with previous releases where only HFL was supported but the term FL capability type was used

-	During the discovery of NWDAF containing MTLF as HFL server, a consumer (e.g. a NWDAF containing MTLF) may include in the request the FL capability type as HFL server and may include Time Period of Interest and ML Model Filter information for the trained ML Model(s) per Analytics ID(s), if available. The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

-	During the discovery of NWDAF containing MTLF as HFL client, a consumer (e.g. an HFL server) may include in the request FL capability type as FL client and may include Time Period of Interest, a list of NF type(s) and/or NF set ID(s) of the data source(s). The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

NOTE 6:	The service consumer to discover an NWDAF containing MTLF with HFL capability is limited to NWDAF containing MTLF in this Release.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In the roaming architecture, the NWDAF with roaming exchange capability (RE-NWDAF) to request analytics or input data is discovered via the NRF. A consumer in the same PLMN as the RE-NWDAF discovers the RE-NWDAF(s) by querying for NWDAF(s) where the roaming exchange capability is indicated in its (their) NF profile. A consumer in a peer PLMN (i.e. RE-NWDAF) discovers the RE-NWDAF(s) by querying for NWDAF(s) in the target PLMN that is (are) supporting the specific services defined for roaming. A RE-NWDAF discovers the RE-NWDAF(s) in a different PLMN (i.e. HPLMN or VPLMN) using the procedure defined in clause 4.17.5 (if delegated discovery is not used) or clause 4.17.10 (if delegated discovery is used) of TS 23.502 [3], where the detailed parameters are determined based on the analytics request or subscription from the consumer 5GC NF, operator policy, user consent and/or local configuration.

In order to support VFL training and inference, the NWDAF shall include its VFL capability information per supported Analytics ID during registering to NRF. The VFL capability information includes VFL capability type (i.e. VFL server or VFL client or both) and Time interval supporting VFL if available. The NWDAF may also include the VFL Interoperability information (e.g. vendor ID(s) that the NWDAF can interoperate with and feature ID(s)) per supported Analytics ID during registering to NRF. To discover NWDAF supporting VFL from NRF, the consumer should consider the VFL capability information and the VFL Interoperability information, the detailed procedure and parameters of NWDAF registration and discovery for VFL are as defined in clause 6.2H.2.1.

Editor’s Note：Whether NWDAF as VFL server registers to NRF with VFL capability information is needed is FFS.

Editor’s note: It is FFS whether and how to enhance the discovery of VFL client that can do VFL with the VFL server.

Editor's note:	Details of ML Model handling, supported features, Feature alignment are FFS.

* * *Next Change * * *

5.X	AF Discovery and Selection for VFL

The AF discovery and selection is defined in the clause 6.3.25, TS 23.501 [2]. In addition to support VFL training and inference, the following factors may be considered for AF discovery and selection:

-	VFL capability information per supported AnalyticsID, which includes:

-	VFL capability type (i.e. VFL client)

-	optional Time interval supporting VFL.

-	optional VFL Interoperability information per supported Analytics ID, which includes:

-	vender ID(s) indicate the AF can interoperate with

-	feature ID(s) inciate the feature type (e.g., feature information from which data source.)

A trusted AF registers the above factor in NRF, for an untrusted AF the NEF is configured via OAM to register this factor into NRF.

For the discovery of a trusted AF or untrusted AF, the consumer may select an AF instance considering the above factor. The detailed procedure and parameters of AF registration and discovery for VFL are as defined in clause 6.2H.2.1.

Editor’s Note：Whether AF as VFL server registers to NRF with VFL capability information is needed is FFS.

* * *Next Change * * *

6.2H	Vertical Federated Learning among NWDAFs and AFs

6.2H.1	General

This clause specifies

procedures for Vertical Federated learning where AFs and NWDAF can can either act as VFL servers or VFL clients. Procedures for registration and discovery, for VFL training preparation, for VFL training, and for VFL inference are covered.

6.2H.2	Procedures

6.2H.2.1	Registration and Discovery procedure for Vertical Federated Learning

6.2H.2.1.1	Registration and Discovery procedure for Vertical Federated Learning when NWDAF is acting as the VFL server

Figure 6.2H.2.1.1-1: Registration and Discovery procedure for Vertical Federated Learning when NWDAF is acting as VFL server and NWDAF(s) and/or AF(s) are the VFL clients

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is NWDAF.

1a. 	VFL Server NWDAF registers to NRF with its NF profile, which includes NF Type (i.e. NWDAF type), Analytics ID(s), service area if available  VFL capability information per analytics ID and VFL Interoperability information per Analytics ID. The VFL capability information includes VFL capability type (i.e. VFL server) and optional Time interval supporting VFL. VFL Interoperability information includes vender ID(s) and feature ID(s).

1b. NWDAF as VFL client registers to NRF with its NF profile, which includes NF Type (i.e. NWDAF type), analytics ID(s), service area if available ,VFL capability information per analytics ID and VFL Interoperability information per Analytics ID. The VFL capability information includes VFL capability type (i.e. VFL client).  and optional Time interval supporting VFL. VFL Interoperability information includes vender ID(s) and feature ID(s).

1c. When untrusted AF as VFL client, it shall register to the NEF via OAM configuration: Analyics ID(s), its VFL capability information per supported analytics ID and VFL Interoperability information per Analytics ID. The AF’s VFL capability information includes VFL capability type (i.e. VFL client) and optional Time interval supporting VFL. VFL Interoperability information includes vender ID(s) and feature ID(s). Then NEF updates NEF profile to NRF including associated AF ID , AF’s VFL capability information and AF’s VFL Interoperability information.

1d. When trusted AF as VFL client, it registers to NRF with its NF profile, which includes NF Type (i.e. AF type), analytics ID(s), service area if available, VFL capability information per analytics ID and VFL Interoperability information. The VFL capability information includes VFL capability type (i.e. VFL client) and optional Time interval supporting VFL. VFL Interoperability information includes vender ID(s) and feature ID(s).

2. The NRF receives the registrations from VFL server and VFL client(s), and stores their NF profile.

3. The NRF sends registration response to VFL server and VFL client(s).

Steps 4 to 6 are the NWDAF and AF Discovery procedures when the VFL server is NWDAF.

4-6.	NWDAF as the VFL server determines that the ML Model requires VFL based on operator policy, Analytics ID, and Service Area.

NOTE:	Step 4 in Figure 6.2H.2.1-1 may be triggered at the VFL Server NWDAF by VFL server NWDAF itself or a request from a consumer.

If the NWDAF can not perform as VFL Server, it first discovers and selects another VFL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytics ID, VFL capability type as VFL server, VFL Interoperability information, Time Period of Interest, and optional Service Area.

Once the VFL Server NWDAF is determined, the VFL Server NWDAF discovers other NWDAF(s) and/or AF(s) as VFL Client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: NF type (i.e. NWDAF type, AF type, or NEF type), Analytics ID, VFL capability type (i.e.VFL client), VFL Interoperability information, Time Period of Interest and and optional Service Area.

6.2H.2.1.2	Registration and Discovery procedure for Vertical Federated Learning when AF is acting as the VFL server

The procedure below shows registration and discovery for VFL training and inference when the AF is the AF server, the AF can be trusted or untrused one. There can be multiple NWDAFs as VFL clients.

Editor´s note: The following is FFS: a deployment scenario that can also be agreed between operator and application service provider is when untrusted AF is the VFL server and only one NWDAF will be the VFL client. In such case the VFL Server can be configured to go directly to preparation procedure in clause 6.2H.2.2.

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is AF.

Editor’s Note：Whether a discovery of an AF acting as VFL server is required and how it can be done is FFS.

- Same as the step 1b, in clause 6.2H.2.1, NWDAF as VFL client registers to NRF with its NF profile. The NWDAF registers with the analytics IDs it supports.

NOTE x: The AF can use unstandardized values of the analytics ID.  The unstandardized values can be used by the AF as the VFL server to intiate the VFL training VFL inference and need to be supported by NWDAFs acting as VFL clients. It is the operator´s responsibility to guarantee that unstndardized analytics ID values within a PLMN are unique.

2-3. Same as the steps 2-3, in clause 6.2H.2.1.

Steps 4-10 are the NWDAF Discovery procedures when the VFL server is an untrusted AF.

4. If untrusted AF acting as the VFL server, it determines that VFL operations are required and the NWDAF(s) as VFL client(s) are required. The AF sends a discovery request for the VFL client(s) to the NEF and provides selection criteria: analytics ID, required NF type (i.e. NWDAF type), VFL capability type (i.e. FL client), VFL Interoperability information, Time Period of Interest and optional Service Area.

Editor's note:	Whether additional selection criteria are required is FFS (e.g. vendor information and feature related information).

5. The NEF checks based on configured policies whether the AF is entitled to request a VFL client for the analytics ID.

6-7. The NEF discovers VFL client (i.e. NWDAF) on behalf of the AF from the NRF by invoking the Nnrf_NFDiscovery_Request using the selection criteria provided by the AF as defined in step 4.

Editor’s note: It is FFS, whether NEF should perform VFL NWDAF selection (or shortlisting) or it should only discover NWDAF client candidates without any further selection.

8. 	The NEF selects an NWDAFs capable as acting as VFL clients and matching the received selection criteria. The NEF anonymizes NWDAF instances ID(s) and assigns temporary NWDAF ID(s) for each selected NWDAF intance as VFL client. The NEF stores the temporary NWDAF ID(s) together with information how to reach the NWDAFs

9. The NEF sends the temporary NWDAF ID(s) to the untrusted AF.

10. The AF stores the received temporary VFL client ID and uses it subsequent interactions with the NEF to indicate the target VFL client.

Editor's note:	Which services are used in step 4 and step 9 are FFS.

Editor's note: How to subsequently handles the Temporary NWDAF ID (e.g. removing and possibly notifying the update of Temporary NWDAF IDs) is FFS.

Steps 11-12 are the NWDAF Discovery procedures when the VFL server is an trusted AF.

11. If the AF as the VFL server is a trusted one, it discovers NWDAF(s) as VFL Client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used by the trusted AF: Analytics ID, required NF type (i.e. NWDAF type), VFL capability type as VFL client, VFL Interoperability information, Time Period of Interest and Service Area.

Editor's note:	Whether additional selection criteria are required is FFS (e.g. vendor information and feature related information).

12. The NRF returns one or more NWDAF intances as VFL Client to the AF.

* * *End of Changes * * *