3GPP TSG-WG SA2 Meeting #166	S2-2411868

Orlando, US, 18 – 22 November, 2024	(revision of)

Source:	Samsung

Title:	Conclusions on AIoT KI#1 for Topology 1

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: This paper proposes conclusion principles on key issue 1 for topology 1 architecture.

1. Introduction/Discussion

During SA2#165, initial conclusions on AIoT Architecture to support Topology 1 are agreed.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

8.1.2	Architecture to Support Topology 1

The principles and aspects in this clause are agreed to support Topology 1:

-	The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1:

-	Communicate with BS Reader (i.e. AIoT RAN), via an AMF, to trigger e.g. inventory request and command request).

NOTE 1:	There is no assumption about whether the AIoT RAN (i.e. BS Reader) also has gNB functionality for NR-Uu or not.

Editor's note:	The relationship between the AIoT RAN and (other) gNB functionality is FFS.

NOTE 2:	It is assumed Ambient IoT services can be deployed independently from existing deployments.

-	The AIoTF connects with a BS Reader via an AMF. The N2 interface between the BS Reader and AMF supports Ambient IoT services including delivery of inventory/command messages.

NOTE 3:	If network isolation is required an AMF instance is deployed for supporting the AIOTF communication with the BS Reader.

-	NGAP between the BS Reader and the AMF is enhanced to support Ambient IoT Services.

NOTE 4:	The details of the enhancements will be concluded by RAN WG3.

-	The AMF shall be enhanced to support Services which are used by an AIoTF for Ambient IoT Operations. New SBI based services on the AMF (to be used by the AIoTF) are introduced in 5GC.

-	The AMF passes information between BS Readers (e.g. operation requests and responses) and the AIOTF.

Editor's note:	The AMF enhancements are FFS.

Editor's note:	Details of the Service (e.g. whether it is a new service, whether the existing Namf service is enhanced is FFS).

-	Figure 8.1.2-1 below shows the aspects related to Topology 1 (indirect path via AMF) reference architecture with other NFs removed.

Figure 8.1.2-1: Non-Roaming 5G System Architecture (indirect Path via AMF)

-	Figure 8.1.2-2 below shows the aspects related to Topology 1 (indirect  Path via AMF) in reference point representation with other NFs removed.

Figure 8.1.2-2: Non-Roaming 5G System Architecture in reference point representation (indirect Path via AMF)

-	Figure 8.1.2-3 below shows the aspects related to Topology 1 (indirect path via AMF) protocol stack between the BS Reader and AIoTF.

Figure 8.1.2-3: Example Protocol Between AIoTF and AIoT Device for Topology 1 (indirect Path via AMF)

Editor's note:	The details of the protocol stack are FFS.

NOTE X: The BS Reader may be preconfigured to select the appropriate AMF to forward UL messages.

# * * * * End of changes * * * *