SA WG2 Meeting #166	S2-2411896

Orlando, USA, November 18 – 22, 2024                  	(revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1516 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transition Between IMS Avatar and Audio/video Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on conclusion for KI of support of IMS Avatar communication in TR 23.700-13, transition between IMS Avatar and audio/video communication should be supported. It is proposed to add procedure for transition between IMS Avatar and audio/video communication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for transition between IMS Avatar and audio/video communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure for transition between IMS Avatar and audio/video communication is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.X.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AC.X.y	Procedure for Transition Between IMS Avatar and Audio/video Communication

Transition between IMS Avatar and audio/video communication is based on the IMS Avatar communication architecture as specified in clasue AC.X.1. The transition procedure includes procedure for transition from audio/video communication to IMS Avatar communication in clause AC.X.y.1 and procedure for transition from IMS Avatar communication to audio/video communication in clause AC.X.y.2.

AC.X.y.1 Procedure for Transition from Audio/video to IMS Avatar Communication

Figure AC.7.X.1-1 depicts a signalling procedure for transition from audio/video to IMS Avatar communication.

Figure AC.X.y.1-1: Procedure for Transition from Audio/video to IMS Avatar Communication

The steps in the procedure are as follows:

0.	UE A has audio/video communication with UE B.

1.	UE A exchanges audio/video media with UE B through MF or directly over RTP.

2.	UE A decides to switch the communication from audio/video to Avatar communication mode.

3.	UE A interacts with the IMS Core and UE B to negotiate the Avatar media and establish application data channel.

NOTE:	It is assumed both UEs have already downloaded the Avatar communication application.

4.	The Avatar representation is transferred from Base Avatar Repository (BAR) to MF.

Avatar representation can be specific for the user of UE A; it is pre-configured in BAR or uploaded by the user.

5.	UE A captures sensing data of the user e.g. facial expression data, body motion data.

6.	UE A sends Avatar animation data (i.e. sensing data) to MF through application data channel.

7.	MF renders Avatar representation received at step 4 based on the Avatar animation data received.

8.	MF sends rendered Avatar representation to UE B.

9.	UE B displays rendered Avatar representation to the user.

10.	UE B also sends its Avatar representation to UE A through MF; and the video exchange between two UEs may be stopped.

AC.X.y.2 Procedure for Transition from IMS Avatar to Audio/video Communication

Figure AC.X.y.2-1 depicts a signalling procedure for transition from IMS Avatar to audio/video communication.

Figure AC.X.y.2-1: Procedure for Transition from IMS Avatar to Audio/video Communication

The steps in the procedure are as follows:

0.	UE A has IMS Avatar communication with UE B.

1.	UE A exchanges Avatar media (i.e. Avatar representation) with UE B transferred by MF.

2.	UE A decides to switch the communication from IMS Avatar to audio/video communication mode.

3.	UE A negotiates with the IMS Core and UE B for exchanging video media between UE A and UE B.

4.	UE A exchanges video media with UE B through MF or directly over RTP.

The video media in step. 3 and 4 is the real video of users and is captured by the camera of UEs, rather than Avatar media.

5.	UE A negotiates with the IMS Core and UE B to stop exchanging Avatar media and may release application data channel.

END OF CHANGES