3GPP TSG-WG SA2 Meeting #166	S2-2411915

Orlando, US, 18th Nov – 22nd Nov, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on reporting for the available data rate

Document for:	Approval

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2 / Rel-19

Abstract: This document discusses the details for the reporting of the available data rate.

# 1. Introduction

In TR 23.700-70 the conclusion for KI#9 states:

-	For GBR QoS Flow, exposure of available data rate:

-	Based on the AF requests, the PCF instructs SMF to enable the available data rate report and exposure for a GBR QoS Flow.

-	Following the instruction of SMF:

-	The NG-RAN reports the available data rate to the UPF via the GTP-U header of UL packets and UPF further exposes, based on mechanisms defined in clause 5.8.2.17 of TS 23.501 [2], and the available data rate is reported to the AF directly or via the local NEF.

-	Available data rate refers to the data rate that RAN is able to provide within a duration. Such duration is based on RAN implementation.

NOTE 1:	It is recommended that the available data rate exposure is not reported very frequently.

NOTE 2:	How the RAN measures the available UL and/or DL data rate provided by the RAN node is to be refined based on the potential feedback from RAN WG3.

There are however no further details mentioned about what the available data rate refers to and how the reporting can be controlled.

# 2. Discussion

## 2.1 Situations in which reporting of available data rate is useful

As the available data rate refers to the data rate that NG-RAN is currently able to provide for a QoS Flow with GFBR, the available data rate should never be above the MFBR (as the NG-RAN is not required to provide more resources that what is required for the MFBR).

The available data rate can be between the GFBR and the MFBR for a QoS Flow with MFBR>GFBR. In such a situation, the reporting of the available data rate informs the AF that the data rate of the video service can be (temporarily) increased.

The available data rate could however also be below the GFBR for a QoS Flow for which Notification control without Alternative QoS Profiles is enabled (see 23.501 clause 5.7.2.4.1a). As Alternative QoS Profiles are not used, the AF does not have any information about which data rate the NG-RAN is currently able to provide for the QoS Flow when the notification about "GFBR can no longer be guaranteed" is received. In this situation, the reporting of the available data rate would inform the AF about whether the data rate of the video service should be decreased, and to which value, or not. And if the situation is changing and the NG-RAN is able to provide a different data rate (lower or higher), the AF would be informed again and can thus adjust the data rate of the video service accordingly.

Proposal: Since the AF as well as the NG-RAN would benefit from a service adaptation when the “available data rate” falls below the GFBR, it is proposed to also support the “available data rate” reporting in the scenario of Notification control without Alternative QoS Profile.

## 2.2 Control for reporting

Periodic reporting should not be useful for the application layer (as the “available data rate” should often be identical to the GFBR).

While it is clear that the SMF has to instruct the NG-RAN to perform the available data rate reporting, the details of the control for the reporting have not been discussed yet.

Regarding how to control the reporting with respect to activation/deactivation, the possible options are:

- Coupled with the scenario (i.e. MFBR>GFBR, Notification control without Alternative QoS Profile)

- Independent control

Proposal: An independent control should not be needed as the reporting is only useful in the described scenarios.

Once reporting is activated, the NG-RAN needs a condition about when to send a report and the possible options are:

- Whenever the “available data rate” is not equal to the GFBR

- Whenever a threshold is crossed

  - Absolute vs. relative value for threshold

  - One or multiple thresholds

Proposal: A threshold-based control with absolute values should be most beneficial for the AF as this would reflect the possibilities for service adaptation in the best way (i.e. the reports are only sent when they are useful for the AF).

Finally, it should be discussed how often a change in the “available data rate” is reported:

- Permanent packet marking

- Marking only the first packets after a change in the “available data rate”

Proposal: Marking of the first packets after a change in the “available data rate” should be sufficient.

There should be almost no performance degradation at NG-RAN based on the following considerations:

- We can assume that the NG-RAN is not changing the data rate for a QoS Flow very frequently (i.e. it should be typically stable for a couple of seconds, unless the conditions of the air interface of the UE get worse)

- Similar to the logic described in 23.501 clause 5.7.2.4.1a, the NG-RAN is expected to immediately report a change to a lower value for the available data rate and a change to a higher value with some hysteresis

# 3. Conclusion and proposal(s)

It is proposed to agree the above proposals.