3GPP TSG-WG SA2 Meeting #166	S2-2411917

# Orlando, USA, 18th Nov – 22nd Nov, 2024		(revision of S2-2410486)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1419 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of available data rate exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As concluded in FS_XRM_Ph2, the 5GS can support the exposure of available data rate for a GBR QoS Flow. This paper intends to add support of the available data rate exposure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds support of the available data rate exposure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality of available data rate exposure support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 5794 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.1.3.27.1	Exposure of network information

For support of real-time media codec/traffic adaptation to the network conditions, the AF may subscribe for exposure of 5GS network information.

The AF may provide the subscription to the following information (as described in clause 5.37.4 of TS 23.501 [2]): the UL and/or DL congestion level information, UL and/or DL packet delay, the round-trip delay over one or two service data flows, the UL and/or DL Available Guaranteed Bitrate for the service data flow, the UL and/or DL data rate for the target service data flow(s) or the QNC for a GBR QoS Flow using AF session with required QoS as described in clause 6.1.3.22. The PCF may generate a PCC rule with the QoS Monitoring policies for the above network information as described in clause 5.37.4 of TS 23.501 [2].

The AF may also provide the value for Averaging Window using AF session with required QoS as described in clause 6.1.3.22.

# * * * * End of changes * * * *