# 3GPP TSG-SA WG2 Meeting #166	S2-2411937

# 18 - 22 November, 2024, Orlando, USA

Title:	[Draft] Reply LS on Analytics Aggregation

Response to:	LS (C3-245520 / S2-2411291) on Analytics Aggregation

Release:	Rel-17

Work Item:	     eNA_Ph2

Source:	SA2

To:	CT3

Cc:

Contact Person:	Vivian Chong

E-mail Address:	vivian DOT chong AT vivo DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR#1290, CR#1291, CR#1292

1. Overall Description:

SA2 would like to thank CT3 for the LS C3-245520 / S2-2411291 on Analytics Aggregation. SA2 has discussed it, and SA2 would like to provide the following response and feedback:

Question 1: Is this Formatting and Processing instruction refer to the Formatting and Processing instructions applied for the analytics? Or refer to the instructions applied for the data that was used to generate the analytics?

For the Q1, SA2 agrees that the data formatting and processing instruction refer to the instructions applied for the data that was used to generate the analytics.

As defined in clause 5A.4 Data Formatting and Processing TS 23.288, the Formatting and/or Processing instructions are used by the data consumer in the data request service to indicate the DCCF/MFAF how to how to process the data and how to feedback data.

In the case of analytics metadata information provided from one NWDAF to another (e.g. NWDAF A to NWDAF B) in clause 6.1.3, the data formatting and processing instructions is one parameter in analytics metadata information from NWDAF A to NWDAF B, it indicates how the data received and processed by NWDAF A before the generation of analytics results, that is similar with the definition of 5A.4. The NWDAF B could decide how to aggregate the analytics outputs based on the Data formatting and processing instructions received from NWDAF A.

Question 2: If the Formatting and Processing instruction is provided in the analytics metadata information, this means that the formatting and processing instructions have been used. When formatting and processing instructions have been used, does that mean the NWDAF may provide the summary report of the analytics based on the formatting and processing instructions in the notification?

For the Q2, since the Data formatting and processing instructions are applied for the data used to generate analytics outputs, they are not applied for the analytics itself according to SA2 specification, so there is no reason for NWDAF to provide the summary report of the analytics based on the formatting and processing instructions.

2. Actions:

To CT3 group,

ACTION:  SA2 kindly asks CT3 group to take above information into consideration.

3. Date of Next SA2 Meeting:

SA2#166-Ad Hoc-e		January 20th – 24th, 2025			Online

TSG-SA2 Meeting #167		17th Feb – 21st Feb, 2025			Athens, GR