3GPP TSG SA WG2 #166	            S2-2411948

18 - 22 November, 2024, Orlando, USA 	(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1519 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#8: Procedures of avatar communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There’s no procedure description for avatar communication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new clause to describe procedures of avatar communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure of avatar communication is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.X.3 (new), AC.X.3.1 (new), AC.X.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text is new) * * * *

AC.X.3	Procedures

AC.X.3.1	Avatar Call Setup using P2P Application Data Channel

Figure AC.X.3.1-1 depicts a signalling procedure for avatar call setup using P2P application data channel.

The steps in the procedure are as follows:

1.	The UE#1 has been configured with Avatar Representations.

2.	The P2P application DC has been established between the UE#1 and the UE#2.

3.	The UE#1 selects an Avatar Representation locally and uses the Avatar Representation to render the media data with animation data, then sends the Avatar media towards the UE#2.

4.	The UE#1 receives the media data, and uses a selected Avatar Representation locally to render the media with animation data.

AC.X.3.2	Avatar Call Setup using P2A2P Application Data Channel

Figure AC.X.3.2-1 depicts a signalling procedure for avatar call setup using P2A2P application data channel.

The steps in the procedure are as follows:

1.	The UE#1 has not been configured with Avatar ID list.

2.	The bootstrap DC has been established between the UE and the DCSF. The DCSF has been configured for providing Avatar ID list for the UE (e.g., corresponding DC AS URL has been configured).

3.	The UE#1 may send a request for downloading the application list and the Avatar ID list to the DCSF via the MF over the bootstrap DC. The MF forwards the request for Avatar ID list to the DCSF via MDC1. The DCSF requests the Avatar ID list for the UE from the DC AS via MDC3. The DC AS may obtain the Avatar ID list for the UE from the BAR. The DC AS returns the Avatar ID list to the DCSF via MDC3. The DCSF returns the application list and the Avatar ID list to the UE#1 via the MF over MDC1. The MF forwards the response to the UE via the bootstrap DC.

NOTE:	When providing Avatar ID list and application list, the Avatar ID in the Avatar ID list may be associated with one or more data channel applications identified in the application list or vise versa, or may not be associated with any data channel application when the Avatar ID can be used for any data channel application identified in the application list.

4.	The P2A2P application DC has been established between the UE#1 and the UE#2.

5.	The UE#1 may send a request for downloading the Avatar ID list to the DC AS via the MF over the application DC. The MF forwards the request to the DC AS via MDC2. The DC AS may obtain the Avatar ID list per the DC application corresponding to the application DC for the UE#1 from the BAR. The DC AS returns the Avatar ID list to the UE#1 via the MF over MDC2. The MF forwards the response to the UE#1 via the bootstrap DC.

The UE#1 selects an Avatar ID and sends to the DC AS via the application data channel, the DC AS requests the BAR to download the Avatar Representation corresponding to the Avatar ID to the MF. The BAR downloads the Avatar Representation to the MF via MDC4 accordingly.

6.	The MF receives media data via the application data channel from the UE#1, it uses the Avatar Representation to render the media data with animation data, and sends the Avatar media towards the remote side (UE#2).

7.	The MF receives media data via the application data channel from the remote side (UE#2), it uses the Avatar Representation to render the media data with animation data, and sends the Avatar media towards the UE#1.

# * * * * End of changes * * * *