3GPP TSG-SA WG2 Meeting #166	S2-2411951

# Orlando, USA, 18 - 22 November, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1520 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call setup of UE-Satellite-UE communication with early media in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Handling of early media for UE-satellite-UE communication has been discussed. S2-2411950 discuss the issue for early media handling, and proposes to sustain using ground-AGW for early media based on operator policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of procedure for supporting UE-Satellite-UE communication with early media in IMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Early media may be terminated or interrupted when UE-satellite-UE communication is activated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AX.5.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented after TS 23.228 CR 1428 and 1492. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change (All following text are new) ---

AX.5.2	At call setup with early media

During the call setup with both originating and terminating UEs using regenerative NR satellite access, the originating P-CSCF selects the IMS-AGW on ground as default, which may be used to transfer early media. In order to keep media continuity for playing the early media, the originating P-CSCF sustain using the selected IMS-AGW on ground for the early media. See Figure AX.5.2-1 that describes the procedure.

Figure AX.5.2-1: IMS session establishment procedure with early media

1.	Steps 1-11 of clause AX.5.1 apply. The UE A or the originating P-CSCF indicates support of early media authorization (i.e., P-Early-Media=support) in the initial SIP INVITE request. The SIP 18X response returned by the UE B uses D1 as the dialog ID. The IMS AS in the IMS Core receives the SIP 18X response (D1).

2.	The IMS AS sends an SIP 18X response including the identification of the satellite serving UE B, the P-Early-Media indicating early media allowed (e.g., P-Early-Media=sendonly), and a modified/generated SDP answer for early media to the originating side. Depends on operator policy, the SIP 18X response may uses D2 as the dialog ID that is different from D1, in this case, the IMS AS will also forward the SIP 18X response (D1) toward the originating side.

3.	Step 12 of clause AX.5.1 applies with the difference that if the P-Early-Media is received in the SIP 18X response, the P-CSCF does not select an IMS-AGW A on satellite for the early media described in the SDP answer included in the received SIP 18X response.

4a-4b. Steps 12a-12b of clause AX.5.1 apply if an SIP 18X response without P-Early-Media is received. Based on the operator policy, if steps 12a-12b of clause AX.5.1 do not apply for early media, the P-CSCF does not release the IMS-AGW A' on ground when an SIP 18X response with P-Early-Media is received.

5. The P-CSCF A instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service operation.

6.	If only SDP answer for early media is received, the parameters sent by the P-CSCF A described in step 13 of clause AX.5.1 will not make the SMF insert UL CL/BP and L-PSA UPF for IMS PDU session. If both SDP answer for regular media and SDP answer for early media is received, the P-CSCF A sends the parameters described in step 13 of clause AX.5.1 to the PCF according to both SDP answers, which will make the SMF to insert UL CL/BP and L-PSA UPF for IMS PDU session to enable UE-Satellite-UE communication for regular media.

7.	Steps 14-15 of clause AX.5.1 apply with the difference that the P-CSCF A may receive multiple Response Confirmations with different dialog IDs.

8.	The P-CSCF A forwards the Response Confirmation to the IMS AS. In case the corresponding SIP 18X response includes P-Early-Media, the P-CSCF A will not include SDP offer generated by P-CSCF A in the Response Confirmation. In the case of multiple SIP 18X responses with different dialog IDs sent in step 2, the IMS AS will receive multiple Response Confirmations with different dialog IDs and will not forward the Response Confirmation using D2 as the dialog ID towards terminating side.

9-10.	Steps 17-18 of clause AX.5.1 applies if an SDP offer received, which is only for regular media.

11.	Subsequent procedures before answering continue.

12.	The UE B answers the call and sends a SIP 200 response to the P-CSCF B. The P-CSCF B forwards the SIP 200 response to the IMS AS in the IMS Core. The IMS AS may trigger the IMS session update procedure.

13.	Subsequent procedures continue, both the P-CSCF A and the P-CSCF B will receive the answer message.

14.	If the IMS-AGW on satellite is not selected and UE-satellite-UE communication is determined to be activated in step 3, the P-CSCF A interacts with IMS-AGW A on satellite to allocate transport resources for both IMS access and core network sides. The P-CSCF B may update the allocated IMS-AGW on the satellite for UE B according to received SDP information.

15.	The P-CSCF A releases the IMS-AGW A' on ground.

## ---End of the Change---