3GPP TSG-WG SA2 Meeting #166	S2-2412011

Orlando, USA, 18-22 Nov, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on UPF onboard for UE-Satellite-UE communication

Document for:	Discussion

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3_ARCH / Rel-19

Abstract: The UPF onboard solution should maximize reuse of principles of UPF&IMS-AGW onboard solution, including how P-CSCF triggers the optimization of media routing during call setup and how the procedure performs at change of satellite.

# 1. Introduction

To support UPF onboard (without IMS-AGW) for UE-Satellite-UE communication, this discussion paper analyses how principles of UPF&IMS-AGW onboard solution can be reused and what differences should be considered for the UPF onboard solution.

# 2. Discussion

The major principles of UPF&IMS-AGW onboard solution described in TS 23.228 CR 1492 and CR 5770 can be summarized as follows:

At call setup:

1) MO side P-CSCF selects a ground IMS-AGW after receiving a SIP INVITE message, and the MT side P-CSCF decides whether to activate UE-Satellite-UE communication based on identifier of satellites serving two UEs.

2) AF influence traffic on routing is used to trigger the UL CL/BP and Local PSA UPF insertion on satellite. The address of IMS-AGW on satellite is proved to SMF to direct traffic from UL CL/BP to Local PSA UPF. N6 traffic routing information is provided to SMF to direction traffic from L-PSA to IMS-AGW.

3) P-CSCF should subscribe UP path change event from SMF to know satellite change event.

4) The IMS-AGW on ground will be released by MO side P-CSCF if UE-Satellite-UE communication is activated.

At change of satellite:

5) At change of satellite, the P-CSCF sends a message to IMS-AS, the IMS-AS sends a SIP re-INVITE message to UE to trigger media re-negation, IMS-AGW address information can be transferred to between P-CSCFs via SDP message.

6) SMF may report ground DNAI to P-CSCF after UE handover to a target satellite and this means ground fall back procedure should be performed, as during handover procedure a target satellite may has no connection with source satellite and the SMF has to direct UE UL traffic to ground PSA for service continuity.

7) If SMF reports a target satellite ID and associated DNAI, the P-CSCF should determine whether UE-Satellite-UE communication can continue or not based on target satellite identifier and peer side satellite identifier or based on IMS-AGW report.

When only UL CL/BP and Local PSA UPF deployed on satellite, the above principles can be reused in general with some minor differences:

At call setup, principle 1), 2), 3), 4) can be reused with following differences:

-  AF influence traffic on routing is used to trigger the UL CL/BP and Local PSA UPF insertion on satellite but the address of peer UE should be provided to SMF to direct traffic to Local PSA UPF. N6 traffic routing information is also provided to SMF to direct traffic form L-PSA to peer side L-PSA.

-  UE address and N6 routing information should be transferred between MO side and MT side P-CSCF via SDP message if target P-CSCF decides to activate UE-Satellite-UE communication.

-  MO/MT side P-CSCF does not need to select IMS-AGW on satellite, MO side P-CSCF only needs to release selected IMS-AGW on ground, if UE-Satellite-UE communication is activated.

At change of satellite, principle 5), 6), 7) can be reused with following differences:

-  At change of satellite, the P-CSCF sends a message to IMS-AS, the IMS-AS sends a SIP re-INVITE message to P-CSCF to trigger address information update, L-PSA N6 tunnel address information (as N6 routing information) can be transferred between two P-CSCFs similar with IMS-AGW address information exchange, the difference is P-CSCF further transfer the N6 routing information to SMF after receiving address information from the other side.

This means the SMFs serving two UEs can still be different, and it is IMS system to transfer peer side tunnel information for this two SMFs. The re-INVITE message sent to UE does not bring new address information but can keep UE informed that there is a satellite change event. In this way, the state machine in P-CSCF is similar with IMS-AGW onboard solution, the difference is configuring/obtaining IMS-AGW address by interacting with IMS-AGW is replaced with configuring/obtaining N6 routing information by interacting with SMF via PCF.

-  If SMF reports a target satellite ID and L-PSA N6 routing information, the P-CSCF should determine whether UE-Satellite-UE communication can continue or not based on target satellite identifier and peer side satellite identifier or P-CSCF can querying SMF by transfer peer side N6 routing information to SMF (SMF can know whether two satellites are connected based on UPF report).

Figure 1: UE address and N6 routing information should be exchanged between two SMFs via IMS system for media traffic optimization

Observation#1: Principles of UPF&IMS-AGW onboard solution can be reused in general with some minor revision for only UPF onboard solution.

Observation#1.1: The major point to enable only UPF onboard solution work is utilizing IMS system to exchange UE address and N6 routing information between two SMFs, SDP message may be enhanced to carry UE address and N6 routing information. Even if the two UEs use the same SMF, the SMF does not need to correlate this two UEs’ context for UE-Satellite-UE communication.

Observation#1.2: N6 routing information needs to be reported from SMF to P-PCSF, and the P-CSF can transfer it to the other side P-CSCF at call setup or change of satellite.

# 3. Conclusion and proposal(s)

Proposal#1: Reuse principles of UPF&IMS-AGW onboard solution with differences listed above including support of SMF reporting N6 routing information to P-CSCF, enhancement of SDP message to carry UE address and N6 routing information and exchange them between P-CSCFs, support of querying SMF whether two satellites are connected or not by sending peer side N6 tunnel information to SMF.