# SA WG2 Meeting #S2-166																			S2-2412021

# 18 - 22 November, 2024, Orlando, USA

Source:	OPPO

Title:	UE – LMF user plane availability for LCS-UPP

Agenda item:	9.6.2 (5G_eLCS_Ph3)

Document for:	Decision

1. Introduction

SA2 received LS on "UE's user plane availability status for LCS-UPP" (S2-2411289 / C1-246023). CT1 identified the following problem. Currently, an LMF is not permitted to subscribe to an SMF's notification service, while the PDU session status information is transparent to an AMF and therefore cannot be retrieved by an LMF. If, for example the 3GPP PS Data Off status is activated in a UE, the UE will not be able to send any user plane (UP) data to the LMF, unless the operator considers such UP data belongs to 3GPP PS Data Off Exempt Service. A problem of the same nature arises when 3GPP PS Data Off status is deactivated in the UE. Also, in this case the LMF stays unaware of the status change.

Another example would be the release of the PDU session that is used for UP communications between the UE and the LMF. CT1 resolved this latter problem by triggering the UE Requested User Plane Connection Release procedure, which puts extra burden on air interface.

2. Discussion

CT1 discussed various solution proposals before agreeing on the LS to SA2 on "UE's user plane availability status for LCS-UPP" (S2-2411289 / C1-246023). The tabled proposals (C1-245361, C1-245504, C1-245505 and C1-245506) and also the majority of those assumed UE should be sending indications over the control plane to the LMF (UE – AMF – LMF).

A timer based solutions were also discussed, but its drawbacks are (a) wasting time and resources on retransmissions and (b) the cause of the failure remains unclear.

Another solution proposed the LMF should retrieve the PDU session status info directly from the SMF, e.g. the 3GPP PS Data Off status. The latter proposal however was deemed outside CT1 scope and therefore could not find its way into the said LS to SA2 (S2-2411289 / C1-246023).

If the SMF notification service will become available also to an LMF, this will keep the LMF aware of all relevant changes to the PDU session, which in turn eliminates the need for the UE to waste the radio resources and also the AMF resources by triggering extra procedures just to inform the LMF about the changes in PDU session status. Another benefit is that the LMF receives the change in the relevant status timely and also will become explicitly aware what went right or what went wrong.

LMF, which features LMF-C and LMF-U functions is the only NF in 5GC that relies in the PDU session, but does not have neither direct interface with SMF, not is permitted to benefit from SMF Service.

3. Conclusions

LMF subscribing to SMF Service notifications-based solution is simpler and elegant. This will provide LMF-U (via LMF-C) with valuable PDU session information in a timely manner. This will also free UE, RAN and AMF resources that are otherwise required to obtain and deliver the very same information to the LMF.

4. Proposal

It is proposed to agree the changes to TS 23.273 and TS 23.502, which are presented in S2-2412023, S2-2412025 and S2-2412027. It is also proposed to reply the LS to CT1 with S2-2412022.