3GPP TSG-SA WG2#166	S2-2412032

Orlando, FL,USA, 18-22 November 2024	(was S2-240xxxx)

Source:	NEC

Title:	Discussion paper attach procedure with default PDN connection

Document for:	Discussion/Approval

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3_ARCH/Rel.19

Abstract of the contribution: Discusses and propose a way forward for attach procedure with default PDN connection and combined EPS/IMSI attach for SMS only attach.

# 1.	Discussion

Current solution for S&F only considers the case of attach without PDN connection. The current attach procedure is based on the Figure 1 which doesn’t consider default PDN connections establishment procedure scenario steps (12-16) of the Figure 2 and combined EPS/IMSI attach procedure for SMS only. Without this procedure the UE supporting attach with default PDN connection will not be able to attach to the network and can’t access the S&F services.

Also Combined attach for EPS/IMSI service is not considered (steps of figure 3 is not supported) and therefore network not supporting “SMS in MME” and deploying SMS over SGs interface will not be able to support SMS transmission in S&F mode.

1. Attach procedure for UE and network supporting S&F operation mode.

#### 2. Attach procedure with default PDN Connection establishment (Subclause 5.3.2.1	E-UTRAN Initial Attach of TS 23.401)

Figure 2:  Figure 5.3.2.1-1: Attach procedure of TS 23.401

## 3. Combined attach procedure (Attach procedure sub clause 5.2 of TS 23.272)

The attach procedure for the CS fallback, SMS over SGs or "SMS in MME" in EPS is realized based on the combined GPRS/IMSI Attach procedure specified in TS 23.060 [3].

Figure 3: Figure 5.2-1: Attach Procedure of TS 23.272

Problem Statement 1: S&F operation is not supported for the UE supporting Attach with default PDN connection.

As per the current conclusion and ongoing normative work based on Figure 1 above, when the feeder link is connected, the MME-GND fetches the subscription data and authentication vectors and gives the subscription and authentication vectors to the MME-SAT.

when the MME-SAT gets UE subscription and Authentication vectors, authentications and security mode procedure takes place between the UE and the MME-SAT when the UE initiates attach procedure when the S&F wait timer expires and the UE accesses the network second time. After successful authentication procedure the UE sends attach accept message.

The above procedure doesn’t cover the steps 12.-16 of Figure 2 which establishes the default PDN connection establishment procedure. Therefore, the UE is attached with S&F mode without default PDN connection establishment. The UE supporting attach with PDN connection establishment will not be able to support S&F service.

Problem Statement 2: Combined EPS/IMSI attach to support SMS over SGs interface is not supported.

As per the current conclusion and ongoing normative work based on Figure 1, the combined attach procedure over SGs interface is not supported i.e., the SGs interface establishment procedures (steps 3 – 7 of Figure 3) is not defined. Some operators have already deployed SGs interface to support SMS over SGs only. For the operators which deployed SMS over SGs interface only, the SMS transmission is not possible when the UE is attached for S&F service operation mode.

# 2.	Proposal

Solution Proposal:

To solve problem 1 and 2 above it is proposed that when the UE with S&F operation enabled is registering to the network supporting S&F operation mode, after the successful authentication and SMC procedure, the network rejects the attach procedure with S&F wait time and optionally satellite monitoring list.

Afterwards when the feeder link is connected between the MME-GND and MME-SAT, the MME-GND performs PDN connection establishment procedure (Steps 12-16 of Figure 2) and thereafter performs SGs interface establishment step 3-7 of figure 3. The MME-GND sync with the MME-SAT regarding, the results of default PDN connection establishment procedure and SGs interface association.

After establishing the service link, when the UE initiates attach procedure with MME-SAT, the MME-SAT sends attach accept message after successful establishment of the PDN connection and result of SGs association.

The scenario flow is proposed for combined attach with default PDN connection establishment.

Figure 4: combined attach procedure with default PDN connection in S&F mode of operation.

NOTE: Step 11 is optional if the operator doesn’t support SGs interface.

# 3.	Conclusion

Contributions S2-2412040 and S2-2412045 are submitted incorporating the solution described above.