3GPP TSG-WG SA2 Meeting #166	S2-2412047

Orlando, Nov 18 – 22, 2024	(revision of S2-24xxxxx)

Source:	Nokia

Title:	Conclusions for KI#1 on topology1

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusions for KI#1 on topology 1 architecture are proposed.

# 1. Introduction/Discussion

This contribution is prepared to update the conclusions for KI#1 on topology 1.

When we conclude the architecture for the AIoT services, both topologies shall be considered together to have one harmonized architecture.

There are two main use cases to support AIoT services, considering both topology 1 and 2:

- AF knows the “Reader IDs” to be Involved in advance.

  - In this case, topology 2 with the AMF is necessary to locate the readers and translate SUPIs/GPSIs into NGAP IDs. When the AF requests a specific UE reader ID, the AIOTF will send the AIoT request to the AMF, including the SUPI of the UE reader. The AMF will then locate the UE and check its reachability. If reachable, it will convert the SUPI into the NGAP ID and forward the AIoT request over the NGAP interface. If the UE is not reachable, the AMF will page the UE to establish a connection before forwarding the message. To support this, a new NGAP procedure, "AIoT Request," will be introduced from the AMF to the gNB, which will include a list of NGAP IDs.

- AF knows the “Area”.

  - If we proceed via AMF for both topologies: We would simply add an “Area ID” or a list of “candidate Reader IDs” pre-selected by the AIOTF in the new NGAP procedure described above.

  - If we proceed via a direct interface between the BS (gNB) reader and the AIOTF:

    - A new interface would need to be defined between the gNB and AIOTF even if it is a lightweight version of NGAP, including new multivendor interworking testing.

    - This would impact the gNB, requiring a new NG Setup* procedure with different IEs and different behavior when connected to the AIOTF. The AIOTF cannot be treated as an AMF.

    - For topology 2, the request would not be fulfilled if there happen to be no UE readers connected (AMF is required to put the UEs into the connected mode) when it is a gNB that selects the UE readers.

    - For topology 2, it would not resolve the issue of per-application authorization to function as a reader.

    - AIOTF would need to support two protocol stacks (i.e., XXAP with gNB as well as SBI with AMF), as RAN3 agreed the gNB will use XXAP/SCTP/IP.

    - It would allow having two separate termination of RAN CP interface (i.e., the AMF and the AIOTF) from the gNB to the CN.

    - If we define a new interface between the gNB and the AIOTF, it could also open the possibility of defining a new interface between the gNB and a new NF for other features, like sensing.

In addition, there is a scenario where an indoor gNB functions both as a BS reader and as an NR gNB serving UE readers.

If we consider standalone BS reader scenarios only, a direct interface might be acceptable as it would simplify the design. However, we believe it’s essential to take a more holistic approach, one that covers all cases.

By involving the AMF, we can create a unified architecture for both topologies with the “combo” gNB, thus avoiding the need for a new interface and protocol and preventing unnecessary duplication.

As shown above, there are many showstoppers to supporting a direct interface between the BS reader and the AIOTF. Therefore, we propose removing the direct path option and conclude the architecture for topology 1 with the AMF-based architecture.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v1.1.0.

FIRST CHANGE

### 8.1.2	Architecture to Support Topology 1

The principles and aspects in this clause are agreed to support Topology 1:

-	The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1:

-	Communicates with BS Reader (i.e. AIoT RAN) via an AMF to trigger e.g. inventory request and command request).

NOTE 1:	There is no assumption about whether the AIoT RAN (i.e. BS Reader) also has gNB functionality for NR-Uu or not.

NOTE 2:	It is assumed Ambient IoT services can be deployed independently from existing deployments.

-	The AIOTF connects with a BS Reader via an AMF. The N2 interface between the BS Reader and AMF supports Ambient IoT services including delivery of inventory/command messages.

NOTE 3:	If network isolation is required an AMF instance is deployed for supporting the AIOTF communication with the BS Reader.

-	NGAP between the BS Reader and the AMF is enhanced to support Ambient IoT Services.

NOTE 4:	The details of the enhancements will be concluded by RAN WG3.

-	The AMF shall be enhanced to support Services which are used by an AIOTF for Ambient IoT Operations. New SBI based services on the AMF (to be used by the AIOTF) are introduced in 5GC.

-	The AMF passes information between BS Readers (e.g. operation requests and responses) and the AIOTF.

Note 5:	Details of the Service (e.g. whether it is a new service or the existing Namf service is enhanced) will be determined during the normative phase.

-	Figure 8.1.2-1 below shows the aspects related to Topology 1 reference architecture with other NFs removed.

Figure 8.1.2-1: Non-Roaming 5G System Architecture

-	Figure 8.1.2-2 below shows the aspects related to Topology 1 in reference point representation with other NFs removed.

Figure 8.1.2-2: Non-Roaming 5G System Architecture in reference point representation

-	Figure 8.1.2-3 below shows the aspects related to Topology 1 protocol stack between the BS Reader and AIOTF.

Figure 8.1.2-3: Example Protocol Between AIOTF and AIoT Device for Topology 1

END OF CHANGES