3GPP SA WG2 Meeting #166 	S2-2412069

Orlando, US, Nov 18 – 22, 2024	(revision of xx-yyxxxx)

# Source:	ZTE

# Title:	New SID on the enhancement to URLLC and LAN

# Document for:	Information

# Agenda Item:	30.8

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on the enhancement to URLLC and LAN

Acronym:	TBD

Unique identifier:	TBD

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No |  |  |  |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

URLLC-FRER related:

From Rel-16, 3GPP has specified redundant transmission for URLLC in 23.501 clause 5.33. However the drawback is that the URLLC replication is either done for all traffic or no traffic for a given application. However, the current replication mechanism assumes that application layer protocols are used to control the traffic over multiple user plane paths for high reliability, with no control available within 3GPP system for operators.

URLLC-Ring topology related:

In many actual industrial deployment, the reliability depends on the ring topology and related protocol. Example the PRP/HSR (Parallel Redundancy Protocol / High-availability Seamless Redundancy, defined in IEC 62439-3) and MRP (Media Redundancy Protocol, defined in IEC 62439-2) are used by industry Ethernet to ensure the reliability.

The HSR node (e.g. call DANH, Doubly Attached Nodes implementing HSR) has two interface/port connecting to the HSR ring. The HSR node sends the traffic in opposite directions, e.g. Port-A sends traffic counter clockwise in the ring, and Port-B sends traffic clockwise in the ring.

In such case, the packets in the ring have the same source and destination MAC address, thus cannot be differentiated by MAC address thus cannot be transferred via two QoS Flows/PDU Sessions.

Loop prevention:

For the scenarios not related to 5G VN, Ethernet PDU sessions (specific DNN+S-NSSAI) integrated with LAN. So far, the (M/R)STP support (loop prevention) is an implementation in the UPF. The benefit of the study is to enable 5G with the flexibility to enable/disable/configure the STP via c-plane.

For the scenarios related to 5G VN, two UEs (one at the beginning and one at the end of a conveyor belt, may make a ring out of the linear topology of the Ethernet LAN over the 5GS. A loop prevention protocol e.g. (M/R)STP avoids circling Multicast/Unicast frames. Additionally, this setup allows a media redundancy using the 5GS – if the wireline breaks, every node is still reachable through the 5GS. 5GS enhancements for support of (M/R)STP needs to be studied in order to support prevention of loops.

User Plane enhancement:

In some vertical scenarios there is no AF deployed and it is not possible to provide the application layer information to 5GC. 5GS needs to derive the application layer information without dependency on the deployment of AF in vertical.

4	Objective

Following are the objectives for this study:

URLLC-FRER related:

Support for 3GPP system-controlled E2E replication/elimination for redundancy for both Ethernet and IP type PDU Sessions. It is assumed that the two PDU Sessions carrying the redundant traffic shall be as following:

- for a single UE, or

- for two UEs that are independently registered.

The study shall address architecture and protocol aspects for the 3GPP network operators on the above scenarios:

WT 1.1 For FRER:

- Ability to enable 3GPP entities (e.g. DS-TT, NW-TT) to support replication and elimination for redundancy, protocol aspects to replicate/eliminate for FRER.

NOTE 1:	Either the terminal or network side replication and elimination endpoint or both can be within 5GS.

WT 1.2 For DetNet:

- Ability to support replication and elimination for redundancy, protocol aspects to replicate/eliminate for DetNet.

- Extended capability exposure framework to allow the entity outside the 5GS to discover, or request selective duplication, 5GS reliability and redundancy functionality for DetNet.

NOTE 2:	It is assumed both the terminal and network side replication and elimination endpoint to be within 5GS.

NOTE 3:	Deployments can support DetNet controller for replication/elimination that is outside 3GPP system

WT 1.3 For 3GPP specific replication and elimination capability:

- Ability to support replication and elimination for redundancy, protocol aspects to replicate/eliminate.

- Extended capability exposure framework to allow the entity outside the 5GS to discover, or request selective duplication, 5GS reliability and redundancy functionality

NOTE 4:	It is assumed both the terminal and network side replication and elimination endpoint to be within 5GS.

URLLC-Ring related:

- Ability to support ring topology mechanisms (without differentiating the redundant packets to avoid updating user devices), e.g. MRP (Media Redundancy Protocol) and HSR (High-availability Seamless Redundancy).

Loop prevention related:

- WT-3.1: How to define support of the Multiple Spanning Tree protocol in the UPF for 5G VN scenarios to ensure loop prevention and enhancements for its activation/deactivation.

- WT-3.2: How to define support òf Multiple Spanning Tree protocol or Rapid Spanning Tree protocol in UPF in Ethernet PDU Sessions which are not related to 5G VN.

User Plane enhancement:

- WT-4.1: How to derive and use application layer information from user plane for policy control and reduce the dependency on Application Function

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.XXX | Study on the enhancement to URLLC and LAN | TBD | TBD |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

TBD

7	Work item leadership

SA2

8	Aspects that involve other WGs

Security aspects are considered by SA3.

RAN aspects are considered by RAN WGs.

9	Supporting Individual Members

Supporting IM name