3GPP TSG|WG-SA2 Meeting #166	S2-2412090

Orlando, USA, November 18 – 22, 2024	(revision of S2-20xxxx)

Source:	Samsung

Title:	Discussion on solution for authorizing calling party to use third party identities

Agenda Item:	19.2.2

Document for:	Information

# 1.	Background

In the SA2#165, S2-2411009 was agreed as part of the KI#4 but there is one Editor’s Note which need to be addressed.

Editor’s Note: It is FFS whether the Originating S-CSCF and/or the IMS AS authorizes the calling party (IP-PBX or UE) to use third party user identity information (this would be based on the user profile retrieved from the HSS).

# 2.	Discussion

In the KI#4, the RCD information (i.e. third party identity information) is added and after successful signature addition/verification these information is displayed in the called party’s device. It helps the called party while accepting the incoming call based on the RCD information. Hence, network should ensure to allow only authorized IMPUs for RCD information to be added and proceed further singing.

There are two options 1) S-CSCF and 2) IMS AS was considered during conclusion to do this functionality of authorizing whether the calling party can use the RCD information (i.e. third party identity information)

| Option 1: (S-CSCF based approach) | Option 2: (IMS AS based approach) |
| --- | --- |
| 1) One parameter like “ Authentication and Authorization to use third party identities” need to be configured in service profile in HSS per IMPU as Enabled or Disabled 2) S-CSCF after receiving INVITE from calling party checks whether the “ Authentication and Authorization to use third party” is Enabled or Disabled 3) If S-CSCF finds the parameter value is disabled then it rejects the INVITE | 1) One parameter like “ Authentication and Authorization to use third party identities” need to be configured in repository data in HSS per IMPU as Enabled or Disabled 2) IMS AS after receiving INVITE from calling party checks whether the “ Authentication and Authorization to use third party” is Enabled or Disabled 3) If IMS AS finds the parameter value is disabled then it rejects the INVITE |

This is a simple IMS call flow with minimum three IMS AS in the path. As per the specification both ENUM AS and SCC-AS shall be invoked before any other IMS AS.

If the authorization check is kept at S-CSCF (Option 1) then by looking at the retrieved user profile from HSS, if the “Authentication and Authorization to use third party identities” is disabled, the INVITE is rejected without invoking any IMS AS

If we keep this authorization check at IMS AS (Option 2) then in comparison to option 1, step 2 to step 8 is having additional signalling in the system and unnecessary processing of INVITE by each IMS AS.

Observation: Irrespective of the option, the parameter “Authentication and Authorization to use third party identities” need to be configured at HSS and this need to be standardized. So the impact is same for both the options. However, as part of option 2, the extra processing by each IMS AS and wastage of resources keeping energy efficiency criteria is not suitable for the requirement.

Proposal: By looking at the impacts and advantages/disadvantages of both the solution, it is proposed to consider S-CSCF based approach as it adds less complexity to the system in comparison to IMS AS based approach.

# 3.	Proposal

To ensure a simple solution, which addresses this important authorization check for calling party to use third party identities without adding any complexity to the system, it proposes to select one option and its corresponding action as a way forward from above two options in SA2 and Option 1 is preferred.