3GPP TSG-SA WG2 Meeting #166	S2-2412095

# Orlando, US, 18-22 November 2024

Source:	Nokia

Title:	Discussion of incoming LS on FS_IMS_RES outcome and future work plan

Agenda item:	4.1

Document for:	Decision

Introduction:

CT4 has studied possible IMS disaster prevention and restoration enhancement in FS_IMS_RES and intends to send 3GPP TR 29.866 to CT#106 for approval, expecting to start normative work based on the conclusions in TR 29.866. CT4 has sent an LS to SA2, CT1 and CT3 to review these solutions and provide feedback to CT4 (C4-244485).

This discussion paper provides some general system architecture aspects regarding IMS disaster prevention and restoration and also observations on some of the solutions in the CT4 TR.

Discussion:

Some general observations:

- IMS is standardized back in Rel-5; commercial deployments exist since decades. Overload control, restoration features and failover procedures are available in implementations and deployments for many years (if not decades) especially for critical network functions like the HSS. Over the last 10-15 years these solutions have constantly being enhanced and refined.

- What is the problem going to be solved with this work done in CT4? Where are the requirements coming from? Which operator has identified problems/deficiencies that cannot been overcome with existing means and implementations? There is a high risk that this work will create solutions in standards that are never implemented or force vendors to implement a solution that is not necessary as the existing implementation provides other means to cope with failure scenarios.

- Changes to basic IMS call flows in TS 23.228 must be carefully treated and studied first by SA2 based on requirements from operators and use cases, considering existing restoration procedures and failover solutions.

- Changing call flows for a single particular failure case may result in behaviour, which is less optimal in regular scenarios or for other failure cases. Given that the HSS is usually the most reliable instance in the network, sacrificing important features or functionality for such a scenario may be counterproductive.

Some observation on specific solutions:

- Solution#1 modifies P-/S-CSCF behavior and changes basic IMS call flows (sending re-register directly to S-CSCF). In addition, I-CSCF has been added to the re-registration procedure to allow for load re-balancing or selecting another S-CSCF after a failure.

- Solution#2: The proposed changes are not conditional to HSS being in overload situation, instead change the normal behavior of S-CSCF, TAS and IP-SM-GW. S-CSCF ignoring error response and send 200 OK create a problem as terminating calls will not be delivered.

- Solution#3: Proposes to store S-CSCF address in a new AS during the registration process. I-CSCF sends REGISTER and INVITE request to the AS in case of HSS failure. It is unclear how this solution solves the HSS failure scenario as it creates just a new single point failure (the AS) storing sensitive data. In addition, this solution requires not only a new AS but also changes to I-CSCF and iFC which are in place and would need to be changed just for this failure handling. It is also doubtful that such an architecture would be superior to redundancy provided within and between implementations of logical HSS NF(s), in an architecture with state-less HSS/frontends and geo-redundant UDR based on state-of-the art technology.

- Solution#4: I-CSCF behaves as a stateful proxy, sending SIP OPTIONS based on some events (REGISTER has been received, HSS does not react) is not proxy behaviour, but rather makes the I-CSCF a B2BUA.

Proposal:

It is proposed to send a reply LS to CT4, highlighting the observations mentioned above. Without a proper understanding of the pain points we want to solve and studying the solutions in detail SA2 cannot agree to make any normative changes to TS 23.228.