3GPP TSG SA WG2 Meeting #166	S2-2412096

# 18 – 22 November, 2024, Orlando, USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1524 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for supporting of third party user identity information in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedure details for supporting third party user identity information in IMS to align the conclusions in KI#4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Feature procedural details along in the lines of KI#4 conclusions specified in TR 23.700-77 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specififcation will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AX (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# AX.1	Feature Procedure

Support for authentication and authorization of third party user identity information, additional functionalities in the S-CSCF, STI-AS, STI-VS, HSS as will be depicted below.

Figure AX.2-1: Authentication and Authorization of third party user identity information during an IMS session

The detailed procedure is described in Figure AX.-1.

- The AF (e.g. third party like an Enterprise) will provision the third party user identitiy information by invoking the service operation Nimsas_ImsPP service operation for the target user (i.e for single IMPU or Wildcard IMPU).

- IMS AS after receiving the RCD information from NEF, stores in HSS using the existing ImsSDM service operation.

3.	UE1 is already registered.

4.	UE1 initiate an IMS session by sending INVITE to UE2.

5.   S-CSCF or IMS AS download the RCD information as part of the subscription information of the IMPU from HSS.

6.	S-CSCF or IMS AS checks the subscription information whether the user is authorized to use the third party user identity information identity service. If the parameter is enabled, then S-CSCF or IMS As adds the RCD information to the INVITE.

If the RCD information is not present in subscription profile then S-CSCF or IMS AS contacts the configured third party server to retrieve the RCD information and then adds to the INVITE.

7.	 S-CSCF or IMS AS will invoke the Signing server (STI-AS)  . This Signing server address, S-CSCF or IMS AS may get it from subscription information.

8.	Signing server after receiving the RCD information INVITE from S-CSCF or IMS AS, will  sign it and provides the response .

NOTE 1:	Adding the RCD information in specific headers by S-CSCF or IMS AS and singing the headers by STI-AS will be defined by CT1.

9.  S-CSCF forwards the INVITE to the terminating side.

10.	S-CSCF or IMS AS at terminating side will invoke the verification server based and forwards the INVITE request to STI-VS

11.	Verification server (STI-VS) will verify the signing and provides the validity status along with the INVITE to S-CSCF or IMS AS.

12.	S-CSCF at terminating side sends the INVITE towards UE2 based on the successful validation result. S-CSCF may reject the call for the unsuccessful result received from verification server.

13.	Successful call happens between UE1 & UE2.