3GPP TSG-WG SA2 Meeting #166	S2-2412129

Orlando, US, 18th Nov – 22nd Nov, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3 Conclusion on Support of Ambient IoT Services

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: Conclusion update for AIoT KI3.

1. Introduction

1.1	Overview

The conclusion for KI3 contains Editor’s notes related to the support of temporary disable and AIoT services exposed by the NEF, these ENs are resolve, and additionally the principle for the procedure is also provided.

1.2	AIoT Services

The AIoT Services supported by 5GC towards an AIoT Device, which match those exposed by the NEF to the AF are: Inventory, Read, Write and Disable.

Memory target for Read and Write Operations

For Read and Write there was a discussion in the last meeting about whether the target of the read and write service is non-volatile memory or some other type of memory. The type of memory that is targeted by the services depends on the AIoT Device implementation. It can be seen that some devices may have non-volatile memory, therefore allowing the AIoT Device to store and return the information. It can also be foreseen that some AIoT Devices may place the written data into volatile memory, for example, if it then needs to additionally process it, and similarly data read may come from volatile memory which is the result of some processing by the device. It can even be foreseen that an AIoT Device may even support both types of memory at different addresses indicated in the read/write operations.

The actual type of memory does not impact the operations, they perform the same from the service and standard point of view – transport data to the AIoT Device, transport data from the AIoT Device and what type of memory is targeted by the operations is an implementation detail for the device manufacture and application requesting the operation.

Therefore, can conclude that memory is targeted, but not be specific for the type of memory.

Temporary Enable & Disable

There is an Editor’s note that says whether and how to support temporary disable is FFS.

At present there is no solutions in the TR which cover these operations, so it is not clear how they can or will work, especially considering enabling a disabled device. While it there are potential solutions about how much of the AIoT Devices radio is disabled (fully, only TX etc), to enable the device it can be expected that some transmission will be required, there is nothing documented about them.

This requirement should be postponed and can be re-examined in a future release.

NEF Exposure

The NEF expose, its services, operations and parameters need to follow the operations supported by 5GC for Ambient IoT. Common to all the service operations are parameters which identify the target AIoT Devices and which Readers to use for the operation, along with operation specific parameters, for example, addresses to read/write data to/from, its length and data to write, etc.

1.3	Principles for procedures

The overall procedure for AIoT Services is shown in the figure below.

The procedure starts with the AF making a request to the NEF and it being passed to an AIOTF. The AIOTF determines relevant information to the handle the request, including which readers are used, and then performs an inventory / paging request. When AIoT Device(s) respond the AIoTF may interact with UDM/other NFs for subscription-like information. If a command is requested by the AF, then a AIoT specific NAS message it send to the AIoT Device for the requested operation, and results obtained. The results are then provided to the AF, via the NEF.

These steps are provided in clause 8.3.4 in the text proposal.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change * * * *

## 8.3	Conclusion on Key Issue #3

### 8.3.1	General

The clause concludes the following aspects:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices.

NOTE:	The above aspect includes studying whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

### 8.3.2	AIoT services supported by 5GC

The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device’s memory.

-	Write: Request to write information to an AIoT Device’s memory.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

The AIoTF provides services to which are used by the NEF to:

-	perform inventory operations,

-	to send read/write command to an AIoT Device to read/write data in the AIoT Device memory, or to send disable command to an AIoT Device permanently.

### 8.3.3	NEF exposure

NEF supports to expose Ambient IoT service towards the AF:

-	The NEF provides an AIoT Service to the AF with service operations to support the list of services in clause 8.3.2.

-.	The services, services operations and related parameters are common for topology 1 and topology 2 and provide the AF with a unified interface, so an AF does not need to differentiate between topology 1 and topology 2.

-	The AF, common to all service operation requests, provides information which identifies the target AIoT Device(s) for the operation and the expected number of AIoT Devices the operation targets.

-	The AF, common to all operation requests, provides a Reader Set ID which is used to identify a set of readers for the operation. The set of readers can contain a single Reader, or multiple Readers and the Readers can be either RAN Readers for topology 1, UE Readers for topology 2, or a mixture with the set.

NOTE:	Management of which readers are in each set is up to the network implementation.

-	The AF, for a Read service operation, provides an address to read from and the length of the data to read.

-	The AF, for a Write service operation, provides an address to write to, the data to write and the length of the data to write.

8.3.4	Principles on the procedures to support AIoT services

To support the services provided by 5GC and the NEF exposure of those AIoT services, the following procedures are supported:

-	Inventory Procedure.

-	Command Procedure, to transfer the Read, Write and Disable requests and responses to AIoT Device as AIoT specific NAS messages.

There are AIOT Reader Control messages that is used from the AIOTF towards the Reader and from the Reader to the AIOTF. The routing of the AIOT Reader Control messages and their encoding depends on the topology and transport to the Reader (see KI#1),

All the procedures follow have the following steps:

1.	The AF makes a service request to the NEF, including parameters to identify the target AIoT Device(s), target Readers and service operation specific parameters.

2.	The NEF determines an AIOTF for the requested operation, and invokes a new service operation on the AIOTF.

3.	For the requested operation the AIOTF:

1).	Performs initial reader selection by either:

-	Using the Reader Set ID, which represents the location/area, to identify the group of Readers which can be used for the operation in the location/area, if it provided by the AF via the NEF. The Reader Set ID can identify a multiple or an individual Reader to use for the request.

NOTE aa:	Management of which readers are identified by a Reader Set ID is up to the network deployment, configuration or implementation. How the Reader Set IDs are synchronised with an AF is dependent on the network deployment and operator policy.

- 	If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader.

- 	If no Reader Set ID or UE Reader ID is provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g., preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc.

NOTE bb:	There may be additional steps during initial reader selection depending upon the topology and routing, see the conclusion to KI#1.

2).	Determines a MASK based on the information from the AF. The MASK is a bit string that is provided to a Reader by the AIOTF to be included in the paging message on the AIoT air interface. AIoT Devices compare the MASK with (part of) their own AIoT Device Identifier (including Home Network Identifier) to determine whether respond to the paging message.

3).	Determines any Reader Assistance information required for the operation.

NOTE cc: The Reader Assistance information that is required for an operation needs to be determined in coordination with RAN, but could include, for example, a number of expected results for inventory, the length of command response data etc.

4). Constructs a AIOT Reader Control message requesting an Inventory operation using the determined MASK or page the AIoT Devices, and a TASK ID used as a correlation identifier for the AIOT to relate responses to request. The Inventory request is routed to the Readers determined by the initial reader selection.

See clause 8.1 for how to provide the request to a Reader.

NOTE dd:	Depending on the topology and routing of AIOT Reader Control messages, and if RAN determines that Reader down-selection is supported then RAN may only send the request to some of the readers initially selected by the AIOTF.

5). The Reader executes the inventory request, reporting AIoT specific NAS message responses from the AIoT Device to the AIOTF, including its Reader ID. The Reader may combine results from multiple AIoT Devices in the responding messages. The AIOTF can determine which requested the results are for using the TASK ID as a correlation identifier.

6).	The AIOTF may, depending on the information within the AIoT Device identifier, obtain subscription-like information from either:

-	A UDM within the service network performing the operation (either as identified by a AIoT Device Identifier or operator policy to check it been provided with information for a specific AIoT Device), or

-	A UDM within another network as identified by a AIoT Device Identifier, or

-	A third party as identified by a AIoT Device Identifier.

7).	Verifies the AIOT Device identity.

NOTE ee: Whether and how AIOT Device Identifiers are verified depends on SA3.

8).	If the operation if a command operation, the AIOT generates an AIoT specific NAS message for the command for and AIoT Device and the command, along with any additional information required by the Reader to execute the command, and TASK ID etc is then routed to the Reader. The Reader executes command, passing the AIoT specific NAS message to the AIoT Device and collecting any AIoT specific NAS responses. The AIoT specific NAS responses are then routed back to the AIOTF.

NOTE ff: Whether and how security protection is applied to the AIoT specific NAS message send to the AIoT Device and the response AIoT specific NAS message from the AIoT Device depends on SA3.

9).	Updates the AIoT Device subscription-like data to include the Reader identifier used for the AIoT Device, any service specific results (e.g., the device is disabled).

10). Provide the results of the operation to the NEF. Results from multiple AIoT Devices may be included in the service response(s).

4.	Provide the results of the operation from the NEF to the AF.

# * * * * End of changes * * * *