SA WG2 Meeting #S2-166	S2-2412131

18 - 22 November, 2024, Orlando, USA

# Liaison Statement

| Liaison Statement Title: | Clarification request on usage and control of UE Local Configuration |
| --- | --- |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| URSPWI-14 | 23rd October 2024 | eMeeting |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| URSPWI 15_003r3 | 04th October 2024 | Michele.Zarri@huawei.com |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| NG URSPWI | See below | gsmaliaisons@gsma.com |
| Security classification: | Non-confidential |  |

| Recipients: |  |
| --- | --- |
| Internal recipients: |  |
| External recipients: | To: 3GPP CT1 CC: 3GPP SA2 |

# Introduction

GSMA URSPWI is in the process of profiling URSP rules in NG.141 and would like to request a clarification about the control and usage of the UE local configuration.

The questions asked are based on the following understanding of clause 4.2.2.2 of TS 24.526 Release 19.

The clause states that if due to UE local configuration non-seamless offload is requested, then the UE shall report information on the non-3GPP access outside of a PDU session and this action takes precedence over evaluating the URSP rules.

Observation 1: the UE scans the UE local configuration to check if non-seamless non-3GPP offload is requested and if one is found the UE will not evaluate URSP rules. Therefore the check of the UE Local Configuration is performed before evaluating URSP rules ①.

If no suitable local configuration requiring non-seamless non-3GPP offload is requested, then the UE will then proceed as per steps a), b) and c) in clause 4.2.2.2.

Step b) states that if no non-default matching URSP rule can be found, the UE checks if UE local configuration for the application is available.

Observation 2: the UE scans the UE local configuration to check if a configuration for the application is available. This test takes place after evaluating URSP rules ②.

Observation 3: The UE Local configuration is checked twice: first to look for configurations related to non-seamless non-3GPP offload (①) and if none is found a second time to look for configurations related to PDU session association when no non-default URSP rule match the application (②).

Based on this understanding GSMA would like to ask:

- Can UE local configuration entries be created both by the operator and the user? If so

  - can the operator also create configuration for non seamless non-3GPP offload or only configuration for associating an application to a PDU session association?

  - Should the entries in the UE Local Configuration entered by one entity e.g., the user be protected so that the other entities e.g., operator or application cannot delete or modify them?

- If the operator can populate the UE local configuration for non seamless non-3GPP offload (see question 1.1.), what is the intended use of the RSD parameter "Non Seamless Offload Indication" defined in the URSP rule?

- Any particular reason why what seem to be three distinct sets of UE local configurations are captured in the same object?

# Action

GSMA URSPWI kindly asks CT1 to provide feedback to the above questions.

# Contact

In case of any further questions and or feedback, please contact Javier Sendin, Networks Group Director GSMA - JSendin@gsma.com.

GSMA looks forward to a continuation of the fruitful cooperation between the two organisations.

Next meetings:

URSPWI#15: 13 November 2024

URSPWI#16: 18 December 2024