SA WG2 Meeting #166	S2-2412136

# 14 - 18 November 2024, Orlando, USA

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1298 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VFL training procedure and VFL inference procedure with AF as a VFL server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR conclusion includes VFL training and VFL inference procedure with AF as a server. But the procedures are missing in the specification TS23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR provides VFL training procedure and VFL inference procedure with AF as VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3.1(new), 62H.2.4.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of 1st Change * * *

##### 6.2H.2.3.1	Training Procedures for Vertical Federated Learning when untrusted AF is acting as VFL server

Figure 6.2H.2.3.1-1: VFL model training with AF as VFL server

- The AF acting as VFL training server assigns a unique VFL Correlation ID.

Steps 2 to 7 are repeated until the MTLF acting as VFL training server determines that the training is complete.

- The AF acting as VFL server sends a request to start a VFL training procedure. It includes the VFL Correlation ID. For starting a VFL training procedure, AF may include sample information (e.g., list of UE ID, area/time condition for data collection) and VFL interoperability information (e.g., a VFL configuration ID when preconfigured VFL configuration used or other information which may indicate requested ML model, etc).

NEF may translate a requested analytic service information into an Analytic ID, when AF includes an analytic service information other than analytic ID.

When AF include temporary ID(s) of NWDAF provided by NEF at discovery procedure (see Clause 6.2H.2.1.1), NEF converts it into NWDAF ID and include in the request to NWDAF as VFL client in step 3.

3.	The NEF forward the request to the NWDAFs based on converted NWDAF ID information. If the message in step 2 contains external identifiers of UEs for sampling, the NEF maps them to internal identifiers of UEs.

4.	Based on the received request, NWDAF as VFL client collects data and locally train its ML Model. If intermediate results were received in step 2 or 3, each NWDAF as VFL client updates its own local ML model based on the received intermediate results and further trains the updated local ML model.

5.	Each NWDAF as VFL client sends the intermediate training results to the VFL server which include VFL correlation ID and NWDAF ID as VFL client. When NWADF receives the request in step 3, it send the response to NEF.

6	For each NWDAF as VFL client, the NEF forwards the intermediate training results to the AF. NEF convert NWDAF_ID(s) into temporary NWDAF ID which was received at step 2.

7.	The AF as VFL server may train and update its own ML Model based on the received intermediate results from NWDAF as VFL clients. The AF may determine intermediate result based on the updated model (e.g., gradient, etc) which may be included in the request to NWDAFs for further training iteration (in step 2).

8.	When AF decides the VFL training complete, it may inform VFL clients (via NEF).

9.	For each NWDAF as VFL client, the NEF forwards the VFL training complete. NEF convert the temporary NWDAF_ID(s) into NWDAF ID.

10.	The AF as VFL server and the NWDAFs as VFL clients store their trained models along with the VFL correlation ID. NEF store list of NWDAFs as VFL clients with VFL correlation ID.

* * * Start of 2nd Changes * * *

##### 6.2H.2.4.2	Inference procedure for vertical federated learning with AF as VFL server

Figure 6.2H.2.4.2-1: VFL inference with AF as VFL server.

1.	The inference procedure may be triggered (e.g., by internal service logic of the AF as VFL server or by a request received from an external entity). AF may determine to use a VFL inference procedure using the stored trained ML model which is represented by VFL correlation ID

- The AF acting as VFL server sends a request to start a VFL inference procedure. It includes the VFL Correlation ID. For starting a VFL training procedure, AF may include sample information (e.g., list of UE ID, area/time condition for data collection).

NEF may translate a requested analytic service information into an Analytic ID, when AF includes an analytic service information other than analytic ID.

When AF include temporary ID(s) of NWDAF which has been stored with VFL Correlation ID, NEF converts it into NWDAF ID based on the store information associated to VFL correlation ID (refer to step 10 in 6.2H.2.3.2).

3.	The NEF forward the request to the NWDAFs based on converted NWDAF ID information. If the message in step 2 contains external identifiers of UEs for sampling, the NEF maps them to internal identifiers of UEs.

4.	Each VFL Client collects its local data based on sample information and generates intermediate local inference results using the stored trained local ML model associated to VFL correlation ID.

5.	Each VFL Client sends the inference results to the VFL server. If VFL client receives a request in step 3, it send the inference results to NEF.

6.	For each VFL client, the NEF forwards the inference result to the VFL server. NEF may convert NWDAF ID(s) into temporary NWDAF ID(s) as received in step 2.

7.	AF as VFL server calculate VFL inference result using the received local inference result.

* * * End of changes* * *