3GPP TSG-WG SA2 Meeting #166	S2-2412140

# Orlando, US, 18 – 22 November, 2024		(revision of S2-2411269)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5750 | rev | 12 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of N3GPP device behind UE/5G-RGas concluded in TS 23.700-34 KI#4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Huawei, HiSilicon, InterDigital Inc., Deutsche Telekom, NEC, Nokia, ZTE, Oppo], Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The UIA_ARC WID includes that adding support of Non-3GPP device connecting behind a UE as concluded in the TR 23.700-32 KI#4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To support Non-3GPP device connecting behind the UE the following changes are needed: - Introduction of general part description and how identification of traffic from Non-3GPP device connecting behind a UEis supported. - Session management - QoS differentiation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusion of KI#4 would not be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (new), 5.x.1 (new), 5.x.2 (new), 5.x.3 (new), 5.x.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The refernce to TS 23.503 [45] clause 6.1.3.xx in CR 1310 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | rev 12 clarifies the following: - Clarifying that inserting the user plane address with device identifier is mandatory - Clarifying that for PDU session type ethernet the Device ID and user plane address is also included at PDU session establishment. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.x	QoS differentiation of traffic for Non-3GPP Device Identifier

### 5.x.1	General

This clause specifies the scenario of a non-3GPP device connecting through the UE. In this scenario QoS differentiation of traffic is applied to the traffic that originates from or is directed to the non-3GPP device. The non-3GPP device does not use NAS and is not authenticated by 5GC.

The support of identification of traffic for non-3GPP devices connecting behind a 5G-RG is specified in TS 23.316 [84].

The Non-3GPP Device Identifier is unique within the scope of the UE’s SUPI.

### 5.x.2 Traffic identification

When a non-3GPP device is connecting to the UE, the UE may bind the Non-3GPP Device Identifier to a non-3GPP device, for the traffic of non-3GPP devices that require differentiated QoS. This binding enables the 5G System to distinguish between the traffic generated by different non-3GPP devices connected through the same UE.

NOTE 1: How the UE identifies the non-3GPP device and binds the Non-3GPP Device Identifier to a non-3GPP device is implementation specific.

NOTE 2: At any point in time the Non-3GPP Device Identifier can be bound to only one non-3GPP device.

Non-3GPP Device Identifier Information is stored in the UDR and includes a Non-3GPP Device Identifier and QoS Information.

### 5.x.3	Session management enhancement

For the traffic of non-3GPP devices requiring differentiated QoS:

-	For Ethernet PDU Session Type, the UE sends the Non-3GPP Device Identifier and the MAC address and/or the VLAN tag ID that is associated with the Non-3GPP Device Identifier to the SMF in PDU Session Establishment and Modification procedure.

-	For IPv4 PDU Session Type, the UE sends the Non-3GPP Device Identifier and the IP Address and/port ranges associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

-	For IPv6 PDU Session Type, the UE sends the Non-3GPP Device Identifier and the IPv6 Address/prefix(sub) that is associated with the Non-3GPP Device Identifier to the SMF in PDU Session Modification procedure.

### 5.x.4	QoS differentiation

QoS differentiation and policy control is defined in TS 23.503 [45] clause 6.1.3.xx

# * * * * End of changes * * * *