3GPP TSG-WG SA2 Meeting #166	S2-2412155

# Orlando, Florida, 18-22 November 2024						          (revision of S2-2411056)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5739 | rev | 7 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: UE subscription and policy control for energy efficiency and energy saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [vivo, ZTE, Samsung, Lenovo, NEC, Huawei, China Mobile, MediaTek Inc., China Unicom, TOYOTA, China Telecom, CATT, Deutsche Telekom, LG Electronics], Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the conclusion for KI#2 of TR 23.700-66, the subscription and policy control for energy efficiency and saving is agreed. The related subscription and policy control function needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The subscription and policy control for energy efficiency and energy saving are added. Revision 7 aims at clarifying the full picture related to subscription handling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The subscription and policy control for energy efficiency and energy saving is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y (new), 5.X.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 5.x is introduced by 23.501 CR 5628 |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text new) * * * *

### 5.X.Y	Subscription information aspects

A PLMN may define policies that are aimed at differentiating the handing of a UE in the network form an energy standpoint, based on the information in the subscription. While the possible policies are not in scope of this specification, the subscription information which may be used to trigger such policies may include, for instance:

-	The subscribed RFSP Index, which can trigger RRM policies which may affect energy consumption in the network for the UE.

-	The subscribed S-NSSAIs which may point to network slices that are configured to trigger energy saving policies end to end, including in the NG-RAN and in the core network.

-	The Subscription data may include an Energy Behaviour Indication which can be used to determined whether and which network energy saving network behaviour applies for a UE. The Energy Behaviour Indication, if stored in the UDM/UDR may be used by a PLMN to differentiate the subscriber handling. This information may be used in the CN and may be passed to the NG-RAN alongside the RFSP Index value and the Allowed NSSAI and partially Allowed NSSAI which further condition the handling of the UE in the NG-RAN from an anergy standpoint. For instance, the RFSP Index value sent to the NG-RAN, the inactivity timers in NG-RAN may be selected also taking the Energy Behaviour Indication into account. The Energy behaviour indication is defined by the operator and can identify one of up to 8 different energy system behaviours for a UE.

### 5.X.Z	Policy control aspects

The PCF may receive the energy related Subscription data described in clause 5.x.y. Based on the subscription data and potential energy related notifications, the PCF may make policy decisions reusing the existing parameters based on the operator policy.

Editor’s note: The notification for the energy related information, if any, should not be provided from the UDM/UDR. The details of notification for the energy related information, e.g. what the notification would be, or which entity (e.g. EIF) provides the notification, is FFS.

Editor’s note: It’s FFS if other entities could also get the energy related subscription data.

Editor’s note: Further details of possible policy decisions are FFS.

# * * * End of changes * * * *