3GPP SA WG2 Meeting #166	 S2-2412166

# Orlando, US, 18-22 November 2024	        (was S2-2411011)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1489 | rev | 7 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Capability Exposure Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Qualcomm?, Samsung?, ZTE? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.2 in 3GPP TS 23.700-77 concludes KI#2 on "Impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session". The corresponding normative work need to be implementd. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures of IMS data channel capabilies exposure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No normative specification for conclusions of KI#2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX and AA.2.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR depends on S2-24xxxxx. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Change summary of R04: - Add the description of the services and related parameters used in the proceudres. - Add intermediate call notification during data channel establishment procedures. - Add a new event in step 4 of clause XX.2.1. - Add a Nhss_ImsUECM_AsInfoGet operation. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# Annex XX (normative): IMS Capability Exposure Framework in IMS

## XX.1	Description

Two scenarios utilizing IMS data channel capability exposure services are depicted in XX.2:

-	Adding data channel to an existing IMS session, for which Nimsas_SessionManagement_Update is used

-	Creating a standalone data channel session, for which Nimsas_SessionManagement_Create is used.

## XX.2		IMS data channel capability exposure procedures

### XX.2.1	Adding Data Channel to an Existing IMS Session

Figure XX.2.1-1 shows the procedure of adding bootstrap data channel(s) or application data channel(s) to an existing IMS session.

Figure XX.2.1-1: Procedure of adding data channel(s) to an existing IMS session

0.	DC AS may determine to add data channel to an existing IMS session based on local policy or event notification of the related IMS Session which DC AS subscribes to.

NOTE 1:	It is assumed that the OMA API can be used for non-DC session notification in step 0.

1.	The DC AS sends a Nnef_IMSSessionManagement_Update request to the NEF requiring to add bootstrap data channel(s) or application data channel(s) in the exisiting IMS session. The request contains a callback URI provide by the DC AS to receive (implicitly subscribed) notification on data channel establishment.

2.	The NEF returns a successful Nnef_IMSSessionManagement_Update response to the DC AS.

3.	The NEF queries the HSS through Nhss_ImsUECM_AsInfoGet request for the address of the IMS AS serving the UE B if needed.

4.	The NEF sends a Nimsas_IMSSessionManagement_Update request to the IMS AS-2 requiring to add bootstrap data channel(s) or application data channel(s) in the specific session. The request contains a callback URI provide by the MEF to receive (implicitly subscribed) notification on data channel establishment.

5.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Update response to the NEF.

6.	The IMS AS-2 validates the user subscription data and determine whether the data channel request should be notified to DCSF-2. If the serving UE does not have subscription of IMS data channel, then the request shall be rejected with appropriate cause and skip the following step.

The IMS AS-2 reports the NetworkInitiatedMediaChangeRequestEvent through a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of adding data channel(s). Then the DCSF-2 instructs the IMS AS-2 to set up the data channel by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource. The existing procedures depicted in Annex AC.7 is reused.

NOTE 2:	The kind of data channel established is dependent on the instruction given by DCSF.

7.	The IMS AS-2 generates a re-INVITE request in which the SDP offer contains the media information of the data channel required by the DC AS with the other existing media descriptions.

The IMS AS-2 sends the re-INVITE request according to the existing IMS DC session update procedure.

8.	The DC media negotiation is completed and corresponding data channel are established.

9-10.	When the required data channel(s) are established, the IMS AS-2 sends a Nimsas_ImsEE_Notify request to the NEF and the NEF returns a successful Nimsas_ImsEE_Notify response to the IMS AS-2.

11-12.	The NEF sends a Nnef_ImsEE_Notify request to the AF and the AF returns a successful Nnef_ImsEE_Notify response to the NEF.

### XX.2.2	Establishing IMS Session with Standalone Data Channel

Figure XX.2.2-1 shows the establishment procedure of a standalone data channel session.

Figure XX.2.2-1: Establishment procedure of a standalone data channel session

0.	The DC AS decides to establish a standalone data channel session.

1.	The DC AS sends a Nnef_IMSSessionManagement_Create request to the NEF requiring to create a standalnone DC session with UE A. The request contains a callback URI provide by the DC AS to receive (implicitly subscribed) notification on data channel establishment.

2.	The NEF returns a successful Nnef_IMSSessionManagement_Create response to the DC AS.

3.	The NEF queries the HSS through Nhss_ImsUECM_AsInfoGet request for the address of the IMS AS serving the service number of the enterprise if needed. If the service number is unregistered, that is, the HSS fails to return the address of IMS AS, the NEF shall reject the Nnef_ImsSessionManagement_Update request from the DC AS with an error response and the following steps shall be skipped.

4.	The NEF sends a Nimsas_SessionManagement_Create request to the IMS AS-2 requiring to create a standalone DC session. The request contains a callback URI provide by the NEF to receive (implicitly subscribed) notification on data channel establishment.

5.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Create response to the NEF.

6.	The IMS AS-2 validates the user subscription data and determine whether the data channel request should be notified to DCSF-2. If the serving UE does not have subscription of IMS data channel, then the request shall be rejected with appropriate cause.

The IMS AS-2 reports the NetworkInitiatedMediaChangeRequestEvent through a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of creating a data channel. Then the DCSF-2 instructs the IMS AS-2 to set up the data channel by sending a Nimsas_MediaControl request. The IMS AS-2 interacts with the MF-2 to allocate data channel resource. The existing procedures in Annex AC.7 is reused.

NOTE 1:	The kind of data channel established is dependent on the instruction given by DCSF.

7.	The IMS AS-2 generates an INVITE request in which the SDP offer contains the media information of the data channel required by the DC AS.

The IMS AS-2 sends the INVITE request according to the existing IMS DC session establishment procedure.

8.	The DC media negotiation is completed and corresponding data channel are established.

9-10.	When the required data channel(s) are established, the IMS AS-2 sends a Nimsas_ImsEE_Notify request to the NEF and the NEF returns a successful Nimsas_ImsEE_Notify response to the IMS AS-2.

11-12.	The NEF sends a Nnef_ImsEE_Notify request to the AF and the AF returns a successful Nnef_ImsEE_Notify response to the NEF.

### XX.2.3	Removing Data Channel from an Existing IMS session

Figure XX.2.3-1 shows the termination procedure of bootstrap data channel(s) or application data channel(s) in an existing IMS session.

Figure XX.2.3-1: Termination procedure of data channel

1.	The DC AS sends a Nnef_IMSSessionManagement_Update request to the NEF requiring to terminate the data channel with UE A in the specific session identified by Session ID. The request contains a callback URI provide by the DC AS to receive (implicitly subscribed) notification on data channel establishment.

2.	The NEF returns a successful Nnef_IMSSessionManagement_Update response to the DC AS.

3.	The NEF queries the HSS through Nhss_ImsUECM_AsInfoGet request for the address of the IMS AS serving the specific UE if needed.

4.	The NEF sends a Nimsas_SessionManagement_Update request to the IMS AS-2 requiring to terminate the data channel in the specific session identified by Session ID. The request contains a callback URI provide by the NEF to receive (implicitly subscribed) notification on data channel establishment.

5.	The IMS AS-2 returns a successful Nimsas_SessionManagement_Update response to the NEF.

6.	The IMS AS-2 reports the NetworkInitiatedMediaChangeRequestEvent through a Nimsas_SessionEventControl_Notify request to the DCSF-2 with the indication of removing the data channel. Then the DCSF-2 instructs the IMS AS-2 to terminate the data channel by sending a Nimsas_MediaControl request.

7.	The IMS AS-2 generates a re-INVITE request in which the port of "m=application" line for the data channel to be removed is set to zero.

The IMS AS-2 sends the re-INVITE request according to existing IMS procedure to terminate the data channel.

8.	The media negotiation is completed and corresponding data channel are removed.

9-10.	When the required data channel(s) are established, the IMS AS-2 sends a Nimsas_ImsEE_Notify request to the NEF and the NEF returns a successful Nimsas_ImsEE_Notify response to the IMS AS-2.

11-12.	The NEF sends a Nnef_ImsEE_Notify request to the AF and the AF returns a successful Nnef_ImsEE_Notify response to the NEF.

# * * * * Second change * * * *

#### AA.2.4.2.2	Nimsas_SessionEventControl_Notify service operation

Service operation name: Nimsas_SessionEventControl_Notify

Description: This service operation enables IMS AS to notify consumers of session events related to a specific served IMS subscriber requesting use of IMS data channel media.

Inputs, Required: Session ID, Event ID,

Session ID is the identity of the IMS session for which the event relates to.

Event ID is the event triggered within the IMS session.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator, Media info list, Network-initiated media info list.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Session case indicates if this is an originating or terminating IMS session. Event initiator indicates initiator of the event, i.e. 'served IMS subscriber' vs 'remote IMS subscriber'. Media info list includes for each media in the list:

-	media ID: uniquely identifies this media item within the list. The identity is allocated by IMS AS,

-	media specification: This depends on media type including relevant media attributes of interest to the consumer. The media specification includes the following media description attributes:

-	Media Type: DC, Audio, or Video.

When the media type is "DC", the elements below are derived from the SDP received by the IMS AS in an SIP INVITE or a re-INVITE related to an IMS Data Channel and the corresponding DTLS connection.

-	Data Channel Mapping and Configuration Information: This attribute is applicable to Data Channel and includes relevant configuration Information, including the stream ID and application binding information of the Data Channel.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security set up of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity.

When the network-side node, including the third party AS/AF through the NEF, requires to create or remove some data channel(s) in a IMS session, Network initiated media info list is used to indicate the media information when the EventID is NetworkInitiatedMediaChangeRequestEvent. For each entry in the list, it contains:

-	media ID: uniquely identifies this media item within the list. The identity is allocated by IMS AS.

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the media type used in the created session, e.g., Data Channel.

-	Media Parameter: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it need indicate the Data Channel Type (ADC or BDC):

-	When ADC is selected, it need indicate the ADC type (P2A, P2A2P), Application Binding Information, and if P2A ADC is selected, the party which this request is targeted, and MDC2 media endpoint address of the application layer.

-	When BDC is selected, it need indicate the party (the caller, or the callee) with which the BDC is used.

Outputs, Required: Result indication.

Outputs, Optional: None.

The table below presents supported EventIDs and related parameters.

Table AA.2.4.2.2-1: List of events and Related Optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentRequestEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |
| SessionEstablishmentProgressEvent | Media info list |
| SessionEstablishmentAlertingEvent | Media info list |
| SessionEstablishmentSuccessEvent | Media info list |
| SessionEstablishmentFailureEvent |  |
| MediaChangeRequestEvent. | Event initiator, Media info list, Calling ID, Called ID, Session case |
| MediaChangeSuccessEvent | Media info list |
| MediaChangeFailureEvent | Media info list |
| SessionTerminationEvent | Session case |
| NetworkInitiatedMediaChangeRequestEvent | Calling ID, Called ID, Network-initiated media info list |

# * * * * End of changes * * * *