3GPP TSG-WG SA2 Meeting #166 meeting	S2-2412170

# Orlando US, 18th Nov – 22th Nov, 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1525 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting Avatar Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the previous meeting, the architecture for avatar communication has been agreed, this contribution proposes to add the call flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the avatar communication call flows |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No normative specification for conclusions of KI#8 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.X.3 (new), AA.2.X (new), AA.2.5.1, AA.2.5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * *

## AC.X.3	Procedure

### AC.X.3.1 Avatar ID List Download Procedure

Figure AC.X.3.1-1: Procedures of Avatar ID List Download

Figure AC.X.3.1-1 depicts a typical call flow procedure of avatar ID list download procedure. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established.

2.	The UE1 downloads application list through established bootstrap data channel.

3.	The UE1 selects and downloads avatar communication application through established bootstrap data channel.

4.	The DCSF checks the application is avatar communication, and select the corresponding DC AS based on local configuration.

5-6.	The DCSF downloads avatar communication from DCAR.

7. The DCSF involves Ndcas_AvatarIDList_Get to request DC AS to get UE1’s avatar ID list.

8.	The DC AS get UE1’s avatar ID list from BAR.

9-10.	The BAR returns UE1’s avatar ID list to the DC AS, the DC AS returns it to the DCSF.

11.	The DCSF sends the avatar ID list together with the avatar application to the UE1.

### AC.X.3.2		UE centric procedure

The UE centric mode includes two scenarios:

-	Local UE centric: When the UE initiates an avatar communication with the peer UE, it downloads the avatar representation from BAR and performs avatar animation locally based on the metadata generated by itself and sends the animated avatar media to the peer UE.

-	Peer UE centric: When the UE initiates an avatar communication with the peer UE, it sends the avatar ID to the peer UE. The peer UE downloads the avatar representation from BAR based on the received avatar ID and performs the avatar animation based on the received metadata.

#### AC.X.3.2.1	Local UE centric procedure

Figure AC.X.3.2.1-1: Procedures of Local UE centric IMS Avatar communication

Figure AC.X.3.2.1-1 depicts a typical call flow procedure of local UE centric IMS Avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application and its avatar ID list as per steps in clause AC.X.3.1. Then UE1 chooses an avatar ID and runs the avatar communication application.

2.	Application data channel(s) are established including a P2P ADC between UE1 and UE2 which is used for avatar metadata transmission, and a P2A ADC between UE1 and DC AS which is used for avatar representation transmission.

3.	UE1 downloads Avatar representation from DC AS using avatar ID and UE1’s identity (e.g., IMSI or MSISDN) through the established application data channel.

4-5.	 The DC AS authenticates the avatar representation download request and requests BAR to download Avatar representation using avatar ID, the BAR responses with avatar representation data.

6.	The DC AS responses with the avatar representation data to MF, and then MF sends it back to UE1.

7.	UE2 may send metadata (e.g., FOV data) to UE1 through the established application data channel.

8.	UE1 may perform media transcoding such as ASR/TTS/SLR/SSL, and animates the avatar representation based on the transcoding result and the metadata which is generated locally and received from UE2.

9.	UE1 transmits the animated avatar media over RTP to UE2.

#### AC.X.3.2.2	Peer UE centric procedure

Figure AC.X.3.2.2-1: Procedures of Peer UE centric IMS Avatar communication

Figure AC.X.3.2.2-1 depicts a typical call flow procedure of peer UE centric IMS Avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application and its avatar ID list as per steps in clause AC.X.3.1. Then UE1 chooses an avatar ID and runs the avatar communication application.

2.	Application data channel(s) are established including a P2P ADC between UE1 and UE2, and a P2A2P ADC between UE1/UE2 and DC AS.

3.	UE1 decides to request peer UE to perform avatar animation based on its status such as power, signal, computing power, internal storage, etc.

4.	UE1 performs avatar animation negotiation with the DC AS and UE2. The negotiation includes usage of the UE1’s avatar ID and the animation data types (e.g., text and/or facial expression) supported by UE1.

NOTE 1:	The negotiation procedure needs to be further defined in SA4, and cooperation with SA4 is needed.

5.	UE2 downloads avatar representation from DC AS using the avatar ID received from UE1 through the established application data channel.

6-7.	The DC AS authenticates the Avatar representation download request and requests BAR to download avatar representation using UE1’s avatar ID, the BAR responses with UE1’s avatar representation data.

8.	The DC AS responses with UE1’s avatar representation data to the MF, and the MF sends it back to UE2.

9.	UE1 may send metadata (e.g., facial expression) to UE2 through the established application data channel or RTP channel between UE1 and UE2.

10.	UE2 animates the UE1’s avatar representation based on the metadata received from UE1.

### AC.X.3.3	Network centric procedure

Figure AC.X.3.3-1: Procedures of network centric IMS Avatar communication

Figure AC.X.3.3-1 depicts a typical call flow procedure of network centric IMS avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application and its avatar ID list as per steps in clause AC.X.3.1. Then UE1 chooses an avatar ID and runs the avatar communication application.

2.	Application data channel(s) are established including a P2P ADC between UE1 and UE2, and a P2A2P ADC between UE1/UE2 and DC AS.

3.	UE1 decides to request network to perform avatar animation based on its status such as power, signal, computing power, internal storage, etc.

4.	The avatar animation negotiation is finished between the DC AS and UE1.

NOTE 1:  The avatar animation negotiation procedure and parameters need to be further defined in SA4, and cooperation with SA4 is needed.

5.	The DC AS instructs the IMS AS through NEF to anchor UE1’s audio/video media and allocate avatar animation resource, by the Nnef_ImsSessionManagement_Update request. The request includes avatar ID, avatar type (i.e. 2D avatar or 3D avatar), animation method (e.g., audio, text or facial expression) and UE1’s audio/video media required indication.

NOTE 2:  DC AS will know the avatar related parameters included in the session management request based on the avatar animation negotiation result in step 4.

6.	The IMS AS requests MF to allocate audio/video media resources by the Nmf_MRM_Update request, and allocate avatar animation media resource by the Nmf_SSM_Create request. The Nmf_SSM_Create request message includes Media Resource Context ID of the ongoing session and the parameters carried in step 5.

7.	The IMS AS sends the Nimsas_ImsSessionManagement_Update response to the DC AS through NEF.

8.	The IMS AS finishes the media re-negotiation between UE1, UE2 and MF, which anchors UE1 and UE2’s audio/video to MF.

9.	The MF downloads UE1’s avatar representation from BAR using the UE1’s avatar ID.

10.	The UE1 sends avatar metadata (such as facial expression) through the established P2A2P ADC, and/or audio/video stream over RTP, to the MF.

11.	The UE2 may send avatar metadata (such as FOV data) through the established P2A2P ADC, to the MF.

12.	MF may perform media transcoding such as ASR/TTS/SLR/SSL, and avatar animation based on the transcoding result and the avatar metadata received from application data channel. MF should support different media processing according the avatar type, e.g., animation with no rendering for 2D avatar, animation and rendering for 3D avatar.

13-14.	MF sends the animated avatar media to UE2, and sends the animated avatar media to UE1 if required.

# * * * * Second change * * *

# AA.2.X DC AS Services

### AA.2.X.1	General

The following table illustrates the DC AS Services and Service Operations.

Table AA.2.X.1-1: NF services provided by the DC AS

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcas_AvatarIDList | Get | Request/Response | DCSF |

### AA.2.X.2	Ndcas_AvatarIDList Service

#### AA.2.X.2.1	General

Service description: This service enables the consumer to get UE’s avatar ID list during avatar communication procedure.

#### AA.2.X.2.2	Ndcas_AvatarIDList_Get service operation

Service operation name: Ndcas_AvatarIDList_Get

Description: This service operation enables DC AS to provide UE’s avatar ID list from BAR.

Inputs, Required: UE identity

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: operation result indication and UE’s avatar ID list.

Outputs, Optional: None.

# * * * * Third change * * *

### AA.2.5.1	General

The following table illustrates the MF Services and Service Operations.

Table AA.2.5.1-1: NF services provided by the MF

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nmf_MediaResourceManagement (MRM) | Create | Request/Response | IMS AS |
|  | Update | Request/Response | IMS AS |
|  | Delete | Request/Response | IMS AS |
| Nmf_ServiceSpecificationManagement (SSM) | Create | Request/Response | IMS AS |
|  | Update | Request/Response | IMS AS |
|  | Delete | Request/Response | IMS AS |

# * * * * Fourth change * * *

### AA.2.5.X	Nmf_ServiceSpecificationManagement (SSM) service

#### AA.2.5.X.1	General

Service description: This service enables the consumer to send the service related parameter to MF for a given session identified by Media Context Resource ID. All service specification shall bind to an existing Media Context Resource ID and will be deleted if the bound media resource is deleted.

#### AA.2.5.X.2	Nmf_SSM_Create service operation

Service operation name: Nmf_SSM_Create

Description: The consumer NF requests MF to create a service specification for an existing media context.

Inputs, Required: Media Context Resource ID, Service Specification.

Service Specification: It indicates the related service parameters.

-	Service Type: Avatar Communication

-	Audio Required: It indicates whether the audio media is required to support this service.

-	Video Required: It indicates whether the video media is required to support this service.

-	Service Data: It indicates the service specific data related to the service. For different service, it contains different service data. For avatar communication, it includes avatar ID, avatar type, animation method.

Inputs, Optional: None.

Outputs, Required: Result Indication, Service Specification ID when the operation is successful.

### Outputs, Optional: None.

#### AA.2.5.X.3	Nmf_SSM_Update service operation

Service operation name: Nmf_SSM_Update

Description: The consumer NF requests MF to update a service specification for an existing media context.

Inputs, Required: Media Context Resource ID, Service Specification ID, Service Specification.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

#### AA.2.5.X.4	Nmf_SSM_Delete service operation

Service operation name: Nmf_SSM_Delete

Description: The consumer NF requests MF to delete a service specification for an existing media context.

Inputs, Required: Media Context Resource ID, Service Specification ID.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

# * * * * End of changes * * * *