3GPP TSG-WG SA2 Meeting #166	S2-2412245

# 18 – 22 November 2024, Orlando, USA		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0510 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on identifiers for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The hop count is useful for 5G Prose End UE to select the path discovered during discovery procedures for both model A and B. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Hop-count as the identifier in Discovery messages of IP PDU type for both model A and B - Correct the clause number for non-IP PDU type |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G Prose End UE is not able to utilize hop count to select the path to target 5G Prose End UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.6.1, 5.8.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.8.6.1	General

There are two types of discovery mechanisms for the support of 5G ProSe Multi-hop UE-to-UE Relays, i.e. for IP PDU type, and for non-IP PDU type (Ethernet or Unstructured). The 5G ProSe End UE and 5G ProSe Multi-hop UE-to-UE Relay choose the discovery mechanism based on the associated indication of the RSC, as defined in clause 5.1.5.1.

The identifiers used for the two types of discovery mechanisms are specified in clause 5.8.6.2 and clause 5.8.6.x respectively.

# * * * * Second change * * * *

5.8.6.2	Identifiers for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type

The Multi-hop UE-to-UE Relay Discovery Announcement message (Model A) is sent by the relay and contains the following information:

-	Source Layer-2 ID: the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay self-selects a Source Layer-2 ID. It is used for the transmitting the message at AS layer, and included in the AS layer headers only.

-	Destination Layer-2 ID: the Destination Layer-2 ID for Multi-hop UE-to-UE Relay Discovery Announcement message (Model A) is selected based on the configuration (associated with the RSC) as defined in clause 5.1.5.1.

-	User Info ID of 5G ProSe Multi-hop UE-to-UE Relay: provides information about the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

-	Relay Service Code: information to indicate the connectivity service the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay provides to the 5G ProSe End UEs.

-    Hop-Count(optional): This value reflects the number of hops for the receving 5G ProSe End UE to reach the announcing 5G ProSe End UE supporting the RSC.

The 5G ProSe Multi-hop UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the source 5G ProSe End UE or a 5G ProSe Layer-3 Multi-hop UE-to-UE Relay, and includes the following information:

-	Source Layer-2 ID: the discoverer 5G ProSe End UE or 5G ProSe Layer-3 Multi-hop UE-to-UE Relay self-selects a Source Layer-2 ID.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe Multi-hop UE-to-UE Relay Discovery Solicitation message is selected based on the configuration (associated with the RSC) as defined in clause 5.1.5.1.

-	(optional) User Info ID of discoverer: this may be used by the receiver of the message to determine whether to respond to the solicitation.

-	Relay Service Code: information about connectivity service offered by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s).

-    Hop-Count(optional): This value reflects the number of hops for the message receiver to reach the source 5G ProSe End UE supporting the RSC.

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC includes the following:

-	Source Layer-2 ID: the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay self-selects a Source Layer-2 ID.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe Multi-hop UE-to-UE Relay Discovery Solicitation message.

-	User Info ID of discoverer: the User Info ID from the received Solicitation message, to ensure the response can be matched at the receiver.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe Multi-hop UE-to-UE Relay.

-	Relay Service Code: information about connectivity service offered by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

-    Hop-Count(optional): This value reflects the number of hops between two 5G ProSe End UEs supporting the RSC.

Editor's note:	Additional information or identifiers for MANET operation assistance may be added.

# * * * * End of changes * * * *