3GPP TSG-WG SA2 Meeting #166	S2-2412288

# Orlando, USA, November 18 – 22, 2024													(was S2-2411020)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2136 | rev | 7 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identifying non-3GPP devices behind 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CableLabs, Nokia, InterDigital, Charter Communications, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To specify the support for identifying non-3GPP devices connecting behind a 5G-RG based on the conclusions in clause 8.4 of TR 23.700-32 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying conclusions on identification of non-3GPP device for wireline access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature not implemented in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10x(new), Annex X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | ‘Clause 5.x’ in this CR’s clause 4.10x refers to the clause 5.x in CR 5750 of TS 23.501 ‘Clause 4.15.6.X’ in this CR’s clause Annex X refers to the clause 4.15.6.X in CR 4848 of TS 23.502 |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 4.10x	Differentiated QoS for non-3GPP devices behind 5G-RG

This clause defines the support for identifying the traffic of individual non-3GPP devices behind a 5G-RG and providing them differentiated QoS.

TS 23.501 [2] clause 5.x applies to the 5G-RG with the following deltas:

-	The UE is replaced by 5G-RG.

The overall architecture is illustrated in Figure 4.10x-1. Non-3GPP devices associated with the same PDU Session can be further differentiated using their Non-3GPP Device Identifiers. This is further described in Annex X.

Figure 4.10x-1: Example scenario for mapping traffic of individual non-3GPP devices behind 5G-RG to a PDU Session

# * * * * Second change * * * *

Annex X (informative):
QoS differentiation of traffic of individual non-3GPP devices behind 5G-RG

This Annex describes how the traffic of individual non-3GPP devices behind a 5G-RG can be identified and provided with differentiated QoS.

Figure X-1: Example scenario for mapping traffic of individual non-3GPP devices to a PDU Session

As in this example, two non-3GPP devices mapped to PDU Session A initially used the default QoS Flow (QFI 1); when differentiated QoS is requested for one device, the 5G-RG binds its traffic to a Non-3GPP Device Identifier, and its traffic is mapped to a separate QoS Flow (QFI 2). Four non-3GPP devices mapped to PDU Session B based on their Connectivity Group ID X initially used the default QoS Flow (QFI 3); when differentitated QoS is requested for two of those four devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers, and their traffic is mapped to separate QoS Flows (QFI 4 and QFI 5). Similarly, three non-3GPP devices mapped to PDU Session C based on their Connectivity Group ID Y initially used the default QoS Flow (QFI 6); when differentiated, but the same, QoS is requested for two of those three devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers, and their traffic is mapped to a separate QoS Flow (QFI 7).

Figure X-2 illustrates a procedure which enables the 5GS to identify the traffic of individual non-3GPP devices initially using the same PDU Session behind a 5G-RG and provide differentiated QoS.

Figure X-2: Example procedure for enabling QoS differentiation for individual non-3GPP devices behind 5G-RG

0a. Non-3GPP device 1 is connected to the 5G-RG.

0b.	To provide connectivity to the non-3GPP device 1, the 5G-RG implements the existing behaviour of either using the URSP rule (optionally containing the Connectivity Group ID as described in clause 4.10b), or using local configurations, to map the traffic of the non-3GPP device 1 to a PDU Session.

0c. Non-3GPP device 2 is connected to the 5G-RG.

0d. To provide connectivity to the non-3GPP device 2, the 5G-RG implements the existing behaviour of either using the URSP rule (optionally containing the Connectivity Group ID as described in clause 4.10b), or using local configurations, to map the traffic of the non-3GPP device 2 to the same PDU Session as non-3GPP device 1.

1.	The 5G-RG subscription owner or an authorized user, using mechanisms out of scope of 3GPP, requests differentiated QoS for the non-3GPP device 2 through the AF.

NOTE 1: 	The request for differentiated service can be made through an operator portal hosted either in the 5G-RG or in the AF.

2.	AF provisions the Non-3GPP Device Identifier Information for the non-3GPP device 2 into the UDR, as defined in clause 4.15.6.X of TS 23.502 [3].

NOTE 2:	Provisioning of the Non-3GPP Device Identifier Information into the UDR is done only for the non-3GPP devices that require differentiated QoS.

3.	PCF decides to update UE policy as defined in clause 4.16.12.2 of TS 23.502 [3] and 6.1.2.2.2 of TS 23.503 [4].

4.	Based on the network configuration either a 5G-RG requested PDU Session Modification, or a 5G-RG requested PDU Session Establishment procedure is triggered.

4a. (5G-RG requested PDU Session Modification) As defined in Clause 7.3.2.

4b. (5G-RG requested PDU Session Establishment) As defined in Clause 7.3.1.

Editor’s note:	Whether the PCF initiated SM Policy Association Modification procedure as defined in clause 4.16.5.2 of TS 23.502 [3] can also be used is FFS.

5.   In response to step 1, using mechanisms out of scope of 3GPP, the operator portal returns a response to the 5G-RG subscription owner or the authorized user about the completion of the differentiated QoS request for non-3GPP device 2.

# * * * * End of changes * * * *